2005 Cassini/CAPS

CAssini Plasma Spectrometer

CAPS STANDARD DATA PRODUCTS

AND ARCHIVE VOLUME

SOFTWARE INTERFACE SPECIFICATION

(CAPS Archive Volumes SIS)

SIS ID:  IO-AR-017

Version 1.14

rev. August 16, 2005

J. Furman & F. Crary

Southwest Research Institute

San Antonio, TX 78238

and

S. Joy

University of California, Los Angeles

Los Angeles, CA 90095-1567

2005 Cassini/CAPS

CAssini Plasma Spectrometer

CAPS STANDARD DATA PRODUCTS

ARCHIVE VOLUMES

SOFTWARE INTERFACE SPECIFICATION

(CAPS Archive Volumes SIS)

Version 1.14

rev. August 16, 2005

Approved:

D. T. Young	Date

Principal Investigator

Diane Conner	Date

Cassini Archive Data Engineer

Ray Walker	Date

PDS Discipline Node Manager

1. Preface	4

1.1. Distribution List	4

1.2. Document Change Log	4

1.3. TBD Items	5

1.4. Acronyms and Abbreviations	5

1.5. Glossary	6

2. Introduction	8

2.1. Content Overview	8

2.2. Scope	9

2.3. Applicable Documents	9

2.4. Audience	10

3. Archive Volume Generation	11

3.1. Data Production and Transfer Methods	11

3.2. Archive Volume Creation and Validation Methods	11

3.3. Labeling and Identification	12

4. Archive Volume Contents	14

4.1. Root Directory Contents	14

4.2. INDEX Directory Contents	14

4.3. DOCUMENT Directory Contents	15

4.4. CATALOG Directory Contents	16

4.5. DATA (Standard Products) Directory Contents and Naming Conventions	17

4.5.1. Required Files	17

4.5.2. File Naming Conventions	18

4.5.3. DATA/UNCALIBRATED/YYYYDDD Directory Contents	18

4.5.4. DATA/CALIBRATED/YYYYDDD Directory Contents	20

4.6. CALIB Directory Contents	21

4.6.1. CALIB/SAMPLE_DATA Directory Contents	23

4.7. EXTRAS Directory Contents	23

4.8. BROWSE Directory Contents	23

5. Archive Volume Format	25

5.1. Disk Format	25

5.2. File Formats	25

5.2.1. Document File Formats	25

5.2.2. Catalog File Formats	25

5.2.3. PDS Label File Formats	25

5.2.4. Data File Formats – Binary Tables	27

5.3. CAPS Standard UNCALIBRATED Data Product Descriptions	27

5.3.1. CAPS ELS Data Product Format	27

5.3.2. CAPS IBS Data Product Format	28

5.3.3. CAPS IMS ION Data Product Format	29

5.3.4. CAPS IMS SNG Data Product Format	30

5.3.5. CAPS IMS LOG Data Product Format	31

5.3.6. CAPS IMS TOF Data Product Format	32

5.3.7. CAPS ACT Data Product Format	33

5.3.8. CAPS ANC Data Product Format	34

5.3.9. CAPS EVN Data Product Format	39

5.4. CAPS Standard CALIBRATED Data Product Descriptions	39

5.4.1. CAPS ELS Data Product Format	39

5.4.2. CAPS IBS Data Product Format	40

5.4.3. CAPS IMS ION Data Product Format	40

5.4.4. CAPS IMS SNG Data Product Format	40

5.4.5. CAPS IMS TOF Data Product Format	40

5.4.6. CAPS ANC Data Product Format	40

6. Support Staff and Cognizant Persons	41

Appendix A. Directory Structure for Archive Volume, COCAPS_1nnn	42

Appendix B. PDS Labels & Format Files for Standard UNCALIBRATED Data Products	44

List Of Tables:

Table 1: Distribution List	4

Table 2: Document Change History	4

Table 3: TBD Items	5

Table 4:  Acronyms and Abbreviations	5

Table 5:  Spacecraft Science Data Products in CAPS Data Sets	8

Table 6: Relationship Between Data Sets, Standard Data Product Types, and Archive Volumes	13

Table 7: Root Directory Contents	14

Table 8: Index Directory Contents	15

Table 9: Document Directory Contents	15

Table 10: Document/CAPS_SIS Directory Contents	16

Table 11: Document/CAPS_CALIB Directory Contents	16

Table 12: Catalog Directory Contents	17

Table 13: YYYYDDD UNCALIBRATED Data Directory Contents	19

Table 14: YYYYDDD CALIBRATED Data Directory Contents	20

Table 15: CALIB Directory Contents	21

Table 16: YYYYDDD BROWSE Directory Contents	24

Table 17: CAPS ELS UNCALIBRATED Data File Contents and Structure	27

Table 18: CAPS IBS UNCALIBRATED Data File Contents and Structure	28

Table 19: CAPS UNCALIBRATED IMS ION Data File Contents and Structure	29

Table 20: CAPS UNCALIBRATED IMS Singles Data File Contents and Structure	30

Table 21: CAPS IMS Logicals UNCALIBRATED Data File Contents and Structure	31

Table 22: CAPS IMS TOF UNCALIBRATED Data File Contents and Structure	32

Table 23: CAPS ACT Data File Contents and Structure (both Calibrated & Un-calibrated)	34

Table 24: CAPS ANC UNCALIBRATED Data File Contents and Structure	34

Table 25: CAPS EVN UNCALIBRATED Data File Contents and Structure	39

Table 26: CAPS Archive Collection Support Staff	41

# Preface

This document describes the contents and types of volumes belonging to all of the CAPS data sets.

## Distribution List

| Table 1: Distribution List |  |
| --- | --- |
| Name | Email |
| Steve Joy | sjoy@igpp.ucla.edu |
| Joe Mafi | jmafi@igpp.ucla.edu |
| David T. Young | dyoung@swri.edu |
| Frank Crary | fcrary@swri.edu |
| Judith Furman | jfurman@swri.edu |
| CAPS Team | caps_team@swri.edu |
| Scott Bolton | sbolton@swri.edu |
| Ray Walker | rwalker@igpp.ucla.edu |
| Diane Conner | Diane.Conner@jpl.nasa.gov |
| Robert Mitchell | Robert.Mitchell@jpl.nasa.gov |

## Document Change Log

| Table 2: Document Change History |  |  |
| --- | --- | --- |
| Change | Date | Affected Portions |
| Initial Draft | 03/2003 | All |
| Updated Items with comments made by Steve Joy | 03/24/2003 | All |
| Updates by J. Furman in response to Steve Joy & continued updates to all sections | 04/2003 | All |
| Updates by J. Furman in response to action items from archive review and updates needed for the team. Also, updated the directory structure (issues with too many files in the same directory) | 06/2003 | Contents of Data Products |
| Update to ACT name & some issues with the format itself (min, max, etc) | 7/28/2003 |  |
| Update instrument tables & description | 11/2003 | All |
| Make sure all tables are up-to-date | 3/9/2004 |  |
| Modified the ANC data format. Added position relative to the Sun & Saturn at all times | 4/28/2004 | 5.3.8 |
| Updated IBS data format. Increased the size of the offset time to 4 bytes, and also changed a few definitions | 6/25/2004 | Table 18 |
| Added in format files and sample labels files | 6/25/2004 | Appendix B. PDS Labels & Format Files for Standard UNCALIBRATED Data Products |
| Made changes to B-cycle & A-cycle number descriptions | 1/25/2005 | Table 17 through Table 25 |
| Update ancillary data product | 3/30/2005 | ANC table |
| Updated some of the TBD items. Added in sections to indicate that we now have 2 separate volumes. Added a few additional tables for new directories that were added as a result of peer review. Updated format files & label files included in this document. | 8/1/2005 | Scattered throughout the document |

## TBD Items

Items that are currently still to be specified:

| Table 3: TBD Items |  |  |
| --- | --- | --- |
| Item | Section | Page(s) |
| Resubmission of calibrated files | Section 2.1 | 8 |
| Size of calibrated data | Table 5 | 8 |
| Data product format for all the calibrated data files | Section 5.4 | 38 |

## Acronyms and Abbreviations

| Table 4: Acronyms and Abbreviations |  |
| --- | --- |
| Acronym | Definition |
| ASCII | American Standard Code for Information Interchange |
| CAPS | CAssini Plasma Spectrometer |
| CD-R | Compact Disc - Recordable Media |
| CD-ROM | Compact Disc - Read-Only Memory |
| DVD | Digital Versatile Disc |
| ELS | Electron Spectrometer |
| EVT | Ion Mass Spectrometer Event Mode Data Product |
| GB | Gigabyte(s) |
| IBS | Ion Beam Spectrometer |
| IMS | Ion Mass Spectrometer |
| ISO | International Standards Organization |
| JPL | Jet Propulsion Laboratory |
| LOG | Ion Mass Spectrometer’s Logical Data Product |
| MB | Megabyte(s) |
| NSSDC | National Space Science Data Center |
| PDB | Project Database |
| PDS | Planetary Data System |
| PNG | Portable Network Graphic. A bit-mapped graphics format |
| PPI | Planetary Data System, Planetary Plasma Interactions Node |
| SDVT | Science Data Validation Team |
| SNG | Ion Mass Spectrometer Singles Data Product |
| SIS | Software Interface Specification |
| TBD | To Be Determined |
| TOF – LEF | Time of Flight – Linear Electric Field |
| TOF – ST | Time of Flight – Straight Through |

## Glossary

Archive – An archive consists of one or more Data Sets along with all the documentation and ancillary information needed to understand and use the data.  An archive is a logical construct independent of the medium on which it is stored.

Archive Volume - An Archive Volume is a single physical media (CDROM, DVD, 9-track tape, etc.) used to permanently store files within the PDS archive. Archive Volumes may only be created on media approved by the PDS as meeting archive quality standards.

Archive Volume Set – A collection of one or more Archive Volumes used to store a single Data Set or collection of related Data Sets.

Catalog Information – High-level descriptive information about a Data Set (e.g., mission description, spacecraft description, instrument description), expressed in Object Description Language (ODL), which is suitable for loading into a PDS catalog.

Data Product – A labeled grouping of data resulting from a scientific observation, usually stored in one file.  A product label identifies, describes, and defines the structure of the data. An example of a Data Product is a planetary image, a spectral table, or a time series table.

Data Set – A Data Set is a collection of Data Products from a single instrument that have a common data processing level, together with supporting documentation and ancillary files.

Standard Data Product – A Data Product generated in a predefined way using well-understood procedures, processed in "pipeline" fashion.  Data Products that are generated in a nonstandard way are sometimes called special Data Products.

# Introduction

## Content Overview

The Cassini Plasma Spectrometer (CAPS) aboard the Cassini spacecraft is an instrument comprised of three different sensors:  the Electron Spectrometer (ELS), the Ion Mass Spectrometer (IMS), and the Ion Beam Spectrometer (IBS).  The primary focus of CAPS’s mission is Saturn science, but we will take data at Earth and Jupiter as well as interplanetary science.

The CAPS instrument is a complex instrument that produces large amounts of data.  We intend to archive both un-calibrated and calibrated data files to the PDS.  Due to the complexity of calibrations, we will resubmit calibrated data files on a TBD basis.  Also, we will be archiving a set of special (higher order) data products on a limited basis.

CAPS will be archiving two data sets: un-calibrated and calibrated.  Each data set will be archived on a separate volume.  The un-calibrated data set will be archived with some very basic calibration procedures.  These procedures may be updated, but the calibrated data volume and files will contain the very latest in calibration information.  There are several different types of data products per each data set.  The ELS and IBS sensors each produce their own data product.  The IMS sensor generates several different data products including Event Mode (EVN), two Time of Flight data products that will be archived in the same file (TOF), a singles data product (SNG), a logicals data product (LOG), and an ion data product (ION).  In addition, we have an actuator data product (ACT) and an ancillary data product (ANC).

Most CAPS data products are collected

on 32-second cycles (called A-cycles).  IMS Time-of-Flight (TOF) data products are a collection of A-cycles (called B-cycles).  Each B-cycle represents one-full time of flight – energy spectrum.  The number of A-cycles per B-cycle varies depending upon the data rate of the instrument, due to data volume limitations.  In version 4.0 (and later) of CAPS flight software, the IBS sensor data will be collected on a fixed 8 A-cycle collection period (called a C-cycle).  One goal with our archive format is for the differences in data rate and flight software version to be transparent to the end user.

The data products mentioned are briefly described in Table 5 below, including the data set in which they will be included and the maximum data volume of each different data type (per day).  Each sensor’s data will be written to a separate file, and the format of each file will be discussed in detail in section 5.3, and Table 17 through Table 25.

| Table 5: Spacecraft Science Data Products in CAPS Data Sets |  |  |  |
| --- | --- | --- | --- |
| Sensor | Data Set Type | Maximum (MB / Day) | Sensor Total (MB / Day) |
| ELS | Un-calibrated | 103.821 |  |
|  | Calibrated | TBD |  |
| IBS | Un-calibrated | 315.170 |  |
|  | Calibrated | TBD |  |
| IMS TOF | Un-calibrated | 1.32544 |  |
|  | Calibrated | TBD |  |
| IMS ION | Un-calibrated | 381.541 |  |
|  | Calibrated | TBD |  |
| IMS SNG | Un-calibrated | 51.9104 |  |
|  | Calibrated | TBD |  |
| ACT | Un-calibrated | 0.360489 |  |
|  | Calibrated | TBD |  |
| ANC | Un-calibrated | 0.37594 |  |
|  | Calibrated | TBD |  |
| IMS LOG | Un-calibrated | 46.7194 | 46.7194 |
| EVN | Un-calibrated | 12.198 | 12.198 |

## Scope

This specification applies to all archive volumes containing CAPS data products for the duration of its mission.

## Applicable Documents

Planetary Science Data Dictionary Document, August 28, 2002, Planetary Data System, JPL D-7116, Rev. E.

Planetary Data System Data Preparation Workbook, February 1995, JPL D-7669, Part 1, Version 3.1.

Planetary Data System Standards Reference, August 1, 2003, JPL D-7669, Part 2, Version 3.6.

Cassini/Huygens Program Archive Plan for Science Data, PD 699-068, JPL D-159576

## Audience

This specification is useful to those who wish to understand the format and content of the CAPS PDS data product archive collection.  Typically, these individuals would be software engineers, data analysts, or planetary scientists.

# Archive Volume Generation

## Data Production and Transfer Methods

The CAPS standard product archive collections will be produced by the CAPS instrument team in cooperation with the PDS Planetary Plasma Interactions (PPI) Node at the University of California, Los Angeles (UCLA). The CAPS team is funded by NASA through the Cassini Project office and the PPI activities are funded by the NASA Planetary Data System.

The CAPS team will produce the individual data files and the associated detached PDS labels for each of the standard data products defined in section 2.1 above. There will be up to 4 files per product, per day.  The files will be split into 6 hour periods, with full B-cycles appearing in the file in which the B-cycle starts.  This implies that a few A-cycles at the start of each file may be in the previous 6-hour block file.  However, this implies multi-sensor analysis by assuring that all the data obtained at a given time is in the file with the same time stamp.  The A and B cycle numbers will be the same for all data products, i.e. if an A-cycle of ELS data is missing, the A-cycle numbers in the ELS file will skip the appropriate number.  Additionally, if there are no A-cycles for a given time period then there will not be a gap in the A-cycle number count.

Data files will be flat, binary data files, with a fixed series of values repeated as many times as necessary.   The files will contain data taken at all rates during the period.  If data are collapsed in elevation, counts will be given for the lowest elevation of the collapsed sample and all other elevations will contain fill values.  The fill values as specified in the label files are different for the data products due to differences in maximum values.  If the data are collapsed in energy or azimuth, this will be indicated by the first and last energy step and azimuth values.  This implies that an A-cycle of data contains a variable number of rows, depending on the data rate.  The format of the data can be found in section 5.3, and Table 17 through Table 25.

Data will be written to DVD media in a format compatible with PDS standards.  PPI will assemble the data products into archive volumes so that each volume will contain the interval of data from each data set in multiples of 5 day periods (or only 1 day if 5 days will not fit). The CAPS team will deliver data DVDs to PDS/PPI on a quarterly basis.  Initially only un-calibrated volumes will be available.  CAPS calibrated data volumes will follow shortly.

## Archive Volume Creation and Validation Methods

The archive validation procedure described in this section applies to volumes generated during all phases of the mission.  PPI will collect the data files and labels provided by the CAPS team onto archive volumes. Each archive volume will contain all CAPS data available (either un-calibrated or calibrated) for the time interval covered by the archive volume. Once all of the data files, labels, and ancillary data files are organized onto an archive volume, PPI will add all of the PDS required files (AAREADME, INDEX, ERRATA, etc.) and produce the physical media, which will then be validated.

Data will be validated using the PDS peer review process. The peer review panel will consist of members of the instrument team, the PPI and Central Nodes of the PDS, and at least two outside scientists actively working in the field of magnetospheric physics, especially low energy ion and electron measurements. The PDS personnel will be responsible for validating that the archive volume(s) are fully compliant with PDS standards. The instrument team and outside science reviewers will be responsible for verifying the content of the data set, the completeness of the documentation, and the usability of the data in its archive format. Because of the large volume of the CAPS data, the peer review panel will seek to validate the process by which the data products are produced rather than the data products themselves. This will be accomplished in two phases. First, a specimen volume will be created and manually reviewed for proper structure and completeness of documentation along with the current reference volume. Once the specimen volume is validated, PPI will develop software to validate that subsequent data volumes comply with PDS standards.  After the volume creation software is complete, a volume created by this process will be reviewed again, this time considering all facets of volume usefulness. Any deficiencies in the archive volume will be recorded as liens against the product by the review panel. After all liens placed against the product or the product generation software are resolved, automated production and validation can begin.  Peer review will need to be done on both CAPS archive volumes.

All of the archive files contained on these volumes are verified through the use of the data by the instrument team.  Archive data products are used on a daily basis to generate browse spectrograms.  In addition, selected periods in all modes are examined in depth by the science team as part of science and research activities.  If an error is found, the response will depend on the source of the error. If the error is in the automation software that produced the data product, the error will be fixed and the data product will be reproduced. If there is a correctable error in a data file, the file will be replaced and a new archive volume will be created. If an error in a data file is uncorrectable (i.e., an error in the downlink data file) the error will be described in the cumulative errata file that is included on each volume in the volume set.

Labeling and Identification

Each CAPS standard data product archive volume will bear a unique volume identifier (volume_id) of the form COCAPS_1nnn for CAPS un-calibrated data with calibration information and COCAPS_2mmm for CAPS calibrated data

where CO identifies the spacecraft (Cassini Orbiter), CAPS identifies the instrument, and nnn and mmm are sequential numbers assigned to each volume. The volume_id is used as the label for the physical medium on which the data are stored.

CAPS PDS data set names will conform to the format: CASSINI E/J/S/SW CAPS UNCALIBRATED V<major version>.<minor version> for un-calibrated data and CASSINI E/J/S/SW CAPS CALIBRATED V<major version>.<minor version> for calibrated data.

PDS data set identifiers (dsid) will be abbreviated versions of the data set names formed according to the PDS formation rule for the DATA_SET_ID keyword (see Section 6 of the PDS Standards Reference).  For example, the dsids for the 1.0 version of the CAPS data sets will be CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0, CO-E/J/S/SW-CAPS-3-CALIBRATED-V1.0.

| Table 6: Relationship Between Data Sets, Standard Data Product Types, and Archive Volumes |  |  |
| --- | --- | --- |
| Data Set ID | Product Type | Product Volume Files |
| CO-E/J/S/SW-2-UNCALIBRATED-V1.0 | ELS | ELS_199923000_U1.DAT |
|  | IBS | IBS_199923000_U1.DAT |
|  | IMS Ions (ION) | ION_199923000_U1.DAT |
|  | IMS Singles (SNG) | SNG_199923000_U1.DAT |
|  | IMS Logicals (LOG) | LOG_199923000_U1.DAT |
|  | IMS TOF (TOF) | TOF_199923000_U1.DAT |
|  | Actuator (ACT) | ACT_199923000_1.DAT |
|  | Ancillary (ANC) | ANC_199923000_U1.DAT |
|  | IMS Event Mode (EVN) | EVN_199923000_U1.DAT |
| CO-E/J/S/SW-CAPS-3-CALIBRATED-V1.0 | ELS | ELS_199923000_C1.DAT |
|  | IBS | IBS_199923000_C1.DAT |
|  | IMS ION | ION_199923000_C1.DAT |
|  | IMS SNG | SNG_199923000_C1.DAT |
|  | IMS TOF | TOF_199923000_C1.DAT |
|  | Actuator | ACT_199923000_1.DAT |
|  | Ancillary | ANC_199923000_C1.DAT |

# Archive Volume Contents

This section describes the contents of the CAPS standard product archive collection volumes, including the file names, file contents, file types, and organizations responsible for providing the files.  The complete directory structure is shown in Appendix A.  All the ancillary files described herein appear on each CAPS archive volume, except where noted.  Based on the type of archive volume, the DATA contents will be contain either un-calibrated data or calibrated data.  All other directory contents will remain the same, though the calibration volume will have the most up-to-date calibration documentation.

## Root Directory Contents

The following files are contained in the root directory (for either volume), and are produced by the PPI Node at UCLA. With the exception of the hypertext file and its label, all of these files are required by the PDS Archive Volume organization standards.

| Table 7: Root Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| AAREADME.TXT | This file completely describes the Volume organization and contents (PDS label attached). | PPI |
| AAREADME.HTM | Hypertext version of AAREADME.TXT (top level of HTML interface to the Archive Volume). | PPI |
| AAREADME.LBL | A PDS detached label that describes AAREADME.HTM. | PPI |
| ERRATA.TXT | A cumulative listing of comments and updates concerning all CAPS Standard Data Products on all CAPS Volumes in the Volume set published to date. | PPI |
| VOLDESC.CAT | A description of the contents of this Volume in a PDS format readable by both humans and computers. | PPI |

## INDEX Directory Contents

The following files are contained in the INDEX directory and are produced by the PDS PPI Node. The INDEX.TAB file contains a listing of all data products on the archive volume. In addition, there is a cumulative index file (CUMINDEX.TAB) file that lists all data products in the CAPS archive volume set to date.  The index and index information (INDXINFO.TXT) files are required by the PDS volume standards. The index tables include both required and optional columns.  The cumulative index file is also a PDS requirement; however, this file may not be reproduced on each data volume if its size grows so large as to affect where volume boundaries lie. An online and web accessible cumulative index file will be maintained at the PPI Node while archive volumes are being produced.

| Table 8: Index Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| INDXINFO.TXT | A description of the contents of this directory | PPI |
| INDEX.TAB | A table listing all CAPS Data Products on this Volume | PPI |
| INDEX.LBL | A PDS detached label that describes INDEX.TAB | PPI |

## DOCUMENT Directory Contents

The document directory contains documentation that is considered to be either necessary or simply useful for users to understand the archive data set. These documents are not necessarily appropriate for inclusion in the PDS catalog. Documents may be included in multiple forms (ASCII, PDF, MS Word, HTML with image file pointers, etc.). PDS standards require that any documentation deemed required for use of the data be available in some ASCII format. HTML and PostScript are acceptable as ASCII formats in addition to plain text.

There will be a separate directory for each document that is to be archived.  Each of the document directories will include the document in hypertext (ASCII) and the document in another format (i.e. .DOC or .PDF).  There will also be a single label file that describes all the different formats of the included documents.

The following files are contained in the DOCUMENT directory and are produced or collected by the PPI Node.

| Table 9: Document Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| DOCINFO.TXT | A description of the contents of this directory and all subdirectories. | PPI |
| CAPS_SIS | Directory containing the CAPS archive SIS | CAPS |
| CAPS_CALIB | Directory containing information regarding calibration | CAPS |
| Other Documents | Additional documents describing data processing, etc. | CAPS, PPI |
| Other Document labels | Detached PDS labels for any additional documents | CAPS, PPI |

The following files are contained in the DOCUMENT/CAPS_SIS directory.

| Table 10: Document/CAPS_SIS Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| CAPS_ARCHIVE_SIS.HTM | The Archive Volume SIS (this document) as hypertext | CAPS, PPI |
| CAPS_ARCHIVE_SIS.DOC | The Archive Volume SIS (this document) in Microsoft Word format | CAPS |
| CAPS_ARCHIVE_SIS.ASC | The Archive Volume SIS (this document) in ASCII format | CAPS, PPI |
| CAPS_ARCHIVE_SIS.LBL | A PDS detached label that describes VOLSIS.ASC, VOLSIS.HTM and VOLSIS.DOC. | CAPS, PPI |

The following files are contained in the DOCUMENT/CAPS_CALIB directory.

| Table 11: Document/CAPS_CALIB Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| CAPS_BASIC_CALIB_PROCEDURES.HTM | The CAPS Basic Calibration Procedures document as hypertext | CAPS, PPI |
| CAPS_BASIC_CALIB_PROCEDURES.DOC | The CAPS Basic Calibration Procedures document in Microsoft Word format | CAPS |
| CAPS_BASIC_CALIB_PROCEDURES.ASC | The CAPS Basic Calibration Procedures document in ASCII format | CAPS, PPI |
| CAPS_BASIC_CALIB_PROCEDURES.LBL | A PDS detached label that describes VOLSIS.ASC, VOLSIS.HTM and VOLSIS.DOC. | CAPS, PPI |

## CATALOG Directory Contents

The completed PDS templates in the CATALOG directory provide a top-level understanding of the Cassini/CAPS mission and its data products.  The information necessary to create the files is provided by the CAPS team and formatted into standard template formats by the PPI Node.  The files in this directory are coordinated with PDS data engineers at both the PPI and the PDS Central Nodes.

| Table 12: Catalog Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| CATINFO.TXT | A description of the contents of this directory | PPI |
| CO_CAPS_UNCALIBRATED_DS.CAT | PDS Data Set catalog description of all the CAPS un-calibrated level 2 data files | CAPS |
| CO_CAPS_CALIBRATED_DS.CAT | PDS Data Set catalog description of all the CAPS calibrated level 3 data files | CAPS |
| INSTHOST.CAT | PDS instrument host (spacecraft) catalog description of the Cassini spacecraft | Cassini Project |
| CO_CAPS_INST.CAT | PDS instrument catalog description of the CAPS instrument | CAPS |
| MISSION.CAT | PDS mission catalog description of the Cassini mission | Cassini Project |
| CO_CAPS_PERS.CAT | PDS personnel catalog description of CAPS Team members and other persons involved with generation of CAPS Data Products | CAPS |
| CO_CAPS_REF.CAT | CAPS-related references mentioned in other *.CAT files | CAPS |
| PROJREF.CAT | Mission-related references mentioned in other *.CAT files | Cassini Project |

## DATA (Standard Products) Directory Contents and Naming Conventions

The DATA directory will contain of the following sub-directories, based upon the archive volume: CALIBRATED or UNCALIBRATED.  Data Products produced by the CAPS team are located in subdirectories of either of these two sub-directories in the DATA subdirectory and are of the form YYYYDDD.  Each subdirectory will contain 1 day of data, for all data types.  Multiple YYYYDDD will be written to the disk (in multiples of 5 days), up to the space limitation of the DVD.

### Required Files

The DATA directory will contain a file named DATAINFO.TXT that is an ASCII text description of the directory and subdirectory contents. Every file in the DATA path of an Archive Volume must be described by a PDS label, hence all files in the DATA directory will have external (detached) labels.  Detached PDS label files have the same root name as the file they describe but have the suffix ".LBL".  In directories where there are numerous data files with the same internal table structure, the table column description is included in a single format file (.FMT) that is referenced by a pointer within each PDS label file. This eliminates repetition of information that is not changing within the PDS label files.

### File Naming Conventions

Data products will have names of the following form:

<sensor>_YYYYDDDHH_<DataType><V>.DAT

where

YYYYDDDHH is the start year, day of year, and hour of the data

sensor is  the 3 letter code chosen from the following list:

ELS, IBS, ION, SNG, TOF, LOG, ACT, EVN, and ANC

DataType is a one (1) letter descriptor for the type of data, where C = calibrated and U = un-calibrated.

V is the data version number of the data product.

There is one exception to the naming convention listed above.  Since the actuator (ACT) data product is both calibrated and un-calibrated, we will drop the <DataType> identifier.  Actuator files will have the naming convention of ACT_YYYYDDDHHH_<V>.DAT.

Since the data files are 6 hour files, HH will only have the valid values of 00, 06, 12, and 18.

Not every combination of sensor and DataType is a valid filename.  Valid combinations can be determined by using the information contained in Table 5.

When data is updated within a specific type of format the data version number will be incremented. When more than nine versions are required, the characters a-z are used to represent further versions.

### DATA/UNCALIBRATED/YYYYDDD Directory Contents

Un-calibrated data files starting on YYYYDDD from all sensors will be stored in the DATA/UNCALIBRATED/YYYYDDD directory.  Each directory will contain one day of data.  Each sensor can have up to 4 files for the day and each sensor file can contain up to 6 hours of data.  The file naming convention is described in Section 4.5.2.  Every data file in the directory will have a detached PDS label with the same root name as the file they describe but have the suffix “.LBL”.  In addition, there will be a brief ASCII text file (INFO.TXT) that describes the DATA/UNCALIBRATED/YYYYDDD directory contents, which are listed in Table 13.  In addition, each YYYYDDD directory will have its own set of format files.  NOTE:  Files will only be available if data from of the appropriate type (during the 6 hour block in question) is available.  Also, we do not take very much event mode data (EVN), so these files are not available very frequently.

| Table 13: YYYYDDD UNCALIBRATED Data Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| DATAINFO.TXT | Brief description of directory contents and naming conventions. | PPI |
| ELS*.DAT | ELS sensor data files. | CAPS |
| ELS*.LBL | PDS label for ELS sensor files of same base name. | CAPS |
| IBS*.DAT | IBS sensor data files. | CAPS |
| IBS*.LBL | PDS label for IBS sensor files of same base name. | CAPS |
| SNG*.DAT | IMS Singles (SNG) sensor data files. | CAPS |
| SNG*.LBL | PDS label for SNG files of same base name. | CAPS |
| LOG*.DAT | IMS Logicals (LOG) data files. | CAPS |
| LOG*.LBL | PDS label for LOG files of same base name. | CAPS |
| ION*.DAT | IMS Ions (ION) data files. | CAPS |
| ION*.LBL | PDS label for ION files of same base name. | CAPS |
| TOF*.DAT | IMS Time of Flight (TOF) data files. | CAPS |
| TOF*.LBL | PDS label for TOF files of same base name. | CAPS |
| ACT*.DAT | Actuator (ACT) data files. | CAPS |
| ACT*.LBL | PDS label for ACT files of same base name. | CAPS |
| ANC*.DAT | Ancillary (ANC) data files. | CAPS |
| ANC*.LBL | PDS label for ANC files of same base name. | CAPS |
| EVN*.DAT | Event Mode (EVN) data files. | CAPS |
| EVN*.LBL | PDS label for EVN files of same base name. | CAPS |
| ELS_U1.FMT | PDS format file containing the data file structure for the ELS file format. | CAPS |
| IBS_U2.FMT | PDS format file containing the data file structure for the IBS file format. | CAPS |
| SNG_U1.FMT | PDS format file containing the data file structure for the SNG file format. | CAPS |
| LOG_U1.FMT | PDS format file containing the data file structure for the LOG file format. | CAPS |
| ION_U1.FMT | PDS format file containing the data file structure for the ION file format. | CAPS |
| TOF_U1.FMT | PDS format file containing the data file structure for the TOF file format. | CAPS |
| ACT_1.FMT | PDS format file containing the data file structure for the ACT file format. | CAPS |
| ANC_U1.FMT | PDS format file containing the data file structure for the ANC file format. | CAPS |
| EVN_U1.FMT | PDS format file containing the data file structure for the EVN file format. | CAPS |

### DATA/CALIBRATED/YYYYDDD Directory Contents

Calibrated data files starting on YYYYDD from all sensors will be stored in the DATA/CALIBRATED/YYYYDDD directory.  Each directory will contain one day of data.  Each type of calibrated file can have up to 4 files for the day and can contain up to 6 hours of data.  The file naming convention is described in Section 4.5.2. Every data file in the directory will have a detached PDS label with the same root name as the file they describe but have the suffix “.LBL”.  In addition, there will be a brief ASCII text file (INFO.TXT) that describes the DATA/CALIBRATED/YYYYDDD directory contents, which are briefly listed in Table 14.  Each YYYYDDD directory will contain a set of its own format files.

In the label file for calibrated data, there will be pointers to the file containing the appropriate algorithms and parameters for the CALIBRATED data.  When calibration changes, the pointers in the labels will be updated to include the correct links.

| Table 14: YYYYDDD CALIBRATED Data Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| DATAINFO.TXT | Brief description of directory contents and naming conventions. | PPI |
| ELS*.DAT | ELS Calibrated data files. | CAPS |
| ELS*.LBL | PDS label for ELS calibrated data files of same base name. | CAPS |
| IBS*.DAT | IBS Calibrated data files. | CAPS |
| IBS*.LBL | PDS label for IBS calibrated data files of same base name. | CAPS |
| SNG*.DAT | IMS SNG Calibrated data files. | CAPS |
| SNG*.LBL | PDS label for IMS SNG calibrated data files of same base name. | CAPS |
| ION*.DAT | IMS ION Calibrated data files. | CAPS |
| ION*.LBL | PDS label for IMS ION calibrated data files of same base name. | CAPS |
| TOF*.DAT | IMS TOF Calibrated data files. | CAPS |
| TOF*.LBL | PDS label for IMS TOF calibrated data files of same base name. | CAPS |
| ANC*.DAT | Ancillary Calibrated data files. | CAPS |
| ANC*.LBL | PDS label for Ancillary calibrated data files of same base name. | CAPS |
| ELS_CAL_U0.FMT | PDS format file containing the data file structure for the ELS calibrated data file format. | CAPS |
| IBS_CAL_U0.FMT | PDS format file containing the data file structure for the IBS calibrated data file format. | CAPS |
| SNG_CAL_U0.FMT | PDS format file containing the data file structure for the IMS SNG calibrated data file format. | CAPS |
| ION_CAL_U0.FMT | PDS format file containing the data file structure for the IMS ION calibrated data file format. | CAPS |
| TOF_CAL_U0.FMT | PDS format file containing the data file structure for the IMS ION calibrated data file format. | CAPS |
| ANC_CAL_U0.FMT | PDS format file containing the data file structure for the Ancillary calibrated data file format. | CAPS |

## CALIB Directory Contents

Given that we will be archiving data to 2 different volumes, the contents of the CALIB directory will include the following information for the un-calibrated archive volume.  Please note that the documentation for CAPS basic calibration procedures can be found in the DOCUMENT/CAPS_CALIB directory.

| Table 15: CALIB Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| CALINFO.TXT | A description of the contents of this directory and all subdirectories. | PPI |
| SAMPLE_DATA | A directory that contains a sample input data file, additional files needed for the calibration process, and a sample output file. | CAPS |
| ELS_ENERGY_ARRAY.TAB | The ELS Sweep Table calibration data | CAPS |
| ELS_ENERGY_ARRAY.LBL | A PDS detached label that describes ELS_ENERGY_ARRAY.TAB | CAPS |
| ELS_GEOM_FACTOR.TAB | The ELS Geometric Factor matrix (see label for full description) | CAPS |
| ELS_GEOM_FACTOR.LBL | A PDS detached label that describes ELS_GEOM_FACTOR.TAB | CAPS |
| ELS_SWEEP_TABLE_ALL_VER.TAB | The ELS Sweep Table for all CAPS data | CAPS |
| ELS_SWEEP_TABLE_ALL_VER.LBL | A PDS detached label that describes ELS_SWEEP_TABLE_ALL_VER.TAB | CAPS |
| IBS_SWEEP_V0_V1_V2.TAB | The IBS Sweep Table for versions 0, 1, and 2 of the CAPS data | CAPS |
| IBS_SWEEP_V0_V1_V2.LBL | A PDS detached label that describes IBS_SWEEP_V0_V1_V2.TAB | CAPS |
| IBS_SWEEP_V3.TAB | The IBS Sweep Table for version 3 of the CAPS data | CAPS |
| IBS_SWEEP_V3.LBL | A PDS detached label that describes IBS_SWEEP_V3.TAB | CAPS |
| IMS_SWEEP_TABLE_0_V0_V1_V2.TAB | The IMS Sweep Table number 0 for versions 0, 1, and 2 of the CAPS data | CAPS |
| IMS_SWEEP_TABLE_0_V0_V1_V2.LBL | A PDS detached label that describes IMS_SWEEP_TABLE_0_V0_V1_V 2.TAB | CAPS |
| IMS_SWEEP_TABLE_16.TAB | The IMS Sweep Table number 16 for all versions of CAPS data. The sweep table has been used for calibrations. | CAPS |
| IMS_SWEEP_TABLE_16.LBL | A PDS detached label that describes IMS_SWEEP_TABLE_16.TAB | CAPS |
| IMS_SWEEP_TABLE_15.TAB | The IMS Sweep Table number 15 for all versions of CAPS data. This sweep table is used only during some Titan flyby periods (less than 1400km) | CAPS |
| IMS_SWEEP_TABLE_15.LBL | A PDS detached label that describes IMS_SWEEP_TABLE_15.TAB | CAPS |
| IMS_SWEEP_TABLE_255.TAB | The IMS Sweep Table number 255 for all versions of CAPS data. This sweep table was used only once, and has been replaced by #15. | CAPS |
| IMS_SWEEP_TABLE_255.LBL | A PDS detached label that describes IMS_SWEEP_TABLE_255.TAB | CAPS |
| ION_AND_GROUPTABLE_NAMING.DOC | Contains the definitions of the group table naming and ion naming in Microsoft Word format | CAPS |
| ION_AND_GROUPTABLE_NAMING.PDF | Contains the definitions of the group table naming and ion naming in Adobe Acrobat format | CAPS |
| ION_AND_GROUPTABLE_NAMING.LBL | A PDS detached label that describes the documents ION_AND_GROUPTABLE_NAMING.* | CAPS |

### CALIB/SAMPLE_DATA Directory Contents

This directory will contain a sample input file, any additional files necessary for the calibration process, and a sample output file.  The goal of files in this directory is to provide data users an example against which to test their calibration routines, which were developed according to the CAPS BASIC CALIB PROCEDURES document (which can be found in DOCUMENT/CAPS_CALIB).  Please note that the output will only include first order calibration, and not the second order corrections that are currently being worked.

## EXTRAS Directory Contents

The EXTRAS directory will contain an EXTRINFO.TXT file that contains a description of the contents of this directory.  Additional files will include example software to read the CAPS un-calibrated data files, open the necessary calibration files, calibrate the data, and write them out.  Example software for generating the CAPS browse spectrograms will also be provided.

## BROWSE Directory Contents

The BROWSE directory will contain browse spectrogram plots that are not intended for publication.  Browse spectrograms starting on YYYYDDD from all sensors will be stored in the BROWSE/YYYYDDD directory.  Each directory will contain one day of data.  Each sensor can have up to 4 spectrograms for the day and can contain up to 6 hours of data.  The file naming convention is described in Section 4.5.2, with a .PNG extension to specify the file format.  Every data file in the directory will have a detached PDS label with the same root name as the file they describe but have the suffix “.LBL”.  In addition, there will be a brief ASCII text file (INFO.TXT) that describes the BROWSE/YYYYDDD directory contents, which are listed in Table 16: YYYYDDD BROWSE Directory Contents.  NOTE:  Files will only be available if data from of the appropriate type (during the 6 hour block in question) is available.  We do not plot ancillary data.

| Table 16: YYYYDDD BROWSE Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| DATAINFO.TXT | Brief description of directory contents and naming conventions. | PPI |
| ACT*.PNG | Actuator plot in PNG format | CAPS |
| ACT*.LBL | PDS label for actuator PNG formatted file of same base name | CAPS |
| ELS*.PNG | ELS plot in PNG format | CAPS |
| ELS*.LBL | PDS label for ELS PNG formatted file of same base name | CAPS |
| IBS*.PNG | IBS plots in PNG format | CAPS |
| IBS*.LBL | PDS label for IBS PNG formatted file of same base name | CAPS |
| ION*.PNG | IMS ION plots in PNG format | CAPS |
| ION*.LBL | PDS label for IMS ION PNG formatted file of same base name | CAPS |
| LOG*.PNG | IMS logicals plot in PNG format | CAPS |
| LOG*.LBL | PDS label for IMS Logicals PNG formatted file of same base name | CAPS |
| SNG*.PNG | IMS singles plot in PNG format | CAPS |
| SNG*.LBL | PDS label for IMS Singles PNG formatted file of same base name | CAPS |
| TOF*.PNG | IMS TOF plot in PNG format | CAPS |
| TOF*.LBL | PDS label for IMS TOF PNG formatted file of same base name | CAPS |

Since we will be archiving our calibrated files on a separate volume, this volume will not contain a DATA/CALIBRATED directory.  When ready, the calibration data will be available in the DATA/CALIBRATED directory.  On the calibrated archive volume, the CALIB directory will contain files that are used in the calibration process.  The files will include only text files and tables.  Any other calibration files will be included in the DOCUMENT/CAPS_CALIB directory.  Contents are still TBD and will be specified when the calibration volume is ready.

# Archive Volume Format

This section describes the format of CAPS standard product archive volumes.  Data that comprise the CAPS standard product archives will be formatted in accordance with Planetary Data System specifications [Planetary Science Data Dictionary, 2002; PDS Data Preparation Workbook, 1995; PDS Standards Reference, 2002].

## Disk Format

Disk formats for the archive volumes will conform to the PDS standard for the applicable media. At present, the plan is to archive CAPS data on DVD-R media. The PDS standard for DVD-R media disk format is UDF-Bridge.

## File Formats

The following section describes file formats for the kinds of files contained on Archive Volumes. For more information, see the PDS Standards Reference.

### Document File Formats

Document files with the .TXT suffix exist in all directories.  They are ASCII files with embedded PDS labels. All document files contain variable-length, 80-byte maximum records, with a carriage return character (ASCII 13) in the 79th byte and a line feed character (ASCII 10) in the 80th byte. This allows the files to be read by the MacOS, DOS, Windows, UNIX, OS2, and VMS operating systems.

However, the documents in the reference volume contain formatting and figures that cannot be rendered as pure ASCII text.   These documents will be provided in formats that support graphics, such as HTML, MS Word, PDF, etc.  The PDS requirement that all documentation critical to the understanding of the data set be provided in ASCII text form will be met by the inclusion of HTML formatted documents.

### Catalog File Formats

Catalog files (suffix .CAT) exist in the Root and Catalog directories.  They are formatted in an object-oriented structure consisting of sets of 'keyword = value' declarations. All files are ASCII and conform to the same structure standards (line length, line terminator) as the PDS label files described in the previous section.

### PDS Label File Formats

All data files in the CAPS Standard Product Archive Collection have PDS labels [Planetary Science Data Dictionary; PDS Standards Reference].  These labels are all detached from the data files (same file name prefix, .LBL suffix).

A PDS label, whether embedded or detached from its associated file, provides descriptive information about the associated file.  The PDS label is an object-oriented structure consisting of sets of 'keyword = value' declarations.  The object that the label refers to (e.g., TABLE, STRUCTURE, etc.) is denoted by a statement of the form:

^object = location

in which the carat character (^, also called a pointer in this context) indicates where to find the object.  In a PDS label, the location denotes the name of the file containing the object, along with the starting record or byte number, if there is more than one object in the file.  For example:

^HEADER = ("98118.TAB",1)

^TABLE = ("98118.TAB",1025 <BYTES>)

indicates that the HEADER object begins at record 1 and that the TABLE object begins at byte 1025 of the file 98118.TAB.  The file 98118.TAB must be located in the same directory as the detached label file.

Below is a list of the possible formats for the ^object definition in labels in this product.

^object	= n

^object	= n <BYTES>

^object	= "filename.ext"

^object	= ("filename.ext", n)

^object	= ("filename.ext", n <BYTES>)

where

n is the starting record or byte offset of the object, counting from the beginning of the file (record 1,
byte 1),

<BYTES> indicates that the number given is in units of bytes (the default is records),

filename is the up-to-8-character, alphanumeric upper-case file name,

ext is the up-to-3-character upper-case file extension.

All CAPS detached labels will conform to the requirement of less than 80-byte per line, including the carriage return character (ASCII 13) and the line feed character (ASCII 10).  The RECORD_TYPE of all the labels is STREAM.

### Data File Formats – Binary Tables

All of the data files for CAPS are binary tables of data (.DAT suffix).  Data files can be found in the YYYYDDD directories, which are located in DATA/CALIBRATED and DATA/UNCALIBRATED.  Missing data are filled with appropriate (and documented) fill values.  The table format for each sensor is described by a detached PDS label of the same base name as the file, but with an .LBL extension.  A description of the data file contents and structure for the standard data set data products can be found in the following sections: 5.3 and 5.4.  The format for the detached labels and format files can be found in Appendix B. PDS Labels & Format Files for Standard UNCALIBRATED Data Products.

## CAPS Standard UNCALIBRATED Data Product Descriptions

The following sections describe the content and structure of each of the standard data products within the UNCALIBRATED level 2 CAPS data set.

### CAPS ELS Data Product Format

The data product format for ELS is listed in Table 17 below.  The fill value for ELS data is 65535 (hex value FFFF).

| Table 17: CAPS ELS UNCALIBRATED Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of A cycle, sec. from J2000 (barycentric dynamic time) |
| Telemetry mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| Collapse flag | Unsigned Integer | 1 | [0,131] | Collapse flag indicates collapse by average (0), sum (1), average with in-flight dead-time correction (2), or sum with in-flight dead-time correction (3). If the most significant bit is 1, it will indicate no HK was available. |
| Offset time | Unsigned Integer | 2 | [0,32000] | Milliseconds from start of A cycle |
| First Energy Step | Unsigned Integer | 2 | [1,63] | Min energy step in collapsed data |
| Last Energy Step | Unsigned Integer | 2 | [1,63] | Max energy step in collapsed data |
| First Azimuth Value | Unsigned Integer | 2 | [1,16] | Min azimuth value in collapsed data |
| Last Azimuth Value | Unsigned Integer | 2 | [1,16] | Max azimuth value in collapsed data |
| Data, Elevation 1 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 1 |
| Data, Elevation 2 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 2 |
| Data, Elevation 3 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 3 |
| Data, Elevation 4 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 4 |
| Data, Elevation 5 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 5 |
| Data, Elevation 6 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 6 |
| Data, Elevation 7 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 7 |
| Data, Elevation 8 | Unsigned Integer | 2 | [0,65504] | Counts in elevation 8 |

### CAPS IBS Data Product Format

The data product format for CAPS IBS is listed in Table 18 below.  The fill value for IBS data is 65535 (hex value FFFF).

| Table 18: CAPS IBS UNCALIBRATED Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of C cycle, sec. from J2000 (barycentric dynamic time) |
| Telemetry mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| IBS mode/submode | Unsigned Integer | 1 | [0,255] | IBS mode and submode flag: 0 = Standard Sweep Collapse, 1 = Standard Sweep Snapshot, 2 = Solar Wind Search, 3 = Solar Wind Track, 4 = Magnetosphere Search, 5 = Magnetosphere Survey, 6 = Calibration Mode, 7-255 = spare. |
| Offset time | Unsigned Integer | 4 | [1,256000] | Milliseconds from start of C cycle |
| First Energy Step | Unsigned Integer | 2 | [1,852] | Min energy step in collapsed data (index into the energy table) |
| Last Energy Step | Unsigned Integer | 2 | [1,852] | Max energy step in collapsed data (index into the energy table) |
| First Azimuth Value | Unsigned Integer | 2 | [1,128] | Min azimuth value in collapsed data |
| Last Azimuth Value | Unsigned Integer | 2 | [1,128] | Max azimuth value in collapsed data |
| Data, Fan 1 | Unsigned Integer | 2 | [1,65504] | Counts in fan 1 |
| Data, Fan 2 | Unsigned Integer | 2 | [1,65504] | Counts in fan 2 |
| Data, Fan 3 | Unsigned Integer | 2 | [1,65504] | Counts in fan 3 |

### CAPS IMS ION Data Product Format

The data product format for CAPS IMS ION is listed in Table 19 below.  The fill value for IMS Ion data is 28671 (hex value 6FFF).

| Table 19: CAPS UNCALIBRATED IMS ION Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information was available |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of A cycle, sec. from J2000 (barycentric dynamic time) |
| Telemetry mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| Spare | Unsigned Integer | 1 | 0 | Spare bits to keep on even byte boundaries |
| Offset time | Unsigned Integer | 2 | [1,32000] | Milliseconds from start of A cycle |
| First Energy Step | Unsigned Integer | 2 | [1,63] | Min energy step in collapsed data |
| Last Energy Step | Unsigned Integer | 2 | [1,63] | Max energy step in collapsed data |
| First Azimuth Value | Unsigned Integer | 2 | [1,8] | Min azimuth value in collapsed data |
| Last Azimuth Value | Unsigned Integer | 2 | [1,8] | Max azimuth value in collapsed data |
| Sam Ion number | Unsigned Integer | 2 | [0,65535] | SAM ion number |
| Data, Elevation 1 | Integer | 2 | [-32,27650] | Counts in elevation 1 (**) |
| Data, Elevation 2 | Integer | 2 | [-32,27650] | Counts in elevation 2 (**) |
| Data, Elevation 3 | Integer | 2 | [-32,27650] | Counts in elevation 3 (**) |
| Data, Elevation 4 | Integer | 2 | [-32,27650] | Counts in elevation 4 (**) |
| Data, Elevation 5 | Integer | 2 | [-32,27650] | Counts in elevation 5 (**) |
| Data, Elevation 6 | Integer | 2 | [-32,27650] | Counts in elevation 6 (**) |
| Data, Elevation 7 | Integer | 2 | [-32,27650] | Counts in elevation 7 (**) |
| Data, Elevation 8 | Integer | 2 | [-32,27650] | Counts in elevation 8 (**) |

(**):  Note that due to on-board spacecraft de-convolution routines used to estimate the number of counts from a particular species, a combination of low counts and background noise can cause the de-convolution routine to give negative numbers.

### CAPS IMS SNG Data Product Format

The data product format for CAPS IMS Singles (SNG) is listed in Table 20 below.  The fill value for Singles data is 65535 (hex value FFFF).

| Table 20: CAPS UNCALIBRATED IMS Singles Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information was available |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of A cycle, sec. from J2000 (barycentric dynamic time) |
| Telemetry mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| Spare | Unsigned Integer | 1 | 0 | Spare byte to have even byte boundaries |
| Offset time | Unsigned Integer | 2 | [1,32000] | Milliseconds from start of A cycle |
| First Energy Step | Unsigned Integer | 2 | [1,63] | Min energy step in collapsed data |
| Last Energy Step | Unsigned Integer | 2 | [1,63] | Max energy step in collapsed data |
| First Azimuth Value | Unsigned Integer | 2 | [1,8] | Min azimuth value in collapsed data |
| Last Azimuth Value | Unsigned Integer | 2 | [1,8] | Max azimuth value in collapsed data |
| Data, Elevation 1 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 1 |
| Data, Elevation 2 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 2 |
| Data, Elevation 3 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 3 |
| Data, Elevation 4 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 4 |
| Data, Elevation 5 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 5 |
| Data, Elevation 6 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 6 |
| Data, Elevation 7 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 7 |
| Data, Elevation 8 | Unsigned Integer | 2 | [0,27500] | Counts in elevation 8 |

### CAPS IMS LOG Data Product Format

The data product format for CAPS IMS Logicals (LOG) is listed in Table 21 below.  The fill value for Logical Data is 65535 (hex FFFF).

| Table 21: CAPS IMS Logicals UNCALIBRATED Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information was available |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of A cycle, sec. from J2000 (barycentric dynamic time) |
| Telemetry mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| TDC log selection | Unsigned Integer | 1 | [0,3] | TDC selectable logical definition 0 = (Logical 13: Start CFD Singles, Logical 14: Stop CFD Singles), 1 = (Logical 13: Acquisition, Logical 14: Deadtimes), 2 = (Logical 13: Single TOF events, Logical 14: Double TOF events), 3 = (Logical 13: Data strobes, Logical 14: Resets) |
| Offset time | Unsigned Integer | 2 | [1,32000] | Milliseconds from start of A cycle |
| First Energy Step | Unsigned Integer | 2 | [1,63] | Min energy step in collapsed data |
| Last Energy Step | Unsigned Integer | 2 | [1,63] | Max energy step in collapsed data |
| First Azimuth Value | Unsigned Integer | 2 | [1,8] | Min azimuth value in collapsed data |
| Last Azimuth Value | Unsigned Integer | 2 | [1,8] | Max azimuth value in collapsed data |
| LEF Stops | Unsigned Integer | 2 | [0,27500] | LEF stop counts |
| ST Stops | Unsigned Integer | 2 | [0,27500] | ST stop counts |
| Timeouts | Unsigned Integer | 2 | [0,27500] | Timeout events |
| Total Events | Unsigned Integer | 2 | [0,27500] | Total events (generated by SAM for dead time) |
| Logical 13 | Unsigned Integer | 2 | [0,27500] | TDC selectable logical 13 |
| Logical 14 | Unsigned Integer | 2 | [0,27500] | TDC selectable logical 14 |

### CAPS IMS TOF Data Product Format

The data product format for CAPS IMS Time of flight (TOF) is listed in Table 22 below.  The fill value for IMS TOF and ST data is 4294967295  (hex value FFFFFFFF).

| Table 22: CAPS IMS TOF UNCALIBRATED Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of B cycle, sec. from J2000 (barycentric dynamic time) |
| Telemetry mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| Collapse Flag | Unsigned Integer | 1 | [0,1] | Flags indicating collapse by average (0) or sum (1) |
| ST start channel | Unsigned Integer | 2 | [1,2048] | Start ST TOF channel |
| ST interval | Unsigned Integer | 1 | [1,4] | ST TOF bin interval 1 = each word is taken starting at the Start channel. 2 = Every other word is taken starting at the Start channel. 4 = Every fourth word is taken starting at the Start Channel |
| ST energy collapse | Unsigned Integer | 1 | [0,3] | ST energy collapse option 0 = sum adjacent energies, 1 = take even energies, 2 = take odd energies, 3 = TBA (to be assigned). |
| LEF start channel | Unsigned Integer | 2 | [1,2048] | Start LEF TOF channel |
| LEF interval | Unsigned Integer | 1 | [1,4] | LEF TOF bin interval 1 = each word is taken starting at the Start channel. 2 = Every other word is taken starting at the Start channel. 4 = Every fourth word is taken starting at the Start Channel |
| LEF energy collapse | Unsigned Integer | 1 | [0,3] | LEF energy collapse option 0 = sum adjacent energies, 1 = take even energies, 2 = take odd energies, 3 = TBA. |
| Energy Step | Unsigned Integer | 2 | [1,32] | Energy step in collapsed data |
| Data, ST TOF bin 1 | Unsigned Integer | 4 | [0, 3268027] | Counts in ST TOF bin 1 |
| Data, ST TOF bin 2 | Unsigned Integer | 4 | [0, 3268027] | Counts in ST TOF bin 2 |
| … | Unsigned Integer | 4x509 | [0, 3268027] | Counts in ST TOF bins 3 - 511 |
| Data, ST TOF bin 512 | Unsigned Integer | 4 | [0, 3268027] | Counts in ST TOF bin 512 |
| Data, LEF TOF bin 1 | Unsigned Integer | 4 | [0, 3268027] | Counts in LEF TOF bin 1 |
| Data, LEF TOF bin 2 | Unsigned Integer | 4 | [0, 3268027] | Counts in LEF TOF bin 2 |
| … | Unsigned Integer | 4x509 | [0, 3268027] | Counts in LEF TOF bins 3 - 511 |
| Data, LEF TOF bin 512 | Unsigned Integer | 4 | [0, 3268027] | Counts in LEF TOF bin 512 |

### CAPS ACT Data Product Format

The data product format for the CAPS actuator is listed in Table 23 below.  The fill value for actuator data is –999.0.  Actuator data products are considered to be both calibrated and un-calibrated data products.  In order to accommodate this, we lose the <DataType> in the filename (as described in section 4.5.2).

| Table 23: CAPS ACT Data File Contents and Structure (both Calibrated & Un-calibrated) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of A cycle, sec. from J2000 (barycentric dynamic time) |
| Data, Actuator angle 1 | Float | 4 | [-115,115] | Actuator angle at time + 0 sec |
| Data, Actuator angle 2 | Float | 4 | [-115,115] | Actuator angle at time + 1 sec |
| … | Float | 4x29 | [-115,115] | Actuator angle (offset times of 2 – 30 sec) |
| Data, Actuator angle 32 | Float | 4 | [-115,115] | Actuator angle at time + 31 sec |

### CAPS ANC Data Product Format

The data product format for the ancillary data product is listed in Table 24 below.  There are no standard fill values for these items.

| Table 24: CAPS ANC UNCALIBRATED Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of A cycle, sec. from J2000 (barycentric dynamic time) |
| SCLK | Unsigned Integer | 4 | [0,3.0x109] | Start time of A cycle, spacecraft clock |
| Spacecraft/Saturn position [x] | Float | 4 | [-9.46x1012, 9.46x1012] | J2000 [km]: Saturn-centered |
| Spacecraft/Saturn position [y] | Float | 4 | [-9.46x1012, 9.46x1012] | J2000 [km]: Saturn-centered |
| Spacecraft/Saturn position [z] | Float | 4 | [-9.46x1012, 9.46x1012] | J2000 [km]: Saturn-centered |
| Spacecraft/Saturn velocity vx | Float | 4 | [-3x105, 3x105] | J2000 [km/s]: relative to Saturn |
| Spacecraft/Saturn velocity vy | Float | 4 | [-3x105, 3x105] | J2000 [km/s]: relative to Saturn |
| Spacecraft/Saturn velocity vz | Float | 4 | [-3x105, 3x105] | J2000 [km/s]: relative to Saturn |
| Spacecraft/Sun position [x] | Float | 4 | [-9.46x1012, 9.46x1012] | J2000 [km]: Sun-centered |
| Spacecraft/Sun position [y] | Float | 4 | [-9.46x1012, 9.46x1012] | J2000 [km]: Sun-centered. |
| Spacecraft/Sun position [z] | Float | 4 | [-9.46x1012, 9.46x1012] | J2000 [km]: Sun-centered |
| Spacecraft/Sun velocity vx | Float | 4 | [-3x105, 3x105] | J2000 [km/s]: Relative to the Sun |
| Spacecraft/Sun velocity vy | Float | 4 | [-3x105, 3x105] | J2000 [km/s]: Relative to the Sun |
| Spacecraft/Sun velocity vz | Float | 4 | [-3x105, 3x105] | J2000 [km/s]: Relative to the Sun |
| Spacecraft orientation [xx] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [xy] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [xz] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [yx] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [yy] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [yz] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [zx] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [zy] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| Spacecraft orientation [zz] | Float | 4 | [-1,1] | Component of rotation matrix to J2000. Fill value = 2. |
| ELS quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| IBS quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| IMS Ion quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| IMS TOF LEF quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| IMS TOF ST quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| IMS Logicals quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| IMS Singles quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| Actuator quality flag | Unsigned Integer | 1 | [0,7] | Missing data and good/bad checksum 0=Everything OK, 1 = Missing Data, 2 = Bad Checksum, 3 = Missing Data & Bad Checksum, 7 = No Data (4,5,6 not valid) |
| Actuator Status Bits (all 32 of them) | Unsigned Integer | 32 | [0,8] | Status bits for the actuator data product. These are represented as 32 bytes with the following values: 0 = Everything is OK 4 = Limit Switch has been hit at +108 degrees 8 = Limit Switch has been hit at -108 degrees 16 = Data not available (data is only available in 16, 8, 4, and 2 kbps modes) |
| TLM Version | Unsigned Integer | 1 | [0,15] | Telemetry Mode version number |
| FSW Major version | Unsigned Integer | 1 | [0,255] | To build the flight software version number: Major.SubMajor.Minor.SubMinor. For example: 3.1.0.2 |
| FSW Sub-Major version | Unsigned Integer | 1 | [0,255] | See description for FSW Major version |
| FSW Minor version | Unsigned Integer | 1 | [0,255] | See description for FSW Major version |
| FSW Sub-Minor version | Unsigned Integer | 1 | [0,255] | See description for FSW Major version |
| Spacecraft pointing type | Unsigned Integer | 1 | [0,2] | 0 = no pointing available, 1 = pointing based on predicts, 2 = pointing based on reconstructs |
| Telemetry rate and mode | Unsigned Integer | 1 | [1,136] | Logical telemetry rate and mode: 1 = 250bps, 2 = 500bps, 4 = 1kbps, 8 = 2kbps, 16 = 4kbps, 32 = 8kbps, 64 = 16kbps, 130 = 500bps solar wind, 132 = 1 kbps solar wind, 136 = 2kbps solar wind. |
| IBS Sweep Table & Index Table Numbers | Unsigned Integer | 1 | [0,250] | The upper 4 bits are the IBS index table, and the lower 4 bits are the IBS sweep table number. (fill 0xFF) |
| IBS Background, Fan 1 | Unsigned Integer | 2 | [0,60000] | IBS Background counts in fan 1 (fill 0xFFFF) |
| IBS Background, Fan 2 | Unsigned Integer | 2 | [0,60000] | IBS Background counts in fan 2 (fill 0xFFFF) |
| IBS Background, Fan 3 | Unsigned Integer | 2 | [0,60000] | IBS Background counts in fan 3 (fill 0xFFFF) |
| IBS starting energy | Unsigned Integer | 2 | [1,852] | IBS starting energy step number (fill 0xFFFF) |
| IBS Subcycle | Unsigned Integer | 1 | [0,7] | IBS subcycle counter (A cycle in C cycle) (fill 0xFF) |
| IBS compression ratio | Unsigned Integer | 1 | [1,32] | Uncompressed/compressed length. This ratio is calculated on the ground from information in the IBS header and rounded down to the nearest integer. (fill 0x0) |
| IBS Peak Fan | Unsigned Integer | 1 | [1,3] | Fan containing the IBS peak (1st in the C cycle). (fill 0x4) |
| IBS Peak A cycle | Unsigned Integer | 1 | [1,8] | A cycle number (1st in the C cycle). (fill 0x9) |
| IBS Peak Sweep | Unsigned Integer | 1 | [1,16] | IBS peak energy sweep or azimuth (1st in the C cycle). (fill 0x0) |
| IBS Peak Energy Step | Unsigned Integer | 1 | [0,255] | IBS peak energy step (1st in the C cycle). (fill 0x0) |
| IBS Threshold Run Length | Unsigned Integer | 2 | [0,255] | Run length compression threshold (fill 0xFFFF) |
| IMS sweep table number | Unsigned Integer | 1 |  | IMS Sweep table number |
| TDC Single Select | Unsigned Integer | 1 | [0,3] | Determines how singles 13 and 14 are set (these are also Logical 13 and Logical 14): Value: Single 13 Single 14 0 Start CFD Stop CFD 1 Acquisition Error Deadtimes 2 Single TOF’s Double TOF’s 3 Data Strobes Resets |
| IMS logicals selection | Unsigned Integer | 2 | [4096,27416] | The TDC logicals selection is a bitmap: Bits 15-13: IMS Logical 1 Bits 12-10: IMS Logical 2 Bits 9-7: IMS Logical 3 Bits 6-4: IMS Logical 4 Bits 3-0: Unused Logical selection decoder: 0 = Unused 1 = LEF Stop 2 = ST Stop 3 = Timeouts 4 = Total Events (As used in SAM dead time correction) 5 = Logical 13 6 = Logical 14 7 = Unused NOTE: Logical 13 and 14 are set with 82TDC_ENG_SING. See previous column. |
| SAM/CPU2 status flags | Unsigned Integer | 1 | [0,255] | Bitmap: Bit 7 is most significant bit. 7 = CPU2/SAM mode change 6 = Background data 5 = Ion deadtime compensation 4 = SAM LEF enable 3 = SAM molecule enable 2 = SW/HW binning 1-0 = HW binning LUT index |
| SAM Ion selection index | Unsigned Integer | 1 | [0,255] | SAM ion selection index |
| SAM Ion group table | Unsigned Integer | 2 | [0,65535] | SAM group table ID number |
| ELS_MCP_ADJ | Float | 4 | [0.0,3700.0] | ELS High voltage adjust (Volts). FILL value is –1.0 |
| IBS_CEM_DAC | Float | 4 | [-4000.0,0.0] | IBS CEM High Voltage Digital to Analog Converter (Volts). FILL value is 1.0 |
| HVU1_RET_DAC | Float | 4 | [0,16.0] | HVU1 Retarding High Voltage Digital to Analog Converter (kVolts). FILL is –1.0 |
| HVU1_ACC_DAC | Float | 4 | [-16.0,0.0] | HVU1 Accelerating High Voltage Digital to Analog Converter (kVolts). FILL is 1.0 |
| HVU2_ST_DAC | Float | 4 | [-3600.0,0.0] | HVU2 ST MCP Digital to Analog Converter (Volts). FILL is 1.0 |
| HVU2_LEF_DAC | Float | 4 | [-2400.0,0.0] | HVU2 LEF MCP Digital to Analog Converter (Volts). FILL is 1.0 |

### CAPS EVN Data Product Format

The data product format for the CAPS IMS event mode data is listed in Table 25 below.  No fill values are necessary.  Data rows exist only if data are present.

| Table 25: CAPS EVN UNCALIBRATED Data File Contents and Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Column Name | Type | Length (bytes) | Range | Description |
| B cycle number | Unsigned Integer | 2 | [1,340] | B cycle number from the start of day, a value of 65535 indicates no B-cycle data is available |
| A cycle number | Unsigned Integer | 2 | [1,2732] | A cycle number from the start of day |
| Time | Float | 8 | [-7.1x107, 1.5x109] | Start time of B cycle, sec. from J2000 (barycentric dynamic time) |
| Offset time | Unsigned Integer | 2 | [0,32000] | Milliseconds from start of A cycle |
| Energy Step | Unsigned Integer | 2 | [1,63] | Energy Step |
| Azimuth Value | Unsigned Integer | 2 | 1 | Azimuth Value. In this case, the value is always 1 (CPU2 samples the first sweep of every other A cycle. Included here for clarity and useful when used in combination with ION data). |
| Elevation | Unsigned Integer | 1 | [1,8] | Elevation or Sector ID. |
| TOF type | Unsigned Integer | 1 | [0,255] | ST/LEF and single/dual event flag 0 = ST, first or single event 1 = LEF, first or single event 2 = ST, second event of a dual event 3 = LEF, second event of a dual event 4 – 255 = Spare |
| TOF | Unsigned Integer | 2 | [1,2048] | Event’s Time of Flight. The particle’s TOF channel. |

## CAPS Standard CALIBRATED Data Product Descriptions

The following sections describe the content and structure of each of the standard data products within the CALIBRATED level 3 CAPS data set.  The format will be similar to the un-calibrated data product format, BUT the counts will be converted to either flux or phase space densities.  In addition, there will be a variance on each measurement.  Also, instead of having just an energy step or an azimuth number, we will have the actual energy value and the azimuth angle.  The size of the files will quadruple.

### CAPS ELS Data Product Format

The data product format for ELS is TBD.

### CAPS IBS Data Product Format

The data product format for CAPS IBS is TBD.

### CAPS IMS ION Data Product Format

The data product format for CAPS IMS ION is TBD.

### CAPS IMS SNG Data Product Format

The data product format for CAPS IMS Singles (SNG) is TBD.

### CAPS IMS TOF Data Product Format

The data product format for CAPS IMS Time of flight (TOF) is TBD.

### CAPS ANC Data Product Format

The data product format for the ancillary data product is TBD.  This structure will include variables for use with the calibrated files.  In addition, the file will include multiple coordinate systems, which are more relevant to our data than the J2000 coordinate system.

# Support Staff and Cognizant Persons

Table 26: CAPS Archive Collection Support Staff

| CAPS Team |  |  |  |
| --- | --- | --- | --- |
| Judith D Furman | Southwest Research Institute 6220 Culebra Road San Antonio, TX 78228 | 210-522-6040 | jfurman@swri.edu |
| Charles Zinsmeyer | Southwest Research Institute 6220 Culebra Road San Antonio, TX 78228 | 210-522-5018 | czinsmeyer@swri.edu |
| Dr. Frank Crary | Southwest Research Institute 6220 Culebra Road San Antonio, TX 78228 | 210-522-6043 | fcrary@swri.edu |
| UCLA |  |  |  |
| Mr. Steven P. Joy PPI Operations Manager | UCLA-IGPP 405 Hilgard Ave Los Angeles, CA 90095-1567 | 310-825-3506 | sjoy@igpp.ucla.edu |

# Appendix A. Directory Structure for Archive Volume, COCAPS_1nnn

CATALOG

EXTRAS

INDEX

DOCUMENT

CALIB

BROWSE

CAPS_SIS

CAPS_CALIB

TBD

SAMPLE_DATA

DATA

UNCALIBRATED

YYYYDDD1

YYYYDDD2

File list as specified in Table 6

File list as specified in Table 6

Directory Structure for Archive Volume, COCAPS_2mmm

CATALOG

EXTRAS

INDEX

DOCUMENT

CALIBRATED

YYYYDDD2

YYYYDDD1

File list as specified in Table 6

File list as specified in Table 6

DATA

CAPS_SIS

CAPS_CALIB

TBD

CALIB

BROWSE

SAMPLE_DATA

# Appendix B. PDS Labels & Format Files for Standard UNCALIBRATED Data Products

ELS_U1.FMT File

/* ELS_U1.FMT */ /* Description of the electron spectrometer data table */ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 DESCRIPTION = "A cycle number from the start of day" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = COLLAPSE_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 14 BYTES = 1 DESCRIPTION = "Flag indicating how data is collapsed: 0: average 1: sum 2: average with in-flight deadtime correction 3: sum with in-flight deadtime correction NOTE: The upper bit will be set to 1 when housekeeping is missing. END_OBJECT = COLUMN OBJECT = COLUMN NAME = OFFSET_TIME DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 15 BYTES = 2 UNIT = MILLISECOND DESCRIPTION = "Milliseconds from start of A cycle" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 17 BYTES = 2 DESCRIPTION = "Minimum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 19 BYTES = 2 DESCRIPTION = "Maximum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 21 BYTES = 2 DESCRIPTION = "Minimum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 23 BYTES = 2 DESCRIPTION = "Maximum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 25 UNIT = COUNTS ITEMS = 8 ITEM_BYTES = 2 BYTES = 16 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 65504 DESCRIPTION = "Counts in elevations 1 through 8" END_OBJECT = COLUMN

Sample ELS Label File: ELS_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "ELS UNCALIBRATED" PRODUCT_ID = "ELS_200522400_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 40 FILE_RECORDS = 122496 START_TIME = 2005-224T00:00:21 STOP_TIME = 2005-224T05:59:48 SPACECRAFT_CLOCK_START_COUNT = "1/1502497703.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502519271.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS data from the ELS sensor acquired at SATURN between 2005-224T00:00:21.000 and 2005-224T05:59:48.000 (orbit 013)." MD5_CHECKSUM = "dca0087dc4ee3c3e5abc68f372d20320" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0xFE38" ^TABLE = "ELS_200522400_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 122496 COLUMNS = 11 ROW_BYTES = 40 ^STRUCTURE = "ELS_U1.FMT" DESCRIPTION = " The file ELS_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

IBS_U1.FMT File

/* IBS_U1.FMT */ /* describes the structure of the IBS Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 DESCRIPTION = "A cycle number from the start of day" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_MODE_SUBMODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 14 BYTES = 1 DESCRIPTION = "IBS mode and submode flag: 0 = Standard Sweep Collapse 1 = Standard Sweep Snapshot 2 = Solar Wind Search 3 = Solar Wind Track 4 = Magnetosphere Search 5 = Magnetosphere Survey 6 = Calibration Mode 7-255 = spare" END_OBJECT = COLUMN OBJECT = COLUMN NAME = OFFSET_TIME DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 15 BYTES = 4 UNIT = MILLISECOND DESCRIPTION = "Milliseconds from start of the IBS collection cycle. An IBS data product is constructed from 16 to 128 azimuths of data, with each azimuth representing 2 seconds of instrument data collection. END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 19 BYTES = 2 DESCRIPTION = "Minimum energy step in collapsed data. This is an index into the energy table." END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 21 BYTES = 2 DESCRIPTION = "Maximum energy step in collapsed data This is an index into the energy table." END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 23 BYTES = 2 DESCRIPTION = "Minimum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 25 BYTES = 2 DESCRIPTION = "Maximum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 27 UNIT = COUNTS ITEMS = 3 ITEM_BYTES = 2 BYTES = 6 MISSING_CONSTANT = 65535 VALID_MINIMUM = 1 VALID_MAXIMUM = 65504 DESCRIPTION = "Counts in fans 1 through 3" END_OBJECT = COLUMN

Sample IBS Label File: IBS_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "IBS UNCALIBRATED" PRODUCT_ID = "IBS_200522400_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 32 FILE_RECORDS = 538815 START_TIME = 2005-223T23:56:37 STOP_TIME = 2005-224T05:55:00 SPACECRAFT_CLOCK_START_COUNT = "1/1502497479.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502518983.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS data from the IBS sensor acquired at SATURN between 2005-223T23:56:37.000 and 2005-224T05:55:00.000 (orbit 013)." MD5_CHECKSUM = "e7e5905adba35ede16ef2245b34c39d3" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0x0229" ^TABLE = "IBS_200522400_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 538815 COLUMNS = 11 ROW_BYTES = 32 ^STRUCTURE = "IBS_U1.FMT" DESCRIPTION = " The file IBS_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

ION_U1.FMT File

/* ION_U1.FMT */ /* describes the structure of the IMS ION Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SPARE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 14 BYTES = 1 DESCRIPTION = "Will contains zeroes" END_OBJECT = COLUMN OBJECT = COLUMN NAME = OFFSET_TIME DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 15 BYTES = 2 UNIT = MILLISECOND DESCRIPTION = "Milliseconds from start of A cycle" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 17 BYTES = 2 DESCRIPTION = "Minimum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 19 BYTES = 2 DESCRIPTION = "Maximum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 21 BYTES = 2 DESCRIPTION = "Minimum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 23 BYTES = 2 DESCRIPTION = "Maximum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SAM_ION_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 25 BYTES = 2 DESCRIPTION = "SAM ion number (identifies ion and group table)" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA DATA_TYPE = MSB_INTEGER START_BYTE = 27 UNIT = COUNTS ITEMS = 8 ITEM_BYTES = 2 BYTES = 16 MISSING_CONSTANT = 28671 VALID_MINIMUM = -32 VALID_MAXIMUM = 27650 DESCRIPTION = "Counts in elevations 1 through 8 (signed value)" END_OBJECT = COLUMN

Sample IMS ION Label File: ION_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "ION UNCALIBRATED" PRODUCT_ID = "ION_200522400_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 42 FILE_RECORDS = 36288 START_TIME = 2005-224T00:08:53 STOP_TIME = 2005-224T05:41:08 SPACECRAFT_CLOCK_START_COUNT = "1/1502498215.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502518151.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS Ion data from the IMS sensor acquired at SATURN between 2005-224T00:08:53.000 and 2005-224T05:41:08.000 (orbit 013)." MD5_CHECKSUM = "eacab590197d3ac2dcfdaba4f6bac6ad" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0x019A" ^TABLE = "ION_200522400_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 36288 COLUMNS = 12 ROW_BYTES = 42 ^STRUCTURE = "ION_U1.FMT" DESCRIPTION = " The file ION_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

SNG_U1.FMT File

/* SNG_U1.FMT */ /* describes the structure of the IMS Singles (SNG) Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SPARE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 14 BYTES = 1 DESCRIPTION = "Contains zeroes" END_OBJECT = COLUMN OBJECT = COLUMN NAME = OFFSET_TIME DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 15 BYTES = 2 UNIT = MILLISECOND DESCRIPTION = "Milliseconds from start of A cycle" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 17 BYTES = 2 DESCRIPTION = "Minimum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 19 BYTES = 2 DESCRIPTION = "Maximum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 21 BYTES = 2 DESCRIPTION = "Minimum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 23 BYTES = 2 DESCRIPTION = "Maximum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 25 UNIT = COUNTS ITEMS = 8 ITEM_BYTES = 2 BYTES = 16 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "Counts in elevations 1 through 8" END_OBJECT = COLUMN

Sample IMS Singles (SNG) Label File: SNG_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "SNG UNCALIBRATED" PRODUCT_ID = "SNG_200522400_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 40 FILE_RECORDS = 48573 START_TIME = 2005-224T00:00:21 STOP_TIME = 2005-224T05:59:48 SPACECRAFT_CLOCK_START_COUNT = "1/1502497703.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502519271.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS Singles data from the IMS sensor acquired at SATURN between 2005-224T00:00:21.000 and 2005-224T05:59:48.000 (orbit 013)." MD5_CHECKSUM = "006c7e1177dc6dbfa1a167d7e84a4639" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0x2645" ^TABLE = "SNG_200522400_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 48573 COLUMNS = 11 ROW_BYTES = 40 ^STRUCTURE = "SNG_U1.FMT" DESCRIPTION = " The file SNG_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

LOG_U1.FMT File

/* LOG_U1.FMT */ /* describes the structure of the IMS Logicals (LOG) Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TDC_LOG_SELECTION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 14 BYTES = 1 DESCRIPTION = "TDC selectable logical definition, where Value: Logical 13: Logical 14: 0 Start CFD singles Stop CFD Singles 1 Acquisition Errors Deadtimes 2 Single TOF events Double TOF events 3 Data strobes Resets" END_OBJECT = COLUMN OBJECT = COLUMN NAME = OFFSET_TIME DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 15 BYTES = 2 UNIT = MILLISECOND DESCRIPTION = "Milliseconds from start of A cycle" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 17 BYTES = 2 DESCRIPTION = "Minimum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 19 BYTES = 2 DESCRIPTION = "Maximum energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FIRST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 21 BYTES = 2 DESCRIPTION = "Minimum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LAST_AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 23 BYTES = 2 DESCRIPTION = "Maximum azimuth value in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LEF_STOPS DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 25 UNIT = COUNTS BYTES = 2 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "LEF stop counts" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ST_STOPS DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 27 UNIT = COUNTS BYTES = 2 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "ST stop counts" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIMEOUTS DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 29 UNIT = COUNTS BYTES = 2 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "Timeout events" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TOTAL_EVENTS DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 31 UNIT = COUNTS BYTES = 2 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "Total events (generated by SAM for dead time)" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LOGICAL_13 DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 33 UNIT = COUNTS BYTES = 2 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "TDC selectable logical 13, see variable, TDC_LOG_SELECTION to determine which logical is represented in the data." END_OBJECT = COLUMN OBJECT = COLUMN NAME = LOGICAL_14 DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 35 UNIT = COUNTS BYTES = 2 MISSING_CONSTANT = 65535 VALID_MINIMUM = 0 VALID_MAXIMUM = 27500 DESCRIPTION = "TDC selectable logical 14, see variable, TDC_LOG_SELECTION to determine which logical is represented in the data." END_OBJECT = COLUMN

Sample IMS Logicals (LOG) Label File: LOG_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "LOG UNCALIBRATED" PRODUCT_ID = "LOG_200522400_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 36 FILE_RECORDS = 84861 START_TIME = 2005-224T00:00:21 STOP_TIME = 2005-224T05:59:48 SPACECRAFT_CLOCK_START_COUNT = "1/1502497703.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502519271.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS Logicals data from the IMS sensor acquired at SATURN between 2005-224T00:00:21.000 and 2005-224T05:59:48.000 (orbit 013)." MD5_CHECKSUM = "6dd05c1fb3105d5394c325c92fce99e3" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0x0CFB" ^TABLE = "LOG_200522400_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 84861 COLUMNS = 16 ROW_BYTES = 36 ^STRUCTURE = "LOG_U1.FMT" DESCRIPTION = " The file LOG_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

TOF_U1.FMT File

/* TOF_U1.FMT */ /* describes the structure of the IMS TOF Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates that there is a problem with archive generation" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 3 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the B cycle, seconds from J2000 (barycentric dynamic time). A B-cycle is the collection cycle of the Time of Flight data. The duration of the collection cycle is dependant upon the flight software version. A collection is 256 seconds, 512 seconds, or 1024 seconds. During each 32 second instrument cycle, data is transmitted and then recombined on the ground. For more information, please see the CO_CAPS_UNCALIBRATED_DS.CAT in the CATALOG directory." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 11 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = COLLAPSE_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 12 BYTES = 1 DESCRIPTION = "Flag indicating collapse in TOF: 0: average 1: sum" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ST_START_CHANNEL DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 2 VALID_MINIMUM = 0 VALID_MAXIMUM = 2048 DESCRIPTION = "Start ST TOF Channel. NOTE: There are a total of 2048 channels in flight." END_OBJECT = COLUMN OBJECT = COLUMN NAME = ST_INTERVAL DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 15 BYTES = 1 MISSING_CONSTANT = 0 DESCRIPTION = "ST TOF bin interval: 0 = FILL value implying housekeping information is unavailable. Check previous of following Bcycle for this information. 1 = every word taken starting at the ST_START_CHANNEL 2 = every other word is taken starting at the ST_START_CHANNEL 4 = every 4th word is taken starting at the ST_START_CHANNEL" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ST_ENERGY_COLLAPSE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 16 BYTES = 1 DESCRIPTION = "ST energy collapse option: 0 = sum adjacent energies 1 = take even energies 2 = take odd energies 3 = TBA (to be assigned)" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LEF_START_CHANNEL DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 17 BYTES = 2 VALID_MINIMUM = 0 VALID_MAXIMUM = 2048 DESCRIPTION = "Start LEF TOF Channel. NOTE: There are a total of 2048 channels in flight." END_OBJECT = COLUMN OBJECT = COLUMN NAME = LEF_INTERVAL DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 19 BYTES = 1 MISSING_CONSTANT = 0 DESCRIPTION = "LEF TOF bin interval: 0 = FILL value implying housekeping information is unavailable. Check previous of following Bcycle for this information. 1 = every word taken starting at the LEF_START_CHANNEL 2 = every other word is taken starting at the LEF_START_CHANNEL 4 = every 4th word is taken starting at the LEF_START_CHANNEL" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LEF_ENERGY_COLLAPSE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 20 BYTES = 1 DESCRIPTION = "LEF energy collapse option: 0 = sum adjacent energies 1 = take even energies 2 = take odd energies 3 = TBA (to be assigned)" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 21 BYTES = 2 DESCRIPTION = "Energy step in collapsed data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA_ST DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 23 ITEMS = 512 ITEM_BYTES = 4 BYTES = 2048 MISSING_CONSTANT = 4294967295 VALID_MINIMUM = 0 VALID_MAXIMUM = 3268027 UNIT = COUNTS DESCRIPTION = "Counts in ST TOF bins 1 through 512" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA_LEF DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 2071 ITEMS = 512 ITEM_BYTES = 4 BYTES = 2048 MISSING_CONSTANT = 4294967295 VALID_MINIMUM = 0 VALID_MAXIMUM = 3268027 UNIT = COUNTS DESCRIPTION = "Counts in LEF TOF bins 1 through 512" END_OBJECT = COLUMN

Sample IMS TOF Label File: TOF_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "TOF UNCALIBRATED" PRODUCT_ID = "TOF_200500100_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-201T15:01 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 4118 FILE_RECORDS = 384 START_TIME = 2005-001T00:15:29 STOP_TIME = 2005-001T05:42:57 SPACECRAFT_CLOCK_START_COUNT = "1/1483231288.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1483250936.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = {"SATURN"} INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS Time of Flight data from the IMS sensor acquired at SATURN between 2005-001T00:15:29.000 and 2005-001T05:42:57.000 (orbit 00C)." NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0x8E6C" ^TABLE = "TOF_200500100_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 384 COLUMNS = 13 ROW_BYTES = 4118 ^STRUCTURE = "TOF_U1.FMT" DESCRIPTION = " The file TOF_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

ACT_1.FMT File

/* ACT_1.FMT */ /* describes the structure of the Actuator Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 DESCRIPTION = "A cycle number from the start of day" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA DATA_TYPE = IEEE_REAL START_BYTE = 13 UNIT = ANGLE ITEMS = 32 ITEM_BYTES = 4 BYTES = 128 MISSING_CONSTANT = -999 VALID_MINIMUM = -115 VALID_MAXIMUM = 115 DESCRIPTION = "Actuator angle at start + (item #) seconds, where item # is between 0 and 31. TIME" END_OBJECT = COLUMN

Sample Actuator (ACT) Label File: ACT_YYYYDDDHH_1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = {"CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0", "CO-E/J/S/SW-CAPS-3-CALIBRATED-V1.0"} STANDARD_DATA_PRODUCT_ID = {"ACT UNCALIBRATED", "ACT CALIBRATED"} PRODUCT_ID = "ACT_200522400_1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 140 FILE_RECORDS = 675 START_TIME = 2005-224T00:00:21 STOP_TIME = 2005-224T05:59:48 SPACECRAFT_CLOCK_START_COUNT = "1/1502497703.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502519271.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS actuator data acquired at SATURN between 2005-224T00:00:21.000 and 2005-224T05:59:48.000 (orbit 013)." MD5_CHECKSUM = "1d2450f06e28196c9bbc031b5ce66f3d" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0xDCB2" ^TABLE = "ACT_200522400_1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 675 COLUMNS = 4 ROW_BYTES = 140 ^STRUCTURE = "ACT_1.FMT" DESCRIPTION = " The file ACT_1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

EVN_U1.FMT File

/* EVN_U1.FMT */ /* describes the structure of the Event Mode Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I2 START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I2 START_BYTE = 3 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "A cycle number from the start of day, a value of 65535 indicates that no A-cycle header information is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL FORMAT = F8 START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = OFFSET_TIME DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I2 START_BYTE = 13 BYTES = 2 UNIT = MILLISECOND DESCRIPTION = "Milliseconds from start of A cycle" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ENERGY_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I2 START_BYTE = 15 BYTES = 2 DESCRIPTION = "Energy step" END_OBJECT = COLUMN OBJECT = COLUMN NAME = AZIMUTH_VALUE DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I2 START_BYTE = 17 BYTES = 2 DESCRIPTION = "Azimuth value (always 1)" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ELEVATION DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I1 START_BYTE = 19 BYTES = 1 DESCRIPTION = "Elevation" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TOF_TYPE DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I1 START_BYTE = 20 BYTES = 1 DESCRIPTION = "ST/LEF and single/dual event flag 0 = ST, first or single event 1 = LEF, first or single event 2 = ST, second event of a dual event 3 = LEF, second event of a dual event 4 - 255 = spare" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TOF DATA_TYPE = MSB_UNSIGNED_INTEGER FORMAT = I2 START_BYTE = 21 BYTES = 2 DESCRIPTION = "Event's Time of Flight Data. The particle's TOF channel." END_OBJECT = COLUMN

Sample EVN Label File: EVN_YYYYDDDHH_U1.LBL

NOT AVAILABLE YET

ANC_U1.FMT File

/* ANC_U1.FMT */ /* describes the structure of the Ancillary Data Table*/ OBJECT = COLUMN NAME = B_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 1 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "B cycle number from the start of the day, a value of 65535 indicates no B-cycle data is available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = A_CYCLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 3 BYTES = 2 DESCRIPTION = "A cycle number from the start of day" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME DATA_TYPE = IEEE_REAL START_BYTE = 5 BYTES = 8 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, seconds from J2000 (barycentric dynamic time). An A-cycle is the 32 second instrument collection cycle." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TIME_SCLK DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 13 BYTES = 4 UNIT = SECOND DESCRIPTION = "Start time of the A cycle, spacecraft clock" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SATURN_POS_X DATA_TYPE = IEEE_REAL START_BYTE = 17 BYTES = 4 UNIT = KILOMETER DESCRIPTION = "J2000[km]: Saturn-centered Spacecraft X Position" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SATURN_POS_Y DATA_TYPE = IEEE_REAL START_BYTE = 21 BYTES = 4 UNIT = KILOMETER DESCRIPTION = "J2000[km]: Saturn-centered Spacecraft Y Position" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SATURN_POS_Z DATA_TYPE = IEEE_REAL START_BYTE = 25 BYTES = 4 UNIT = KILOMETER DESCRIPTION = "J2000[km]: Saturn-centered Spacecraft Z Position" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SATURN_VELOCITY_VX DATA_TYPE = IEEE_REAL START_BYTE = 29 BYTES = 4 DESCRIPTION = "J2000 [km/s]: Relative to Saturn" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SATURN_VELOCITY_VY DATA_TYPE = IEEE_REAL START_BYTE = 33 BYTES = 4 DESCRIPTION = "J2000 [km/s]: Relative to Saturn" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SATURN_VELOCITY_VZ DATA_TYPE = IEEE_REAL START_BYTE = 37 BYTES = 4 DESCRIPTION = "J2000 [km/s]: Relative to Saturn" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SUN_POS_X DATA_TYPE = IEEE_REAL START_BYTE = 41 BYTES = 4 UNIT = KILOMETER DESCRIPTION = "J2000[km]: Sun-centered Spacecraft X Position." END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SUN_POS_Y DATA_TYPE = IEEE_REAL START_BYTE = 45 BYTES = 4 UNIT = KILOMETER DESCRIPTION = "J2000[km]: Sun-centered Spacecraft Y Position." END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SUN_POS_Z DATA_TYPE = IEEE_REAL START_BYTE = 49 BYTES = 4 UNIT = KILOMETER DESCRIPTION = "J2000[km]: Sun-centered Spacecraft Z Position." END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SUN_VELOCITY_VX DATA_TYPE = IEEE_REAL START_BYTE = 53 BYTES = 4 DESCRIPTION = "J2000 [km/s]: Relative to the Sun" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SUN_VELOCITY_VY DATA_TYPE = IEEE_REAL START_BYTE = 57 BYTES = 4 DESCRIPTION = "J2000 [km/s]: Relative to the Sun" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_SUN_VELOCITY_VZ DATA_TYPE = IEEE_REAL START_BYTE = 61 BYTES = 4 DESCRIPTION = "J2000 [km/s]: Relative to the Sun" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_XX DATA_TYPE = IEEE_REAL START_BYTE = 65 BYTES = 4 DESCRIPTION = "XX component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_XY DATA_TYPE = IEEE_REAL START_BYTE = 69 BYTES = 4 DESCRIPTION = "XY component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_XZ DATA_TYPE = IEEE_REAL START_BYTE = 73 BYTES = 4 DESCRIPTION = "XZ component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_YX DATA_TYPE = IEEE_REAL START_BYTE = 77 BYTES = 4 DESCRIPTION = "YX component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_YY DATA_TYPE = IEEE_REAL START_BYTE = 81 BYTES = 4 DESCRIPTION = "YY component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_YZ DATA_TYPE = IEEE_REAL START_BYTE = 85 BYTES = 4 DESCRIPTION = "YZ component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_ZX DATA_TYPE = IEEE_REAL START_BYTE = 89 BYTES = 4 DESCRIPTION = "ZX component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_ZY DATA_TYPE = IEEE_REAL START_BYTE = 93 BYTES = 4 DESCRIPTION = "ZY component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SC_ORIENT_ZZ DATA_TYPE = IEEE_REAL START_BYTE = 97 BYTES = 4 DESCRIPTION = "ZZ component of rotation matrix to J2000" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ELS_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 101 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 102 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ION_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 103 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TOF_LEF_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 104 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TOF_ST_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 105 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = LOG_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 106 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SNG_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 107 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ACT_QUALITY_FLAG DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 108 BYTES = 1 DESCRIPTION = "Missing data and good/bad checksum 0 = Everything is OK 1 = Missing Data 2 = Bad Checksum 3 = Both Missing Data & Bad Checksum 4,5,6 = Not used 7 = No Data" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ACT_STATUS_BITS DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 109 ITEMS = 32 ITEM_BYTES = 1 BYTES = 32 VALID_MINIMUM = 0 VALID_MAXIMUM = 4 DESCRIPTION = "Actuator Status Bits: 0 = Everything is OK 4 = Hit the Limit Switch at +108 8 = Hit the Limit Switch at -108 16 = Data Not Available" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TLM_VERSION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 141 BYTES = 1 DESCRIPTION = "Telemetry mode version number" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FSW_MAJOR_VERSION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 142 BYTES = 1 DESCRIPTION = "Flight software major version number. To build the full flight software version: Major.SubMajor.Minor.SubMinor For example: 3.1.0.2" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FSW_SUBMAJOR_VERSION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 143 BYTES = 1 DESCRIPTION = "Flight software sub-major version number. To build the full flight software version: Major.SubMajor.Minor.SubMinor For example: 3.1.0.2" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FSW_MINOR_VERSION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 144 BYTES = 1 DESCRIPTION = "Flight software minor version number. To build the full flight software version: Major.SubMajor.Minor.SubMinor For example: 3.1.0.2" END_OBJECT = COLUMN OBJECT = COLUMN NAME = FSW_SUBMINOR_VERSION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 145 BYTES = 1 DESCRIPTION = "Flight software sub-minor version number. To build the full flight software version: Major.SubMajor.Minor.SubMinor For example: 3.1.0.2" END_OBJECT = COLUMN OBJECT = COLUMN NAME = POINTING_TYPE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 146 BYTES = 1 DESCRIPTION = "Describes the type of pointing we have: 0 = no pointing available 1 = pointing based on predicts 2 = pointing based on reconstructs." END_OBJECT = COLUMN OBJECT = COLUMN NAME = TELEMETRY_MODE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 147 BYTES = 1 DESCRIPTION = "Logical telemetry rate and mode: 1 = 250 bps 2 = 500 bps 4 = 1 kbps 8 = 2 kbps 16 = 4 kbps 32 = 8 kbps 64 = 16 kbps 130 = 500 bps solar wind 132 = 1 kbps solar wind 136 = 2 kbps solar wind" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_SWEEP_TABLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 148 BYTES = 1 MISSING_CONSTANT = 255 DESCRIPTION = "IBS sweep table and index table numbers: Upper 4 bits are the IBS index table Lower 4 bits are the IBS sweep table number Fill: 0xFF" END_OBJECT = COLUMN OBJECT = COLUMN NAME = DATA_IBS_BKGD DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 149 UNIT = COUNTS ITEMS = 3 ITEM_BYTES = 2 BYTES = 6 VALID_MINIMUM = 0 VALID_MAXIMUM = 65534 MISSING_CONSTANT = 65535 DESCRIPTION = "IBS background counts in fans 1 through 3. Fill is 0xFFFF" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_STARTING_ENERGY DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 155 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "IBS starting energy step number. Fill is 0xFFFF" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_SUBCYCLE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 157 BYTES = 1 MISSING_CONSTANT = 255 DESCRIPTION = "IBS subcycle counter. Fill is 0xFF" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_COMPRESSION_RATIO DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 158 BYTES = 1 MISSING_CONSTANT = 0 DESCRIPTION = "ratio: (uncompressed length/compressed length). Calculated on ground from info in the IBS header and rounded down to the nearest integer. Fill is 0" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_PEAK_FAN DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 159 BYTES = 1 MISSING_CONSTANT = 4 DESCRIPTION = "Fan containing the IBS peak. Fill is 4" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_PEAK_ACYCLE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 160 BYTES = 1 MISSING_CONSTANT = 9 DESCRIPTION = "A cycle number containing the IBS peak Fill is 9" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_PEAK_SWEEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 161 BYTES = 1 MISSING_CONSTANT = 0 DESCRIPTION = "IBS peak energy sweep. Fill is 0" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_PEAK_STEP DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 162 BYTES = 1 MISSING_CONSTANT = 0 DESCRIPTION = "IBS peak energy step. Fill is 0" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_THRESHOLD_RL DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 163 BYTES = 2 MISSING_CONSTANT = 65535 DESCRIPTION = "IBS Run length compression threshold. Fill is 0xFFFF" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IMS_SWEEP_TABLE_NUMBER DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 165 BYTES = 1 DESCRIPTION = "IMS sweep table number" END_OBJECT = COLUMN OBJECT = COLUMN NAME = TDC_SINGLE_SELECT DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 166 BYTES = 1 DESCRIPTION = "TDC Singles Selection: Value: Single 13 Single 14 0 Start CFD Stop CFD 1 Acquisition Error Deadtimes 2 Single TOF's Double TOF's 3 Data Strobes Resets" END_OBJECT = COLUMN OBJECT = COLUMN NAME = IMS_LOGICALS_SELECTION DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 167 BYTES = 2 DESCRIPTION = "TDC logicals selection: Bits 15-13: IMS Logical 1 Bits 12-10: IMS Logical 2 Bits 9-7: IMS Logical 3 Bits 6-4: IMS Logical 4 Bits 3-0: Unused Logical selection decoder: 0 = Unused 1 = LEF Stop 2 = ST Stop 3 = Timeouts 4 = Total Events (As used in SAM deadtime correction) 5 = Logical 13 6 = Logical 14 7 = Unused NOTE: Logical 13 and 14 are set with 82TDC_ENG_SING. See OBJECT name TDC_SINGLE_SELECT." END_OBJECT = COLUMN OBJECT = COLUMN NAME = SAM_CPU2_STATUS_FLAGS DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 169 BYTES = 1 DESCRIPTION = "Bit 7 = CPU2/SAM mode change 6 = Background data 5 = Ion deadtime compensation 4 = SAM LEF enable 3 = SAM molecule enable 2 = SW/HW binning 1-0 = HW binning LUT index" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SAM_ION_SELECTION_INDEX DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 170 BYTES = 1 DESCRIPTION = "SAM Ion selection index number" END_OBJECT = COLUMN OBJECT = COLUMN NAME = SAM_ION_GROUP_TABLE DATA_TYPE = MSB_UNSIGNED_INTEGER START_BYTE = 171 BYTES = 2 DESCRIPTION = "SAM group table ID number" END_OBJECT = COLUMN OBJECT = COLUMN NAME = ELS_MCP_ADJ DATA_TYPE = IEEE_REAL START_BYTE = 173 BYTES = 4 UNIT = VOLTS VALID_MINIMUM = 0.0 VALID_MAXIMUM = 3700.0 MISSING_CONSTANT = -1.0 DESCRIPTION = "ELS High Voltage Adjust. converted using: V = DAC * 58.73. Where DAC is the digital to analog value transmitted by the instrument in housekeeping." END_OBJECT = COLUMN OBJECT = COLUMN NAME = IBS_CEM_DAC DATA_TYPE = IEEE_REAL START_BYTE = 177 BYTES = 4 UNIT = VOLTS VALID_MINIMUM = -4000.0 VALID_MAXIMUM = 0.0 MISSING_CONSTANT = 1.0 DESCRIPTION = "IBS CEM (channel-electron multiplier) High Voltage. Converted using: V = DAC * (-15.68627451). DAC is the digital to analog value transmitted by the instrument in housekeeping." END_OBJECT = COLUMN OBJECT = COLUMN NAME = HVU1_RET_DAC DATA_TYPE = IEEE_REAL START_BYTE = 181 BYTES = 4 UNIT = KILOVOLTS VALID_MINIMUM = 0.0 VALID_MAXIMUM = 16.0 MISSING_CONSTANT = -1.0 DESCRIPTION = "HVU1 (high voltage unit 1) Retarding High Voltage, converted using: kV = DAC * 0.0627451 Where DAC is the digital to analog value transmitted by the instrument in housekeeping." END_OBJECT = COLUMN OBJECT = COLUMN NAME = HVU1_ACC_DAC DATA_TYPE = IEEE_REAL START_BYTE = 185 BYTES = 4 UNIT = KILOVOLTS VALID_MINIMUM = -16.0 VALID_MAXIMUM = 0.0 MISSING_CONSTANT = 1.0 DESCRIPTION = "HVU1 (high voltage unit 1) Accelerating High Voltage, converted using: kV = DAC * -0.0627451 Where DAC is the digital to analog value transmitted by the instrument in housekeeping." END_OBJECT = COLUMN OBJECT = COLUMN NAME = HVU2_ST_DAC DATA_TYPE = IEEE_REAL START_BYTE = 189 BYTES = 4 UNIT = VOLTS VALID_MINIMUM = -3600.0 VALID_MAXIMUM = 0.0 MISSING_CONSTANT = 1.0 DESCRIPTION = "HVU2 (high voltage unit 2) Straight Through MCP (multichannel plate), converted using: V = DAC * -14.1176 Where DAC is the digital to analog value transmitted by the instrument in housekeeping." END_OBJECT = COLUMN OBJECT = COLUMN NAME = HVU2_LEF_DAC DATA_TYPE = IEEE_REAL START_BYTE = 193 BYTES = 4 UNIT = VOLTS VALID_MINIMUM = -2400.0 VALID_MAXIMUM = 0.0 MISSING_CONSTANT = 1.0 DESCRIPTION = "HVU2 (high voltage unit 2) Linear Electric Field MCP (multichannel plate), converted using: V = DAC * -9.4118 Where DAC is the digital to analog value transmitted by the instrument in housekeeping." END_OBJECT = COLUMN

Sample Ancillary (ANC) Label File: ANC_YYYYDDDHH_U1.LBL

PDS_VERSION_ID = PDS3 DATA_SET_ID = "CO-E/J/S/SW-CAPS-2-UNCALIBRATED-V1.0" STANDARD_DATA_PRODUCT_ID = "ANC UNCALIBRATED" PRODUCT_ID = "ANC_200522400_U1" PRODUCT_TYPE = "DATA" PRODUCT_CREATION_TIME = 2005-228T19:58 RECORD_TYPE = FIXED_LENGTH RECORD_BYTES = 196 FILE_RECORDS = 675 START_TIME = 2005-224T00:00:21 STOP_TIME = 2005-224T05:59:48 SPACECRAFT_CLOCK_START_COUNT = "1/1502497703.000" SPACECRAFT_CLOCK_STOP_COUNT = "1/1502519271.000" INSTRUMENT_HOST_NAME = "CASSINI ORBITER" INSTRUMENT_HOST_ID = "CO" TARGET_NAME = "SATURN" INSTRUMENT_NAME = "CASSINI PLASMA SPECTROMETER" INSTRUMENT_ID = "CAPS" DESCRIPTION = " This file contains Cassini CAPS ancillary data and some spacececraft pointing information acquired at SATURN between 2005-224T00:00:21.000 and 2005-224T05:59:48.000 (orbit 013)." MD5_CHECKSUM = "d991a15bbf9572cb5ab9f9e7dbfc1ff5" NOTE = " The end around carry checksum, with seed 0x55AA, of this file is 0x54A7" SPICE_FILE_NAME = {"SPK: 050505AP_SCPSE_05119_08222.bsp", "00: 05212_05242pk_fsiv.bc", "06: 05212_05242pk_fsiv.bc", "12: 05212_05242pk_fsiv.bc", "18: 05212_05242pk_fsiv.bc"} ^TABLE = "ANC_200522400_U1.DAT" OBJECT = TABLE INTERCHANGE_FORMAT = "BINARY" ROWS = 675 COLUMNS = 65 ROW_BYTES = 196 ^STRUCTURE = "ANC_U1.FMT" DESCRIPTION = " The file ANC_U1.FMT describes the column structure and content of the data file." END_OBJECT = TABLE END

The SAM Ion number shall uniquely identify the ion and the group table used by SAM. This shall be based on a table generated and kept on the ground, and will not be the ion number used inside SAM software (which represents different species in different group tables) nor the ion number in the current CDF files (which represents the order in which ions are selected and passed on by CPU2, and which depends on the group table and ion selection index.)