***** File MODE_FIT.DOC

Mission: VEGA 1

Instrument: PUMA

Submission: IKI FITS Files

Original Format

The data for these files were received as 4027 separate FITS files, each

containing a single PUMA spectrum in a binary form.  Each file began with

the same sequence of parameter values.  The spectrum followed these in a

format which varied slightly with spectrum mode.  In the amplitude sample

bytes of the spectra, the sign bit was as a flag bit to indicate amplitude

triggered samples.  In this form a value of 1 (sign bit set) indicated a

non-triggered value.

Reformatting at SBN

A program was written to take each FITS file, re-order the parameter

values slightly, and modify the treatment of flagged samples in the

spectra.  The resulting binary files were sorted and combined into

tables for each mode.

Specifically:

o  The sign bit in all spectrum samples was turned off.

o  The mode 1, 2 and 3 spectra, the only ones which included sample

flagging, were expanded so that a separate flag field could be

associated with each amplitude sample.

o  Where the sample had previously been flagged as amplitude-triggered

(original sign bit not equal to 1), the accompanying flag byte was

set equal to 1.

o  The spectrum length parameter was adjusted to give the total number

of bytes in the spectrum itself, without the parameter values. (The

length of the parameter values was included in the original field.)

o  All spectra were then padded with nulls (zeroes) out to the maximum

spectrum length of the particular mode.

o  The converted spectra were then collected into a single fixed-length

table for each mode.  The original ordering of the spectra (by FITS

file number) was maintained within each table, as this was assumed to

be chronological.

Known Anomalies/Problems:

o  Little or no explanation was available for some of the parameters in

the spectrum headers.  This is indicated in the DESCRIPTION part of

the PDS label for these datasets.

o  There are fewer spectra in this submission than in the binary table

also submitted by IKI or the text files submitted by MPI.  These data

appear to be a strict subset of the IKI binary table, with most but

not all of the "extra" spectra in the second IKI submission having

later time values than the spectra in this dataset.  However, while

there is a large amount of overlap between the IKI and MPI

submission, each contains spectra not included in the other.

o  It has not yet been possible to translate the times in this file,

which are apparently in units of seconds, to UT.