- 2002 Cassini/INMS

- Ion and Neutral Mass Spectrometer

- INMS STANDARD DATA PRODUCTS

- AND ARCHIVE VOLUME

- SOFTWARE INTERFACE SPECIFICATION

- (INMS Archive Volumes SIS)

- SIS ID: IO-AR-016

- David Gell and Greg Fletcher

Space Physics Research Laboratory

University of Michigan

Ann Arbor, Mi 48109

and

S. Joy

University of California, Los Angeles

Los Angeles, CA 90095-1567

- Version 1.1

September 16, 2005

- 2002 Cassini/INMS

- Ion and Neutral Mass Spectrometer

- INMS STANDARD DATA PRODUCTS

ARCHIVE VOLUMES

SOFTWARE INTERFACE SPECIFICATION

(INMS Archive Volumes SIS)

- Version 1.1

September 16, 2005

Approved:

J. H. Waite 	Date

Team Leader

Diane Conner	Date

Cassini Archive Data Engineer

Ray Walker	Date

PDS Discipline Node Manager

- Table of Contents

1.	Preface	1

1.1	Distribution List	1

1.2	Document Change Log	2

1.3	TBD Items	3

1.4	Acronyms and Abbreviations	3

1.5	Glossary	4

1.6	Content Overview	4

1.7	Scope	9

1.8	Applicable Documents	9

1.9	Audience	9

2.	Archive Volume Generation	9

2.1	Archive Structure and Identification	9

2.2	Data Production and Transfer Methods	11

2.3	Archive Volume Creation	11

2.4	Volume Validation Methods	12

3.	Archive Volume Contents	12

3.1	Root Directory Contents	12

3.2	INDEX Directory Contents	13

3.3	DOCUMENT Directory Contents	13

3.4	CATALOG Directory Contents	14

3.5	CALIB Directory Contents and Naming Conventions	14

3.5.1	Required Files	15

3.5.2	CALIB Directory Contents	15

3.6	DATA (Standard Products) Directory Contents and Naming Conventions	15

3.6.1	Required Files	16

3.6.2	DATA Directory Contents	16

3.7	BROWSE Directory Contents and Naming Conventions	16

3.7.1	Required Files	17

3.7.2	BROWSE Directory Contents	17

3.8	EXTRAS Directory Contents	17

3.8.1	SOFTWARE Directory Contents	18

3.8.2	CALREPORT Directory Contents	18

4.	Archive Volume Format	18

4.1	Disk Format	18

4.2	File Formats	18

4.2.1	Document File Formats	18

4.2.2	Catalog File Formats	18

4.2.3	PDS Label File Formats	19

4.2.4	Data File Formats – Comma Separated Value Files	19

4.3	General Data Product Format	19

4.3.1	INMS SCI Data Product Formats	19

4.3.2	INMS L1A Data Product Formats	23

4.3.3	INMS HKG Data Product Formats	23

4.3.4	INMS OTS Data Product Formats	27

4.3.5	INMS HMD Data Product Formats	30

Appendix A: Directory Structure for volumes of the CO-S-INMS-3-L1A-U-V1.0 Data Set	33

Appendix B: Directory Structure for volumes of the CO-S-INMS-2-PKT-U-V1.0 Data Set	35

Appendix C: CALIBRATION REPORT Directory Structure	36

Appendix D: L1A Structure File Contents	42

Appendix E: HKG Structure File Contents	54

# Preface

This document describes the contents and types of volumes belonging to all of the three INMS data sets.

## Distribution List

| Table 1: Distribution List |  |
| --- | --- |
| Name | Email |
| Hunter Waite | hunterw@umich.edu |
| Greg Fletcher | gfletch@umich.edu |
| David Gell | david.a.gell@umich.edu |
| Ryan Miller | rpmiller@umich.edu |
| Rob Thorpe | rthorpe@swri.edu |
| Erin Walter | emwalter@umich.edu |
| Julia Xu | juliaxu@umich.edu |
| Vince Anicich | vincent.g.anicich@jpl.nasa.gov |
| Tom Cravens | cravens@kuphsx.phsx.ukans.edu |
| Wayne Kasprzak | wayne.kasprzak@gsfc.nasa.gov |
| Janet Luhman | jgluhman@sunspot.ssl.berkeley.edu |
| Ralph McNutt | ralph_mcnutt@jhuapl.edu |
| Wing Ip | wingip@jupiter.ss.ncu.edu.tw |
| Diane Conner | Diane.Conner@jpl.nasa.gov |
| Steve Joy | sjoy@igpp.ucla.edu |
| Robert Mitchell | Robert.Mitchell@jpl.nasa.gov |
| Dennis Matson | Dennis.L.Matson@jpl.nasa.gov |
| Linda Spilker | Linda.J.Spilker@jpl.nasa.gov |

## Document Change Log

| Table 2: Document Change History |  |  |
| --- | --- | --- |
| Change | Date | Affected Portions |
| - Initial Draft | 01/20/2003 | All |
| - updates based on review by J. Mafi - Added L1A, and TTN data products - Replaced target identifier E/J/S by S in data set names. No Earth or Jupiter Data - Changed directory content tables to reflect the fact that all data is stored in a directories by day of year | 12/01/2004 |  |
| - Changed document number to reflect draft status, first approved version will be 1.0 - updates based on review by D. Conner - omit event table product. - detailed database schema deleted as unneeded - Improve description of electronic data transfer - Extensive changes based on discussions with discipline node, - Deleted Geometry directory - unneeded | 02/15/05 |  |
| - Extensive changes based on discussions with discipline node, - Update signature page, reflecting new PDS Project manager - Update reference page to reflect current PDS documentation versions - Expanded descriptions of volume structure - Added map of PKT archive volume to appendices - Simplification of production volume validation | 3/1/05 |  |
| - Add mission and instrument prefixes to CAT file names - Correct typo’s in Data Set Ids. - Add instrument prefix to standard data product Ids - Add PPI Spot check of production file structure - Add EXTRA directory to L1A archive files - Modify volume Ids to permit sequential runs of more than 1000 volumes - Add calibration summary directory to reference volume - Indicate that INMS_L1A volumes will contain calibration summaries valid at the time of volume creation. | 3/16/05 |  |
| - Move contents of reference volume to EXTRAS Directory on volume COINMS_3000 - Minor editorial changes - Estimate of L1A data volume - Added Appendix D, the L1A Structure | 3/31/2005 |  |
| - Reflects sample volume delivery - changed specialty of outside peer reviewers | 4/15/2005 |  |
| - Peer review completed, changed target in data set names to S from SSA | 7/1/2005 |  |
| - Added Appendix E with housekeeping file format | 9/16/2005 | Appendix E, section 4.3.3 |

## TBD Items

Items that are currently TBD or not finalized, but will be defined in the next few months:

| Table 3: TBD Items |  |  |
| --- | --- | --- |
| Item | Section | Pages |
| Monthly Data Volumes | 1.6 | 9 |

## Acronyms and Abbreviations

| Table 4: Acronyms and Abbreviations |  |
| --- | --- |
| Acronym | Definition |
| ASCII | American Standard Code for Information Interchange |
| CD-R | Compact Disc - Recordable Media |
| CD-ROM | Compact Disc - Read-Only Memory |
| CSN | INMS ion source, Closed Source Neutral |
| DVD | Digital Versatile Disc |
| GB | Gigabyte(s) |
| GSFC | Goddard Space Flight Center |
| INMS | Ion and Neutral Mass Spectrometer |
| ISO | International Standards Organization |
| JHU/APL | Johns Hopkins University / Applied Physics Laboratory |
| JPL | Jet Propulsion Laboratory |
| MB | Megabyte(s) |
| NSSDC | National Space Science Data Center |
| OSI | INMS ion source, Open Source Ion |
| OSNB | INMS ion source, Open Source Neutral Beam |
| OSNT | INMS ion source, Open Source Neutral Thermal |
| PDB | Project Database |
| PDS | Planetary Data System |
| PPI | Planetary Data System, Planetary Plasma Interactions Node |
| SDVT | Science Data Validation Team |
| SIS | Software Interface Specification |
| TBD | To Be Determined |
| UCLA | University of California, Los Angeles |

## Glossary

Archive – An archive consists of one or more Data Sets along with all the documentation and ancillary information needed to understand and use the data. An archive is a logical construct independent of the medium on which it is stored.

Archive Volume - An Archive Volume is a single physical media (CDROM, DVD, 9-track tape, etc.) used to permanently store files within the PDS archive. Archive Volumes may only be created on media approved by the PDS as meeting archive quality standards.

Archive Volume Set – A collection of one or more Archive Volumes used to store a single Data Set or collection of related Data Sets.

Catalog Information – High-level descriptive information about a Data Set (e.g., mission description, spacecraft description, instrument description), expressed in Object Description Language (ODL), which is suitable for loading into a PDS catalog.

Data Product – A labeled grouping of data resulting from a scientific observation, usually stored in one file. A product label identifies, describes, and defines the structure of the data. An example of a Data Product is a planetary image, a spectral table, or a time series table.

Data Set – A Data Set is a collection of Data Products from a single instrument that have a common data processing level, together with supporting documentation and ancillary files.

Standard Data Product – A Data Product generated in a predefined way using well-understood procedures, processed in "pipeline" fashion. Data Products that are generated in a nonstandard way are sometimes called special Data Products.

## Content Overview

The Ion and Neutral Mass Spectrometer (INMS) aboard the Cassini spacecraft consists of a closed ion source and an open ion source; various focusing lenses; an electrostatic quadrupole switching lens; a radio frequency quadrupole mass analyzer; two secondary electron multiplier detectors; and the associated supporting electronics and power supply systems. The INMS will be operated in three different modes: a closed source neutral mode (CSN), for the measurement of non-reactive neutrals such as N2 and CH4; an open source neutral (OSN) mode, for reactive neutrals such as atomic nitrogen; and an open source ion (OSI) mode, for positive ions with energies less than 100 eV. The primary focus of the INMS investigation is on the composition and structure of Titan’s upper atmosphere and its interaction with Saturn’s magnetospheric plasma. Note that the INMS ‘Closed Source’ mode only measures neutral particles so ‘neutral’ is redundant when referring to this mode. However CSN is used throughout this document so that file naming conventions are the same number of characters for all modes.

INMS is a complex instrument, and will be taking data continuously throughout the tour phase of the mission (the exception being the release of the Huygens Probe and the associated quiet period, during which most instruments on Cassini will be turned off). This complexity, coupled with tracking the minute changes in the characteristics of the instrument over time, mean calibration data and calibration techniques will be dynamic. For this reason it will be impractical to redeliver data volumes as calibration improves. The solution is to deliver a data set that has scaling factors applied (i.e. converted to engineering units), and provide the calibration data as a separate ancillary volume. Thus, as calibrations improve, only the reference volume will need to be redistributed.

Each of the three modes of the INMS will produce packets containing total counts per sample. The number of integration periods that are co-added to form a packet depends on the rate at which the instrument is taking data.

INMS data will be divided into three data sets, one containing telemetry packet contents converted from data numbers to engineering units, one containing detector data annotated with instrument state and geometric data, and a third containing higher level data produced from the lower level data. The data sets may contain one or more product types, which correspond to collections of related data in files of specific organization. The two high level data sets contain one product type apiece, whilst the low level data set contains one product type for each telemetry packet. The data set identification, CODMAC levels, and product types are enumerated in Table 5.

| Table 5: Relationship Between Data Sets and Standard Data Products |  |  |  |
| --- | --- | --- | --- |
| Data Set ID | CODMAC Level | Standard Data Product ID | Description |
| CO-S-INMS-5-TTN-C-V1.0 | 5 | INMS_TTN | Profiles of species densities |
| CO-S-INMS-3-L1A-U-V1.0 | 3 | INMS_L1A | Annotated instrument output |
| CO-S-INMS-2-PKT-U-V1.0 | 2 | INMS_PKT_SCI | Telemetry Packets with contents converted from data numbers to dimensional quantities |
|  |  | INMS_PKT_HKG |  |
|  |  | INMS_PKT_HMD |  |
|  |  | INMS_PKT_SMD |  |
|  |  | INMS_PKT_OTS | Operational table description |

The data set CO-S-INMS-2-PKT-U-V1.0 contains the contents of the INMS telemetry packets, with each variety of packet forming a product type. INMS produces 4 packet types, Science (SCI), Housekeeping (HKG), Science Memory Dumps (SMD), and Housekeeping memory Dumps (HMD). An additional data type, OTS, is included in this data set which contains information describing the operational table set controlling the data collection.

The data set CO-S-INMS-3-L1A-U-V1.0 contains annotated detector signals as a single data type. The data is aggregated from the science and housekeeping telemetry, spacecraft ephemeris and attitude data, Saturnian system ephemeris, and operations table contents. This data is provided as the fundamental data from which to produce higher level products. Besides the data files, this data set includes graphical browse products and a calibration summary valid at the time of volume creation.

It is anticipated that the  data set CO-S-INMS-5-TTN-C-V1.0 containing altitude profiles of atmospheric species abundances, will be produced from data analysis preformed by the INMS science team and archived on a best-effort basis. The contents of this data set is related to Titan’s atmosphere and will contain profiles of both neutral and ion species, included in Table 6 and Table 7, along with derived temperature profiles. Additional data product types may be added to this data set as the analysis results merit. Possibilities include  magnetospheric ion and neutral flux as a function of look direction, space, and energy for each species. Magnetospheric constituents which INMS is capable of detecting, consist largely of water products: For the inner magnetosphere, ions such as H3O+, H2O+, O+, H+, OH+, and neutrals such as H2O, H, OH, O (though OH and O are both highly reactive neutrals, and thus may not be present at detectable levels). For the outer magnetosphere, ions such as H+, N+, N2+, H2CN+ and/or C2H5+, and neutrals such as N, and N2. At the icy satellites, INMS expects to detect species similar to those in the inner magnetosphere, along with additional trace species.

| Table 6: Most Abundant Ion Species in Titan’s Ionosphere |  |  |
| --- | --- | --- |
| Mass Group | Mass Number | Species |
| Light | 1 | H+ |
|  | 2 | H2+ |
|  | 3 | H3+ |
| Medium | 14 | N+, CH2+ |
|  | 15 | CH3+, NH+ |
|  | 16 | CH4+ |
|  | 17 | CH5+ |
| Heavy | 27 | C2H3+ |
|  | 28 | N2+, C2H4+, HCNH+ |
|  | 29 | N2H+, C2H5+ |
| Very Heavy | 39 | C3H3+ |
|  | 41 | C3H5+, H5+ |
|  | 51 | C4H4+ |
|  | 52 | C3H2N+ |
|  | 53 | C4H5+, C5H5+C5H5+ |
|  | 65 | C5H5+ |
|  | 67 | C5H7+ |
|  | 69 | C5H9+ |
|  | 77 | C6H5+ |
|  | 79 | C6H7+, C5H5N+ |
|  | 91 | C7H7+ |

| Table 7: Expected Neutral Species in Titan’s Upper Atmosphere |  |  |
| --- | --- | --- |
| Mass Number | Closed Source | Open Source Neutral |
| 2 | H2 |  |
| 3 | HD |  |
| 4 | He |  |
| 14 |  | N |
| 15 |  | NH |
| 16 | CH4 | CH4, O |
| 17 | 13CH4 | OH |
| 18 | H2O | H2O |
| 26 | C2H2 |  |
| 27 | HCN | HCN |
| 28 | N2, C2H4, CO | N2, C2H4, CO |
| 29 | 15N14N, 13C2H4 |  |
| 30 | C2H6 |  |
| 36 | (36Ar) |  |
| 39 |  | CHCN |
| 44 | CO2, C3H8 |  |
| 50 |  | C3N |
| 51 | CH3CN, HC3N |  |
| 52 | C2N2 |  |
| 74 |  | C6H2 |
| 76 | C4N2 |  |
| 78 | C6H6 |  |

contains a list of targeted flybys for which INMS is currently prime. When INMS is the prime instrument, the spacecraft pointing design was specified by the INMS team to support INMS data collection goals. The inner magnetosphere passes are still being determined in the science planning process, thus this list stops at Rev 46 (where the planning process is at the time this document was written). However, since these types of encounters are typically on the order of 90 minutes to two hours, this will be a relatively small subset of the INMS data.

| Table 8: INMS Targeted Flybys |  |  |  |
| --- | --- | --- | --- |
| Target | FLYBY | REV | Alt (Km) |
| Titan | A | A | 1200 |
| Titan | T5 | 6 | 951 |
| Titan | T17 | 28 | 950 |
| Titan | T18 | 29 | 950 |
| Titan | T21 | 35 | 950 |
| Titan | T26 |  |  |
| Titan | T27 | 41 | 952 |
| Titan | T32 | 46 | 950 |
| Titan | T36 | 50 | 950 |
| Titan | T37 | 52 | 950 |
| Titan | T39 | 54 | 952 |
| Titan | T40 | 55 | 950 |
| Enceladus | E61 | 61 | TBD |
| Inner Mag |  | 25 |  |
| Inner Mag |  | 26 |  |
| Inner Mag |  | 29 |  |
| Inner Mag |  | 31 |  |
| Inner Mag |  | 34 |  |
| Inner Mag |  | 43 |  |
| Inner Mag |  | 46 |  |

In addition to the data sets outlined above, INMS will be producing and archiving a reference volume. This volume will contain the documentation that describes the INMS instrument and investigation. It will also contain the detailed calibration report. This report, contained in the CALIBRATION directory is the structured collection of document files, data files, command sequences and programs used to perform the characterizations of the flight and engineering model. Additional engineering model characterization activities are possible. In that event, the calibration report will be extended and the reference volume re-issued.

Table 9 contains estimates of the INMS monthly deliverable data volume, as well as a maximum possible. The L1A volume is computed based on INMS operating at full telemetry rate approximately 3 days per month and at reduced rate for the remainder of the period. Browse product files are negligible in volume compared with the data files.

| Table 9: Deliverable Products and Their Approximate Size |  |  |  |
| --- | --- | --- | --- |
| Data Set Identifier / Volume | Standard Data Product Type | Delivery Frequency | Estimated Size |
| CO-S-INMS-2-PKT-U-V1.0 | INMS_PKT_SCI INMS_PKT_HKG INMS_PKT_HMD INMS_PKT_SMD INMS_PKT_OTS | monthly | tbd tbd tbd tbd tbd |
| CO-S-INMS-3-L1A-U-V1.0 | INMS_L1A | monthly | 3.2GB |

## Scope

This specification applies to all archive volumes containing INMS data products for the duration of its mission.

## Applicable Documents

Planetary Science Data Dictionary Document, August 28,2002, Planetary Data System, JPL D-7116, Rev. E.

Planetary Data System Archive Preparation Guide, January 20, 2005, JPL D-31224, Version 0.050120.

Planetary Data System Standards Reference, August 1, 2003, Version 3.6. JPL D-7669, Part 2.

The Cassini Ion and Neutral Mass Spectrometer, To appear in Space Science Review 2003, J. H. Waite et al.

Cassini/Huygens Program Archive Plan for Science Data, PD 699-068, JPL D-15976

## Audience

This specification is useful to those who wish to understand the format and content of the INMS PDS data product archive collection. Typically, these individuals would be software engineers, data analysts, or planetary scientists.

# Archive Volume Generation

## Archive Structure and Identification

PDS data set names shall conform to the following format: CASSINI <target> INMS <data type> <calibration state> DATA V<major version>.<minor version>. For example, version one of the L1A science data set will be named CASSINI S INMS L1A UNCALIBRATED DATA V1.0.

PDS data set identifiers (dsid) will be abbreviated versions of the data set names formed according to the PDS formation rule for the DATA_SET_ID keyword. For example, the dsid for the data set above would be CO-S-INMS-3-L1A-U-V1.0.

Each data set making up the INMS archive is contained in a distinct set of physical volumes. At the PPI node, each data set is maintained on-line.  The off-line archive for each data set consists of a series of volumes, each bearing a unique volume identifier. The volume identifiers consist of the mission and instrument prefix and a volume identification number as follows:

COINMS_Dnnn

where COINMS is the instrument prefix, D indicates the data set to which the volume belongs and nnn is a sequential number assigned to each volume. The values for the data set indicator are specified in the Table 10.

| Table 10: Volume Identifiers |  |  |
| --- | --- | --- |
| Data Set ID | Data Set Indicator D | Example |
| CO-S-INMS-2-PKT-U-V1.0 | 0 | COINMS_0001 |
| CO-S-INMS-3-L1A-U-V1.0 | 3 | COINMS_3001 |
| CO-S-INMS-5-TTN-C-V1.0 | 6 | COINMS_6001 |

Data file names are formed from the date and type of the data according to the convention in Table 11. The names of INMS_SCI, INMS_L1A, and INMS_TTN type files, which contain one hour of data contain the hour in the file names. The names of the remaining data types, produced once per day, have “00” in the hour field of the name. All file names include a version number, which is a sequentially assigned number, beginning at 01. The version number is incremented when the contents or format of the file is changed.   The relationship between the various identifiers is illustrated in Table 12.

| Table 11: Data File Naming Convention Filename format: yyyydddhh_ttt_ vv.CSV |  |  |
| --- | --- | --- |
| Identifier | Description | Options |
| yyyydddhh | Start time of data file | yyy - Year ddd - Day of Year hh - Hour |
| ttt | Data type | L1A - Science data annotated with ancillary data TTN - Titan atmosphere profiles SCI - Science Data Packet contents, scaled HKG - Housekeeping Data Packet contents, scaled HMD - Housekeeping Memory Dump Packet Contents, scaled SMD - Science Memory Dump Packet Contents, scaled OTS - Operational Table Descriptions |
| vv | Version number of data | 01, 02, ..., etc. |

Each archive volume has the same general structure, consisting of a set of fixed  top level directories, INDEX, DOCUMENT, CATALOG, CALIB, DATA, BROWSE and optionally EXTRAS, the contents of which will be described in detail below. The BROWSE  directory is only part of the CO-S-INMS-3-L1A-U-V1.0 data set. The EXTRAS directory contains files that are helpful but not required for interpretation of the archived data. Only the INDEX, DOCUMENT, CATALOG, EXTRAS and CALIB directories will exist only on the first physical volume of a data set to avoid the requirement to redistribute all physical volumes if the contents of either the EXTRA or CALIB directories change.

The relationship between the data set, standard data product identifiers and file names are illustrated in Table 12. The data structures for the archive volumes for data set are shown in the Appendixes.

| Table 12: Relationship Between Data Sets, Standard Data Product Types, and File Names |  |  |  |
| --- | --- | --- | --- |
| Data Set ID | CODMAC Level | Standard Data Product ID | Example File Name |
| CO-S-INMS-5-TTN-C-V1.0 | 5 | INMS_TTN | 200430015_TTN_01.CSV |
| CO-S-INMS-3-L1A-U-V1.0 | 3 | INMS_L1A | 200430015_L1A_01.CSV |
| CO-S-INMS-2-PKT-U-V1.0 | 2 | INMS_PKT_SCI | 200430000_SCI_01.CSV |
|  |  | INMS_PKT_HKG | 200430000_HKG_01.CSV |
|  |  | INMS_PKT_OTS | 200430000_OTS_01.CSV |
|  |  | INMS_PKT_SMD | 200430000_SMD_01.CSV |
|  |  | INMS_PKT_HMD | 200430000_HMD_01.CSV |

## Data Production and Transfer Methods

The INMS standard product archive collection is produced by the INMS instrument team in cooperation with the PDS Planetary Plasma Interactions (PPI) Node at the University of California, Los Angeles (UCLA). The INMS team is funded by NASA through the Cassini Project office and the PPI activities are funded by the NASA Planetary Data System.

The INMS team produces the individual data files and the associated PDS labels for each of the standard data products defined in section 1.6 above. Data files will all be comma-separated value, ASCII files containing all data of the appropriate type for the time interval contained in the data product. Data products will be compressed (Gzipped) and transferred via secure FTP to the PPI node. Each data transfer is logged in the Cassini Archive Tracking System, CATS. Upon notification of the data transfer provided by the CATS, the PPI node decompresses the transfer and compares its contents against the CATS transfer information. Each data file is validated against the MD5 checksum contained the corresponding detached label. The PPI node will post a positive or negative acknowledgement of the data receipt on CATS. If the acknowledgement is positive, no further action is required on the part of INMS. If the acknowledgement is negative, the transfer is repeated.

## Archive Volume Creation

PPI collects the data files and labels provided by the INMS team onto archive volumes. Each archive volume contains all INMS data available for the time interval covered by the archive volume. Once all of the data files, labels, and ancillary data files are organized onto an archive volume, PPI adds all of the PDS required files (AAREADME, INDEX, ERRATA, etc.) and produces the physical media.

## Volume Validation Methods

Validation of the INMS data archive is completed in two phases. The first phase is performed by the PPI node and consists of reviewing a sample data set for compliance with the PDS standards. The INMS team will submit a set of data files following the procedure of section above. Upon receipt, the PPI node will confirm the structure of the files and labels. Once the sample data is validated, PPI will develop software to generate subsequent data volumes in an automated fashion.

The second phase of the validation consists of a peer review to ensure usability and completeness. The peer review panel will consist of members of the instrument team, the PPI and Central Nodes of the PDS, and at least two outside scientists actively working in the field of mass spectrometry or planetary atmospheres. The PDS personnel will be responsible for validating that the archive volume(s) are fully compliant with PDS standards. The instrument team and outside science reviewers will be responsible for verifying the content of the data set, the completeness of the documentation, and the usability of the data in its archive format. Any deficiencies in the archive volume will be recorded as liens against the product by the review panel. After all liens placed against the product or the product generation software are resolved, automated production and validation can begin.

Once automated production begins, the data file content will be spot checked by members of the INMS team. Browse products corresponding to the L1A product will be produced routinely and examined by members of the INMS team. In addition, the data will be actively used by team members to perform their analysis. Any discrepancies in the data noted during these activities will be investigated. If the discrepancy is a data error, the response will depend on the source of the error. If the error is in the software producing the data product, the error will be corrected and the data products affected will be reproduced. If there is a correctable error in a data file, the file will be replaced. If an error in a data file is uncorrectable, the error will be described in the cumulative errata file included on each volume in the volume set. The structure of data files and labels will be spot checked by the PPI node for compliance with PDS standards and this SIS.

# Archive Volume Contents

This section describes the contents of the INMS standard product archive collection volumes, including the file names, file contents, file types, and organizations responsible for providing the files. The complete directory structures are shown in Appendix A, B, and C. All directories and ancillary files described herein appear on each INMS archive volume, except where noted.

## Root Directory Contents

The following files are contained in the root directory, and are produced by the PPI Node at UCLA. With the exception of the hypertext file and its label, all of these files are required by the PDS Archive Volume organization standards.

| Table 13: Root Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| AAREADME.TXT | This file completely describes the Volume organization and contents (PDS label attached). | PPI |
| AAREADME.HTM | Hypertext version of AAREADME.TXT (top level of HTML interface to the Archive Volume). | PPI |
| AAREADME.LBL | A PDS detached label that describes AAREADME.HTM. | PPI |
| ERRATA.TXT | A cumulative listing of comments and updates concerning all INMS Standard Data Products on all INMS Volumes in the Volume set published to date. | PPI |
| VOLDESC.CAT | A description of the contents of this Volume in a PDS format readable by both humans and computers. | PPI |

## INDEX Directory Contents

The following files are contained in the index directory and are produced by the PDS PPI Node. The INDEX.TAB file contains a listing of all data products on the archive volume. In addition, there is a cumulative index file (CUMINDEX.TAB) that lists all data products in the INMS archive volume set to date. The index and index information (INDXINFO.TXT) files are required by the PDS volume standards. The index tables include both required and optional columns. The cumulative index file is also a PDS requirement; however, this file is not reproduced on each data volume. An online and web accessible cumulative index file is maintained at the PPI Node while archive volumes are being produced. Only the last archive volume in the volume series will contain a cumulative index file.

| Table 14: Index Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| INDXINFO.TXT | A description of the contents of this directory | PPI |
| INDEX.TAB | A table listing all INMS Data Products on this Volume | PPI |
| INDEX.LBL | A PDS detached label that describes INDEX.TAB | PPI |

## DOCUMENT Directory Contents

The document directory contains documentation that is considered to be either necessary or simply useful for users to understand the archive data set. These documents are not necessarily appropriate for inclusion in the PDS catalog. Documents may be included in multiple forms (ASCII, PDF, MS Word, HTML with image file pointers, etc.). PDS standards require that any documentation deemed required for use of the data be available in some ASCII format. Clean HTML  is acceptable as ASCII formats in addition to plain text. The following files are contained in the DOCUMENT directory and are produced or collected by the PPI Node.

| Table 15: Document Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| DOCINFO.TXT | A description of the contents of this directory | PPI |
| VOLSIS.DOC | The Archive Volume SIS (this document) in Microsoft Word format | INMS, PPI |
| VOLSIS.ASC | The Archive Volume SIS (this document) in ASCII format | INMS, PPI |
| VOLSIS.LBL | A PDS detached label that describes VOLSIS.ASC, VOLSIS.HTM and VOLSIS.DOC. | PPI |
| Other Documents | Additional documents describing data processing, calibration etc. | INMS |
| Other Document labels | Detached PDS labels for any additional documents | PPI |

## CATALOG Directory Contents

The completed PDS catalog files in the catalog directory provide a top-level understanding of the Cassini/INMS mission and its data products. The data set catalog files (e.g. CO_INMS_PKT_DS.CAT) will be provided by the INMS team, and the CATINFO.TXT by the PPI Node.

| Table 16: Catalog Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| CATINFO.TXT | A description of the contents of this directory | PPI |
| CO_INMS_PKT_DS.CAT or CO_INMS_L1A_DS.CAT or CO_INMS_TTN_DS.CAT | PDS Data Set catalog description of appropriate to the data set | INMS |
| CO_INSTHOST.CAT | PDS instrument host (spacecraft) catalog description of the Cassini spacecraft | Cassini Project |
| CO_INMS_INST.CAT | PDS instrument catalog description of the INMS instrument | INMS |
| CASSINI_MISSION.CAT | PDS mission catalog description of the Cassini mission | Cassini Project |
| CO_INMS_PERSON.CAT | PDS personnel catalog description of INMS Team members and other persons involved with generation of INMS Data Products | INMS |
| CO_INMS_REF.CAT | INMS-related references mentioned in other *.CAT files Additional bibliographic references, as appropriated | INMS |
| PRJREF.CAT | Cassini-relative references mentioned in other *.CAT files. | Cassini Project |

## CALIB Directory Contents and Naming Conventions

The calibration directory, included in the CO-S-INMS-3-L1A-U-V1.0 data set only, contains one or more spreadsheets of instrument calibration data. An additional file will be added when the instrument characteristics have either changed or been more precisely determined. The files will be named according to their time range of applicability. For example, if it is determined that the calibration after 2007-030 is sufficiently changed, a file incorporating that date in its name will be added. The naming convention is defined in Table 17, below.  To avoid the requirement of redistributing all physical media data volumes in the event of a calibration update, the CALIB directory will be present on only the first physical volume COINMS_3000, and a note to that effect will be placed in the AAREADME file on all physical volumes.

Detailed documentation of the preflight characterization is contained in the EXTRAS directory.

| Table 17: Calibration Summary File Naming Convention Filename format: yyyyddd_CAL_ vv.CSV |  |  |
| --- | --- | --- |
| Identifier | Description | Options |
| yyyyddd | Date of validity | yyy - Year ddd - Day of Year |
| vv | Version number of data | 01, 02, ..., etc. |

### Required Files

The calibration directory contains a file named INFO.TXT that is an ASCII text description of the CALIB directory contents. The calibration summary files will be described by detached PDS labels. The label files will have the same root name as the calibration file that they describe with the suffix “.LBL” replacing the “.CSV” suffix.

### CALIB Directory Contents

The calibration data is organized in a flat directory structure, with the CALIB directory containing all of the calibration summary files and their labels.

| Table 18:Calibration Summary Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| INFO.TXT | Brief description of directory contents and naming conventions. | PPI |
| *CAL*.CSV | Calibration summary file. | INMS |
| *CAL*.LBL | PDS label for CAL file of same base name. | INMS |

## DATA (Standard Products) Directory Contents and Naming Conventions

The data directory contains the actual data products produced by the INMS team.  The CO-S-INMS-2-PKT-U-V1.0 archive volumes will have SCI, HKG, OTS, EVT, SMD, and HMD files in daily subdirectories  The CO-S-INMS-3-L1A-U-V1.0 archive volumes will have L1A files in daily subdirectories. (see 3.6.2. for details description of this directory structure).

### Required Files

Every subdirectory beneath the data directory contains a file named INFO.TXT that is an ASCII text description of the directory contents. Every file in the Data path of an Archive Volume must be described by a PDS label. All labels will be detached, having the same root name as the file they describe with the suffix “.LBL”. In directories where there are multiple data files with the same internal table structure, the table column description is included in a single format file (.FMT) that is referenced by a pointer within each PDS label file. This prevents the needless repetition of information that is not changing within the PDS label files.

### DATA Directory Contents

The data directory contains a separate subdirectory for each day. The daily subdirectories are grouped by year into yearly directories. The structure may be seen in Appendix A. The yearly directories will be named for the year, 2004, 2005, and so forth. The daily subdirectories will be named with the ordinal day-of-year, 001, 002,...366.  There may be more than one SCI, or L1A data file in each subdirectory, depending on what events take place on a given day. For HKG, OTS, SMD and HMD only one file will be produced per day. In addition to the data files there will be a brief text file (INFO.TXT) that describes the directory contents.

| Table 19: Daily Data Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| INFO.TXT | Brief description of directory contents and naming conventions. | PPI |
| yyyydddhh_ttt_vv.CSV | Data file. | INMS |
| yyyydddhh_ttt_vv.LBL | PDS label for data files of same base name. | INMS |
| ttt_STRUCT_VV.FMT | PDS format file containing the data file structure portion of the PDS label for all of the INMS_L1A and INMS_SCI data files. | INMS |
| ttt is replaced with L1A on the CO-S-INMS-3-L1A-U-V1.0 data set volumes SCI,HKG, OTS, SMD, and HMD on the CO-S-INMS-2-PKT-U-V1.0 data set volumes There are no structure files for standard data products INMS_HKG, INMS_OTS, INMS_SMD, or INMS_HMD |  |  |

## BROWSE Directory Contents and Naming Conventions

The browse directory contains images of INMS summary data and is included only on CO-S-INMS-3-L1A-U-V1.0 data set volumes. Two type of images are included, mass-time spectra and time series data. Each image spans six hours beginning at 0h, 6h, 12h, and 18h. (examples to be added). The image files are all portable network graphics files. The files are named in accordance with the convention in Table 20.

| Table 20: Browse Product File Naming Convention Filename format: yyyydddhh_ttt_ vv.PNG |  |  |
| --- | --- | --- |
| Identifier | Description | Options |
| yyyydddhh | Start date file | yyy - Year ddd - Day of Year hh - Hour of Day |
| ttt | Plot type | SPECTRA - mass-time spectra LINE - mass time series |
| vv | Version number of data | 01, 02, ..., etc. |

### Required Files

Every subdirectory beneath the browse directory contains a file named INFO.TXT which contains a description of the directory contents in ASCII text. Every file in the browse path is described by a PDS label. All labels will be detached, having the same root name as the file they describe with the suffix ‘.LBL’ replacing the suffix of the file name.

### BROWSE Directory Contents

The browse directory contains a separated subdirectory for each day. The daily subdirectories are grouped by year into yearly directories. The structure may be seen in Appendix A. The yearly directories will be named for the year, 2004, 2005, and so forth. The daily subdirectories will be named with the ordinal day-of-year, 001, 002,...366. Each daily directory contains eight files, four  mass-time spectra and four mass time series. In addition there will be a brief text file (INFO.TXT) that describes the directory contents.

| Table 21: Daily Browse Product Directory Contents |  |  |
| --- | --- | --- |
| File Name | File Contents | Provided By |
| INFO.TXT | Brief description of directory contents and naming conventions. | PPI |
| yyyydddhh_SPECTRA_vv.PNG | Mass-time spectra browse plots | INMS |
| yyyydddhh_SPECTRA_vv*.LBL | PDS label for SPECTRA plot file of same base name. | PPI |
| yyyydddhh_LINE_vv.PNG | Time series browse plots | INMS |
| yyyydddhh_LINE_vv.LBL | PDS label for LINE plot file of same base name | PPI |

## EXTRAS Directory Contents

The EXTRAS directory contains files that are helpful, but are not required to interpret the INMS data.  Files in the EXTRAS directory are exempt from labeling requirements. An EXTRAS directory is included in the CO-S-INMS-3-L1A-U-V1.0 archive. Subdirectories are used to organize the items into groups of related files. The EXTRAS directory will contain two subdirectories, SOFTWARE and CALREPORT. To avoid the requirement of redistributing all physical media data volumes in the event that the EXTRAS contents changes, this directory will be present on only the first physical volume COINMS_3000, and a note to that effect will be placed in the AAREADME file on all physical volumes.

### SOFTWARE Directory Contents

The SOFTWARE directory contains a library of IDL routines that may be used to read, manipulate and display the contents of the L1A data files. In addition to the IDL source code files, the directory will contain a user’s guide and an HTML help file.

### CALREPORT Directory Contents

The calibration  report directory in the EXTRAS directory contains the complete report of the pre-launch instrument characterization. The report is in the form of a structured series of files containing data from one characterization test. These files are the source for the initial calibration summary file included in the archive volume.  The organization of this directory is shown in Appendix C.

# Archive Volume Format

This section describes the format of the INMS standard product archive volumes. Data that comprise the INMS standard product archives will be formatted in accordance with Planetary Data System specifications.

## Disk Format

Disk formats for the archive volumes will conform to the PDS standard for the applicable media. At present, the plan is to archive INMS data on DVD-R media.

## File Formats

The following section describes file formats for the kinds of files contained on Archive Volumes. For more information, see the PDS Archive Preparation Guide.

### Document File Formats

Document files with the .TXT suffix exist in all directories. They are ASCII files with attached PDS labels. All document files contain variable-length, 80-byte maximum records, with a carriage return character (ASCII 13) in the 79th byte and a line feed character (ASCII 10) in the 80th byte. This allows the files to be read by the MacOS, DOS, Windows, UNIX, OS2, and VMS operating systems.

However, some of the documents in the reference volume contain formatting and figures that cannot be rendered as pure ASCII text. These documents will be provided in formats that support graphics, such as HTML, MS Word, PDF, etc. The PDS requirement that all documentation critical to the understanding of the data set be provided in ASCII text form will be met by the inclusion of CLEAN HTML formatted documents.

### Catalog File Formats

Catalog files (suffix .CAT) exist in the Root and Catalog directories. They are formatted in an object-oriented structure consisting of sets of 'keyword = value' declarations. All files are ASCII and conform to the same structure standards (line length, line terminator) as the PDS labels described in the next section.

### PDS Label File Formats

All data files in the INMS Standard Product Archive Collection have PDS labels. INMS is producing ASCII comma separated value files for all products. Each file will have an detached label, whose name is identical to the data file name with the .CSV suffix replaced by .LBL.

A PDS label, whether attached or detached from its associated file, provides descriptive information about the associated file. The PDS label is an object-oriented structure consisting of sets of 'keyword = value' declarations.

All detached labels contain 80-byte fixed-length records, with a carriage return character (ASCII 13) in the 79th byte and a line feed character (ASCII 10) in the 80th byte. This allows the files to be read by the MacOS, DOS, Windows, UNIX, OS2, and VMS operating systems.

### Data File Formats – Comma Separated Value Files

Delimited field, ASCII data files (.CSV suffix) exist in the DATA directories. These files are formatted for direct reading into many database management systems on various computers. In the INMS application of the delimited field format, fields are separated by commas. Records vary in length in bytes but will have a fixed number of fields. Missing data are represented by empty fields. All fields  are described by detached PDS labels.

## General Data Product Format

All INMS data products are CSV files that contain a series of homogenous records. Records are delimited by the end-of-line characters, the carriage return character (ASCII 13) followed by the line feed character (ASCII 10). Each file contains one or more lines of column headers. The starting location of the data is indicated in the label by the spreadsheet pointer. The column headers are the same as the field name in the tables below, additional header rows contain the units and strings to use as axis labels in plots. Blank fields indicate that the field is not applicable in the context of the record, the data is not available, is out of nominal range, or is otherwise not suitable for archiving. The following sections describe the content and structure of each of the standard data products within the INMS data sets, as described in Table 12 in Section 2.1.

### INMS SCI Data Product Formats

Table 22, below, describes the contents and ranges of the SCI data files. These files contain the contents of the science data packets, converted to dimensional quantities. Each record consists of 9 header items followed by the contents of the telemetry packet.

| Table 22: SCI Data File Contents and Structure |  |  |  |
| --- | --- | --- | --- |
| Field Name | Type | Units | Range |
| OriginalPacketID | integer |  |  |
| EventID | integer |  |  |
| PacketID | integer |  |  |
| AssocSciPacketID | integer |  |  |
| SCLKTime | string |  |  |
| SCLKRaw | real |  |  |
| ERTTime | string |  |  |
| Processedtime | string |  |  |
| Mode | string |  |  |
| Valid | integer |  | 0 ≤ x ≤ 1 |
| MiniPktID | integer |  | 0 ≤ x ≤ 65535 |
| ElapsedSeqTime | integer |  | 0 ≤ x ≤ 65535 |
| ScanIndex | integer |  | 0 ≤ x ≤ 31 |
| SeqTable | integer |  | 0 ≤ x ≤ 31 |
| CycleNum | integer |  | 0 ≤ x ≤ 63 |
| IL1DAC | integer |  | 0 ≤ x ≤ 255 |
| C6DAC | integer |  | 0 ≤ x ≤ 15 |
| ThPresDAC | integer |  | 0 ≤ x ≤ 15 |
| CF1DAC | integer |  | 0 ≤ x ≤ 15 |
| BADAC | integer |  | 0 ≤ x ≤ 15 |
| DT1DAC | integer |  | 0 ≤ x ≤ 15 |
| DT2DAC | integer |  | 0 ≤ x ≤ 15 |
| EM1DAC | integer |  | 0 ≤ x ≤ 255 |
| EM2DAC | integer |  | 0 ≤ x ≤ 255 |
| TmModeIndex | integer |  | 0 ≤ x ≤ 31 |
| MassTable | integer |  | 0 ≤ x ≤ 2047 |
| IP1C1 | real |  | 0 ≤ x ≤ 33711902.01 |
| IP1C2 | real |  | 0 ≤ x ≤ 33711902.01 |
| IP2C1 | real |  | 0 ≤ x ≤ 33711902.01 |
| IP2C2 | real |  | 0 ≤ x ≤ 33711902.01 |
| • ¦ • ¦ • | • • • | • • • | 0 ≤ x ≤ 33711902.01 |
| IP67C1 | real |  | 0 ≤ x ≤ 33711902.01 |
| IP67C2 | real |  | 0 ≤ x ≤ 33711902.01 |
| IP68C1 | real |  | 0 ≤ x ≤ 33711902.01 |
| IP68C2 | real |  | 0 ≤ x ≤ 33711902.01 |
| Spare | integer |  | 0 ≤ x ≤ 65536 |
| Loop | integer |  | 0, 1 |
| Sthi | integer |  | 0, 1 |
| DCONState | integer |  | 0 ≤ x ≤ 63 |
| CycleIndex | integer |  | 0 ≤ x ≤ 255 |
| SequenceRecord | string |  |  |
| CycleRecord | string |  |  |
| FilCmd4On | integer |  | 0, 1 |
| FilCmd3On | integer |  | 0, 1 |
| FilCmd2On | integer |  | 0, 1 |
| Fil4Cmd1On | integer |  | 0, 1 |
| Fil6On | integer |  | 0, 1 |
| Fil4On | integer |  | 0, 1 |
| Fil3On | integer |  | 0, 1 |
| Fil2On | integer |  | 0, 1 |
| Fil1On | integer |  | 0, 1 |
| FilCollision | integer |  | 0, 1 |
| FilEmissAdjust | integer |  | 0, 1 |
| USeqFilError | integer |  | 0, 1 |
| Spare | integer |  | 0 ≤ x ≤ 511 |
| SciRTICounter | integer |  | 0 ≤ x ≤ 65535 |
| SciDTCounter | integer |  | 0 ≤ x ≤ 65535 |
| SciSTMCounter | integer |  | 0 ≤ x ≤ 65535 |
| CPUIdleCounter | integer |  | 0 ≤ x ≤ 15 |
| Disc0Reset | integer |  | 0, 1 |
| AllSysGo | integer |  | 0, 1 |
| AllSysGoLatched | integer |  | 0, 1 |
| ExcNoHandler | integer |  | 0, 1 |
| SafeMode | integer |  | 0, 1 |
| ForcedSleep | integer |  | 0, 1 |
| BIU_Auto_Init | integer |  | 0 ≤ x ≤ 7 |
| GSETest | integer |  | 0, 1 |
| SystemInitialized | integer |  | 0, 1 |
| SystemConfigured | integer |  | 0, 1 |
| INMSBitFail | integer |  | 0, 1 |
| TMBuffOverflow | integer |  | 0, 1 |
| AlfBoot | integer |  | 0, 1 |
| AlfError | integer |  | 0, 1 |
| BIUMemError | integer |  | 0, 1 |
| ODBfrOverflow | integer |  | 0, 1 |
| USeqoOfflinew | integer |  | 0, 1 |
| USeqiOffline | integer |  | 0, 1 |
| USeqError | integer |  | 0, 1 |
| 1750ME | integer |  | 0, 1 |
| AmuxError | integer |  | 0, 1 |
| CDSoOffline | integer |  | 0, 1 |
| CDSiOnline | integer |  | 0, 1 |
| CDSError | integer |  | 0, 1 |
| RTEError | integer |  | 0, 1 |
| bSpare | integer |  | 0, 1 |
| Imon1 | real | A | 0.087772 ≤ x ≤ 1.646842 |
| BACollector | real | V | -0.04 ≤ x ≤ 5.06 |
| Imon2 | real | A | 0.087772 ≤ x ≤ 1.646842 |
| BACurrent | real | A RMS | -0.010828 ≤ x ≤ 1.369742 |
| BAEmission | real | µA | 103.302002 ≤ x ≤ 193.595596 |
| RFAGC | real | V | -0.04 ≤ x ≤ 5.06 |
| MultAna1 | real | µA | -0.08 ≤ x ≤ 10.12 |
| MultAna2 | real | µA | 0.258 ≤ x ≤ 1.518 |
| Imon3 | real | A | 0.087772 ≤ x ≤ 1.646842 |
| ThermPressInt | real | V | -0.04 ≤ x ≤ 5.06 |
| 1Emission | real | µA | 5.8812 ≤ x ≤ 25.6542 |
| 3Emission | real | µA | -0.2022 ≤ x ≤ 25.5783 |
| EM1Current | real | µA | -0.30072 ≤ x ≤ 38.04108 |
| EM2Current | real | µA | 7.518 ≤ x ≤ 38.04108 |
| CmdsRec | integer |  | 0 ≤ x ≤ 65535 |
| CmdsRej | integer |  | 0 ≤ x ≤ 65535 |
| AlfsRec | integer |  | 0 ≤ x ≤ 65535 |
| AlfsRej | integer |  | 0 ≤ x ≤ 65535 |
| CmdSerNo1 | integer |  | 0 ≤ x ≤ 255 |
| Time1 | integer |  | 0 ≤ x ≤ 16777215 |
| CmdSerNo2 | integer |  | 0 ≤ x ≤ 255 |
| Time2 | integer |  | 0 ≤ x ≤ 16777215 |
| CmdSerNo2 | integer |  | 0 ≤ x ≤ 255 |
| Time3 | integer |  | 0 ≤ x ≤ 16777215 |
| CRC | integer |  | n/a |

### INMS L1A Data Product Formats

The L1A files contain the annotated science data. These files contain data abstracted from the science and housekeeping packets along with derived ancillary quantities. The data contained in these files is the basic data from which higher order products are derived.  The structure file, included as Appendix D, further describes these records.

### INMS HKG Data Product Formats

The HKG files contain the complete contents of the housekeeping telemetry packets, converted to dimensional quantities. Each record consists of 9 header items followed by the contents of the telemetry packet. The structure file, included as Appendix E, further describes these records.

### INMS OTS Data Product Formats

The OTS is the Operations Table Set that was in instrument memory when the data was taken. Table 23 below describes the contents of the OTS data files. Each row in the file contains the description of a table set and its period of use. If more than one table set was in use during a day, this file will have multiple records.

| Table 23: OTS Data File Contents and Structure |  |
| --- | --- |
| Field Name | Type |
| TABLESETID | Integer |
| OPSTABLESTYPEID | Integer |
| CREATEID | Integer |
| CREATEDATE | Date/Time |
| UPDATEID | Integer |
| UPDATEDATE | Date/Time |
| INSTRUMENT | String |
| ACTIVE | Integer |
| ACTIVEDATE | Date/Time |
| INACTIVEDATE | Date/Time |
| LOCKED | Integer |
| TITLE | String |
| DESCRIPTION | String |

5.3.5 INMS SMD Data Product Formats

The SMD files contain the complete contents of the science memory dump telemetry packets. The INMS team uses this file to verify the state of the instrument. Table 24 below describes the contents and ranges of the SMD data files. These files contain the contents of the memory dump telemetry packets, converted to dimensional quantities, where appropriate. Each record consists of 9 header items followed by the contents of the telemetry packet.

| Table 24: SMD Data File Contents and Structure |  |  |  |
| --- | --- | --- | --- |
| Field Name | Type | Units | Range |
| OriginalPacketID | integer |  |  |
| EventID | integer |  |  |
| PacketID | integer |  |  |
| SCLKTime | string |  |  |
| SCLKRaw | real |  |  |
| ERTTime | string |  |  |
| Processedtime | string |  |  |
| Mode | string |  |  |
| Valid | integer |  | 0, 1 |
| MiniPacketID | integer |  | 0 ≤ x ≤ 65535 |
| DumpSource | integer |  | 0 ≤ x ≤ 7 |
| NumberofWords | integer |  | 0 ≤ x ≤ 255 |
| DumpAddress | string |  | 0 ≤ x ≤ 65535 |
| IL1DAC | integer |  | 0 ≤ x ≤ 255 |
| C6DAC | integer |  | 0 ≤ x ≤ 15 |
| ThPressDAC | integer |  | 0 ≤ x ≤ 15 |
| CF1DAC | integer |  | 0 ≤ x ≤ 15 |
| BADAC | integer |  | 0 ≤ x ≤ 15 |
| DT1DAC | integer |  | 0 ≤ x ≤ 15 |
| DT2DAC | integer |  | 0 ≤ x ≤ 15 |
| EM1DAC | integer |  | 0 ≤ x ≤ 255 |
| EM2DAC | integer |  | 0 ≤ x ≤ 255 |
| TMModeIndex | integer |  | 0 ≤ x ≤ 31 |
| MassTable | integer |  | 0 ≤ x ≤ 2047 |
| Word0 | integer |  | 0 ≤ x ≤ 0xFFFF |
| • ¦ • ¦ • | • • • |  | 0 ≤ x ≤ 0xFFFF |
| Word170 | integer |  | 0 ≤ x ≤ 0xFFFF |
| WordSpare | string |  |  |
| CPUIdleCounter | integer |  | 0 ≤ x ≤ 15 |
| Disc0Reset | integer |  | 0, 1 |
| AllSysGo | integer |  | 0, 1 |
| AllSysGoLatched | integer |  | 0, 1 |
| ExcNoHandler | integer |  | 0, 1 |
| Safemode | integer |  | 0, 1 |
| ForcedSleep | integer |  | 0, 1 |
| BIU_Auto_Init | integer |  | 0, 1 |
| GSETest | integer |  | 0, 1 |
| SystemInitialized | integer |  | 0, 1 |
| SystemConfigured | integer |  | 0, 1 |
| INMSBitFail | integer |  | 0, 1 |
| TMBuffOverflow | integer |  | 0, 1 |
| AlfBoot | integer |  | 0, 1 |
| AlfError | integer |  | 0, 1 |
| BIUMemError | integer |  | 0, 1 |
| ODBfrOverflow | integer |  | 0, 1 |
| USeqoOffline | integer |  | 0, 1 |
| USeqiOffline | integer |  | 0, 1 |
| USeqError | integer |  | 0, 1 |
| 1750ME | integer |  | 0, 1 |
| AmuxError | integer |  | 0, 1 |
| CDSoOffline | integer |  | 0, 1 |
| CDSiOnline | integer |  | 0, 1 |
| CDSError | integer |  | 0, 1 |
| RTEError | integer |  | 0, 1 |
| Spare | integer |  | 0, 1 |
| Imon1 | real | A | 0.087772 ≤ x ≤ 1.646842 |
| BaCollector | real | V | -0.04 ≤ x ≤ 5.06 |
| Imon2 | real | A | 0.087772 ≤ x ≤ 1.646842 |
| BaCurrent | real | A RMS | -0.010828 ≤ x ≤ 1.369742 |
| BaErrission | real | µA | 103.302002 ≤ x ≤ 193.595596 |
| RFAGC | real | V | -0.04 ≤ x ≤ 5.06 |
| MultAna1 | real | µA | -0.08 ≤ x ≤ 10.12 |
| MultAnal2 | real | µA | 0.258 ≤ x ≤ 1.518 |
| Imon3 | real | A | 0.087772 ≤ x ≤ 1.646842 |
| ThermPressInt | real | V | -0.04 ≤ x ≤ 5.06 |
| 1Emission | real | µA | 5.8812 ≤ x ≤ 25.6542 |
| 3Emission | real | µA | -0.2022 ≤ x ≤ 25.5783 |
| EM1Current | real | µA | -0.30072 ≤ x ≤ 38.04108 |
| EM2Current | real | µA | 7.518 ≤ x ≤ 38.04108 |
| CmdsRec | integer |  | 0 ≤ x ≤ 65535 |
| CmdsRej | integer |  | 0 ≤ x ≤ 65535 |
| AlfsRec | integer |  | 0 ≤ x ≤ 65535 |
| AlfsRej | integer |  | 0 ≤ x ≤ 65535 |
| ConfigNum | integer |  | 0 ≤ x ≤ 255 |
| Time1 | integer |  | 0 ≤ x ≤ 16777215 |
| CmdSerNo2 | integer |  | 0 ≤ x ≤ 255 |
| Time2 | integer |  | 0 ≤ x ≤ 16777215 |
| CmdSerNo3 | integer |  | 0 ≤ x ≤ 255 |
| Time3 | integer |  | 0 ≤ x ≤ 16777215 |
| CRC |  |  | n/a |

### INMS HMD Data Product Formats

The HMD files contain the complete contents of the housekeeping memory dump telemetry packets. As with the SMD the INMS team uses this file to verify the state of the instrument. Table 25 below describes the contents and ranges of the HMD data files. These files contain the contents of the housekeeping memory dump packets, converted to dimensional quantities, where appropriate. Each record consists of 9 header items followed by the contents of the telemetry packet.

| Table 25 :HMD Data File Contents and Structure |  |  |  |
| --- | --- | --- | --- |
| Field Name | Type | Units | Range |
| OriginalPacketID | integer |  |  |
| EventID | integer |  |  |
| PacketID | integer |  |  |
| SCLKTime | string |  |  |
| SCLKRaw | real |  |  |
| ERTTime | string |  |  |
| Processedtime | string |  |  |
| Mode | string |  |  |
| Valid | boolean |  |  |
| InstMode | integer |  | 0-7 |
| SequenceTable | integer |  | 0-31 |
| CycleIndex | integer |  | 0, 15 |
| ScanIndex | integer |  | 0, 15 |
| ElapsedTime | integer | s | 0-65535 |
| CycleTable | integer |  | 0-63 |
| Spare | integer |  | n/a |
| MemoryType | integer |  | 0-7 |
| NumberofWords | integer |  | (1-32) |
| StartAddress | integer |  | 0-65535 |
| TCReceived | integer |  | 0-255 |
| BIU_Bus_Fault | integer |  | 0-255 |
| Machine_Error | integer |  | 0, 15 |
| TCRejected | integer |  | 0, 15 |
| InstTMMode | integer |  | 0-255 |
| BIUSpare | integer |  | 0-3 |
| TGORDT | boolean |  | 0, 1 |
| AlfOverrideInhibit | boolean |  | 0, 1 |
| AlfBootEnable | boolean |  | 0-255 |
| DefSciEnable | boolean |  | 0-3 |
| BIUSleep | boolean |  | 0, 1 |
| BIUPORD0 | boolean |  | 0, 1 |
| HKFull | boolean |  | 0, 1 |
| HKSSysF | boolean |  | 0, 1 |
| HKMSGError | boolean |  | 0, 1 |
| TMSCError | boolean |  | 0, 1 |
| StTblError | boolean |  | 0, 1 |
| AncBCError | boolean |  | 0, 1 |
| BIUSpare2 | boolean |  | 0, 1 |
| RTIsFail | boolean |  | 0, 1 |
| Fil1ON | boolean |  | 0, 1 |
| Fil2ON | boolean |  | 0, 1 |
| Fil3ON | boolean |  | 0, 1 |
| Fil4ON | boolean |  | 0, 1 |
| DconSpare1 | boolean |  | 0, 1 |
| Fil6ON | boolean |  | 0, 1 |
| DconSpare2 | integer |  | 0-3 |
| BIUWDX | boolean |  | 0, 1 |
| BIUFT | boolean |  | 0, 1 |
| FCFT | boolean |  | 0, 1 |
| FCTgo | boolean |  | 0, 1 |
| FCIllad | boolean |  | 0, 1 |
| FCNPU | boolean |  | 0, 1 |
| FCSuren | boolean |  | 0-3 |
| BIUDTsLoaded | boolean |  | 0, 1 |
| Word0 | integer |  | 0 ≤ x ≤ 0xFFFF |
| • ¦ • ¦ • | • • • |  | 0 ≤ x ≤ 0xFFFF |
| Word31 | integer |  | 0 ≤ x ≤ 0xFFFF |
| TgoDetect | boolean |  | 0, 1 |
| BIUResetOcc | boolean |  | 0, 1 |
| ICError | boolean |  | 0, 1 |
| TGOToggle | boolean |  | 0, 1 |
| SubmoduleID | integer |  | 0, 15 |
| TrapFlag | integer |  | 0-3 |
| ModuleID | integer |  | 0-63 |
| CRC |  |  | n/a |

# Appendix A: Directory Structure for volumes of the CO-S-INMS-3-L1A-U-V1.0 Data Set

+-INDEX ----+---INDXINFO.TXT

|	+---INDEX.TAB

|	\---INDEX.LBL

|

+-DOCUMENT--+

|	+---DOCINFO.TXT

|	+---VOLSIS.DOC

|	+---VOLSIS.ASC

|	+---VOLSIS.LBL

|	\---additional documentation files and labels

|

+-CATALOG --+---CATINFO.TXT

|	+---CO_INMS_L1A_DS.CAT

|	+---CO_INSTHOST.CAT

|	+---CO_INMS_INST.CAT

|	+---CASSINI_MISSION.CAT

|	+---CO_INMS_PERSION.CAT

|	\---CO_INMS_REF.CAT

|

+-CALIB-----+---yyyyddd_CAL_vv.CSV

|	\---yyyyddd_CAL_vv.CSV

|

+-DATA-------+----yyyy----+---ddd-----+---yyyyddd00_L1A_vv.CSV

|	|	|	+---yyyyddd01_L1A_vv.CSV

|	|	|	+---yyyyddd02_L1A_vv.CSV

|	|	|	+---yyyyddd03_L1A_vv.CSV

|	|	|	+---up to 20 additional L1A files

|	|	|	+---yyyyddd00_L1A_vv.LBL

|	|	|	+---yyyyddd01_L1A_vv.LBL

|	|	|	+---yyyyddd02_L1A_vv.LBL

|	|	|	+---yyyyddd03_L1A_vv.LBL

|	|	|	+---up to 20 additional L1A labels

|	|	|	+---L1A_STRUCT_vv.FMT

|	|	+---ddd-----+

|	\----yyyy---+---ddd-----+

|

v

continued on next page

continued from previous page

^

|

+-BROWSE----+----yyyy---+---ddd-----+---yyyddd00_SPECTRA_vv.PNG

|	|	|	+---yyyddd00_SPECTRA_vv.LBL

|	|	|	+---yyyddd00_LINE_vv.PNG

|	|	|	+---yyyddd00_LINE_vv.LBL

|	|	|	+---up to 5 more browse product sets

|	|	+---ddd-----+

|	\----yyyy---+---ddd-----+

|

\-EXTRAS----+--SOFTWARE-+---INFO.TXT

|	+---DISCLAMER.TXT

|	+---INMS_*.PRO	idl v6.1 source code files

|	+---SPRL_*.PRO	idl v6.1 source code files

|	+---USERGUIDE.DOC	user’s guide to routines

|	\---INMS_ANALYSIS_HELP.HTL	online help

|

\-CALREPORT--> see APENDIX C

# Appendix B: Directory Structure for volumes of the CO-S-INMS-2-PKT-U-V1.0 Data Set

+-INDEX ----+---INDXINFO.TXT

|	+---INDEX.TAB

|	\---INDEX.LBL

|

+-DOCUMENT--+

|	+---DOCINFO.TXT

|	+---VOLSIS.DOC

|	+---VOLSIS.ASC

|	+---VOLSIS.LBL

|	\---additional documentation files and labels

|

+-CATALOG --+---CATINFO.TXT

|	+---CO_INMS_PKT_DS.CAT

|	+---CO_INSTHOST.CAT

|	+---CO_INMS_INST.CAT

|	+---CASSINI_MISSION.CAT

|	+---CO_INMS_PERSION.CAT

|	\---CO_INMS_REF.CAT

|

\-DATA------+----yyyy---+---ddd-----+---yyyyddd00_SCI_vv.CSV

|	|	+---up to 23 additional SCI files

|	|	+---yyyyddd00_SCI_vv.LBL

|	|	+---up to 23 additional L1A Labels

|	|	+---yyyyddd00_HKG_vv.CSV

|	|	+---yyyyddd00_OTS_vv.CSV

|	|	+---yyyyddd00_SMD_vv.CSV

|	|	+---yyyyddd00_HMD_vv.CSV

|	|	+---yyyyddd00_SCI_vv.LBL

|	|	+---yyyyddd00_HKG_vv.LBL

|	|	+---yyyyddd00_OTS_vv.LBL

|	|	+---yyyyddd00_SMD_vv.LBL

|	|	+---yyyyddd00_HMD_vv.LBL

|	+---ddd-----+

|	+---ddd-----+

\----yyyy---+---ddd-----+

# Appendix C: CALIBRATION REPORT Directory Structure

CALREPORT->

+---INMS_FU_Characterization

|   +---1-Directory

|   +---2-General

|   |   +---2.1-Document_status

|   |   +---2.2-Data_flow

|   |   +---2.3-Data_programs

|   |   +---2.4-Data_program_ouput

|   |   |   +---2.4.01-0130n2pri

|   |   |   +---2.4.02-0131n2sec

|   |   |   +---2.4.03-0131arsec

|   |   |   +---2.4.04-0201arpri

|   |   |   +---2.4.05-0201ch4sec

|   |   |   +---2.4.06-0202ch4pri

|   |   |   +---2.4.07-0202hepri

|   |   |   +---2.4.08-0202hesec

|   |   |   +---2.4.09-0204h2pri

|   |   |   +---2.4.10-0204h2sec

|   |   |   +---2.4.11-0205krpri

|   |   |   +---2.4.12-0205krsec

|   |   |   +---2.4.13-0208ar02.8ev

|   |   |   +---2.4.14-0209ar08.0ev

|   |   |   +---2.4.15-0209ar14.0ev

|   |   |   +---2.4.16-0210kr02.8ev

|   |   |   +---2.4.17-0210kr08.0ev

|   |   |   +---2.4.18-0210kr14.0ev

|   |   |   +---2.4.19-0211he02.8ev

|   |   |   +---2.4.20-0213c2h2sec

|   |   |   +---2.4.21-0213c2h2pri

|   |   |   +---2.4.22-0213c2h4pri

|   |   |   +---2.4.23-0213c2h4sec

|   |   |   \---2.4.24-0213noble

|   |   \---2.5-Miscellaneous_files

|   +---3-GSE_files

|   |   +---3.1-ATOL_sequences

|   |   +---3.2-ATOL_serial_numbers

|   |   +---3.3-SUN_files

|   |   |   +---3.3.4-GSE_data_therm_ion

|   |   |   \---3.3.5-GSE_phd_summary

|   |   +---3.4-Programs

|   |   |   +---3.4.1-avghk

|   |   |   \---3.4.2-avgscience_sp1

|   |   \---3.5-Misc_files

|   |       +---3.5.1-Commands

|   |       |   \---Commands

|   |       |       +---bak

|   |       |       \---tmp

|   |       \---3.5.2-Scripts

|   |           \---Scripts

v   v

continued on next page

continued from previous page

^   ^

|   |

|   +---4-Ions

|   |   +---4.1-Description

|   |   \---4.2-Data

|   |       +---4.2.1-Ion_energy

|   |       \---4.2.2-Other

|   |           +---4.2.2.1-INMS_quad_lens_setup

|   |           |   +---4.2.2.1.2.02-thermal_gas

|   |           |   +---4.2.2.1.2.03-2.8ev_ions

|   |           |   +---4.2.2.1.2.04-3.8ev_ions

|   |           |   +---4.2.2.1.2.05-6ev_ions

|   |           |   +---4.2.2.1.2.06-7.8ev_ions

|   |           |   +---4.2.2.1.2.07-10ev_ions

|   |           |   +---4.2.2.1.2.08-12ev_ions

|   |           |   +---4.2.2.1.2.09-14.3ev_ions

|   |           |   +---4.2.2.1.2.10-16.5ev_ions

|   |           |   +---4.2.2.1.2.11-20.15ev_ions

|   |           |   \---4.2.2.1.2.13-switch_lens_plots

|   |           +---4.2.2.2-Ion_ang_amp_plots

|   |           +---4.2.2.3-ion_sens

|   |           \---4.2.2.4-18_dalton_sweep

|   +---5-Thermal_gas

|   |   +---5.1-Description

|   |   \---5.2-Data

|   |       +---5.2.1-Gas_filament

|   |       |   +---5.2.1.1-Thermal_gas_plots

|   |       |   |   +---5.2.1.1.1-Therm_gas_18_swp

|   |       |   |   |   +---5.2.1.1.1-Therm_gas_18_swp.html

|   |       |   |   |   +---5.2.1.1.1-Therm_gas_18_swp.p65

|   |       |   |   |   \---5.2.1.1.1-Therm_gas_18_swp.pdf

|   |       |   |   \---5.2.1.1.2-Therm_gas_Hi_Lo

|   |       |   \---5.2.1.2-Thermal_gas_data

|   |       |       +---5.2.1.2.1-Initial_th_gas_data

|   |       |       \---5.2.1.2.1-Rev_th_gas_data

|   |       |           +---5.2.1.2.1_rev_1

|   |       |           +---5.2.1.2.1_rev_2

|   |       |           \---5.2.1.2.1_rev_3

|   |       |               +---0130n2pri_r3

|   |       |               +---0131arsec_r3

|   |       |               +---0131n2sec_r3

|   |       |               +---0201arpri_r3

|   |       |               +---0201ch4sec_r3

|   |       |               +---0202ch4pri_r3

|   |       |               +---0202hepri_r3

|   |       |               +---0202hesec_r3

|   |       |               +---0204h2pri_r3

|   |       |               +---0204h2sec_r3

|   |       |               +---0205krpri_r3

v   v       v               v

continued on next page

continued from previous page

^   ^       ^               ^

|   |       |               +---0205krsec_r3

|   |       |               +---0213c2h2pri_r3

|   |       |               +---0213c2h2sec_r3

|   |       |               +---0213c2h4pri_r3

|   |       |               +---0213c2h4sec_r3

|   |       |               \---FU_Thermal_Summary_r3

|   |       |                   +---FU_description_r3

|   |       |                   \---FU_files_r3

|   |       \---5.2.2-Other

|   |           +---5.2.2.1-N2_tracking

|   |           |   +---5.2.2.1.1-N2_tracking_descript

|   |           |   +---5.2.2.1.2-N2_tracking_data

|   |           |   +---5.2.2.1.3-N2_tracking_GSE_files

|   |           |   \---5.2.2.1.4-N2_tracking_plots

|   |           +---5.2.2.2-SRG_Baratron

|   |           +---5.2.2.3-Sensitivity_summary

|   |           \---5.2.2.4-Noble_gas_scan

|   |               \---5.2.2.4.2-noble_plots

|   \---6-Miscellaneous

|       +---6.1-Ion_defl_V

|       |   \---6.1.2-defl_dac_plots

|       +---6.2-INMS_volts

|       +---6.3-Chacterization_system

|       |   +---6.3.1-system_description

|       |   +---6.3.2-procedures

|       |   |   \---6.3.2.2-Therm_gas_inlet_dia

|       |   \---6.3.3-system_pictures

|       +---6.4-SEM_gain

|       |   +---6.4.2-SEM_gain_data

|       |   +---6.4.2.1_SEM_gain_descript

|       |   \---6.4.3-Sem_gain_plots

|       +---6.5-Open_source_quad_bias

|       +---6.6-INMS_trend_analysis

|       \---6.7-INMS_sensor

|           +---6.7.1-sensor_schematic

|           \---6.7.2-sensor_on_char_sys

v

continued on next page

continued from previous page

^

|

\---INMS_EU_Characterization

|   +---General

|   +---Ions

|   |   +---Argon_Ions

|   |   |   +---1998-0917-ArgonIons10V

|   |   |   +---1998-0917-ArgonIons13_5V

|   |   |   +---1998-0917-ArgonIons20V

|   |   |   \---1998-0917-ArgonIons6_5V

|   |   +---Helium_Ions

|   |   |   +---1998-0916-HeliumIons10V

|   |   |   +---1998-0916-HeliumIons13_5V

|   |   |   +---1998-0916-HeliumIons20V

|   |   |   \---1998-0916-HeliumIons6_5V

|   |   +---Hydrogen_Ions

|   |   |   +---1998-0917_HydrogenIons13_5V

|   |   |   +---1998-0917_HydrogenIons10V

|   |   |   +---1998-0917_HydrogenIons20V

|   |   |   \---1998-0917_HydrogenIons6_5V

|   |   +---KryptonIons

|   |   |   +---1998-0918-KryptonIons10V

|   |   |   +---1998-0918-KryptonIons13_5V

|   |   |   +---1998-0918-KryptonIons20V

|   |   |   \---1998-0918-KryptonIons6_5V

|   |   +---Neon_Ions

|   |   |   +---1998-0918-NeonIons10V

|   |   |   +---1998-0918-NeonIons13_5V

|   |   |   +---1998-0918-NeonIons20V

|   |   |   \---1998-0918-NeonIons6_5V

|   |   +---Nitrogen_Ions

|   |   |   +---1998-0910-NitrogenIons20V

|   |   |   +---1998-0911-NitrogenIons13_5V

|   |   |   +---1998-0914-NitrogenIons10V

|   |   |   +---1998-0915-NitrogenIons6_5V

|   |   |   \---1998-0928-NitrogenIons6_5V

|   |   +---Switching Lens

|   |   \---X_Ion_Summary

|   |       +---EU_FU_6_5ev_Compare

|   |       \---ion_fit

|   |           +---10ev_L1_30v

|   |           +---13_5ev_L1_30v

|   |           +---20ev_L1_30v

|   |           \---6.5ev_L1_30v

v   v

continued on next page

continued from previous page

^   ^

|   +---Neutral Beam

|   |   +---1998-0919_NeutralBeamTest

|   |   +---1998-0921-MoreBeaming

|   |   +---1998-0922-NeutralBeamArKrInH2

|   |   \---X_Neutral_Beam_Summary.xls

|   +---Nitrogen_Tracking

|   +---Thermal_Gas

|   |   +---Acetylene

|   |   |   \---1998-0720-AcetyleneCal

|   |   +---Argon

|   |   |   \---1998-0629-Argon

|   |   +---Argon in Helium

|   |   |   \---1998-0824-ArgonInHelium

|   |   +---Benzene

|   |   |   \---1998-1001-Benzene

|   |   +---Carbon Dioxide

|   |   |   \---1998-0729-CarbonDioxideCal

|   |   +---Carbon Monoxide in Helium

|   |   |   +---1998-0723-COinHelium

|   |   |   \---1998-0819-COinHelium

|   |   +---CarbonMonoxide

|   |   |   \---1998-0925-ThermalCO

|   |   +---Ethane

|   |   |   \---1998-0723-EthaneCal

|   |   +---Ethylene

|   |   |   \---1998-0722-EthyleneCal

|   |   +---Helium

|   |   |   \---1998-0715-Helium

|   |   +---Hydrogen

|   |   |   \---1998-0716-HydrogenCal

|   |   +---HydrogenCyanide in Helium

|   |   |   +---1998-0930-HCNinHelium_run_1

|   |   |   \---1998-0930-HCNinHelium_run_2

|   |   +---Krypton

|   |   |   \---1998-0716-KryptonCal

|   |   +---Methane

|   |   |   \---1998-0629_Methane

|   |   +---Methylacetylene (Propyne) in Helium

|   |   |   +---1998-0824-MethylacetyleneInHelium_run_1

|   |   |   \---1998-0824-MethylacetyleneInHelium_run_2

|   |   +---Neon

|   |   |   \---1998-0717-NeonCal

|   |   +---Nitrogen

|   |   |   \---1998-0629-Nitrogen

|   v

continued on next page

continued from previous page

|   ^

|   |   +---Oxygen

|   |   |   +---1998-0727-OxygenCal

|   |   |   \---1998-0728-OxygenCal

|   |   +---Propadiene (Allene) in Helium

|   |   |   +---1998-0820-PropadieneInHelium

|   |   |   \---1998-0821-PropadieneInHelium

|   |   +---Propane

|   |   |   \---1998-0717-Propane

|   |   \---X_Thermal_Summary

|   |       \---EU_FU_Thermal_Summary

|   |           +---EU_save

|   |           +---FU_description_r3

|   |           +---FU_save

|   |           \---V _Anicich_Communications

|   \---Miscellaneous

|

+---Other

# Appendix D: L1A Structure File Contents

OBJECT        = FIELD

FIELD_NUMBER  = 1

BYTES         = 21

DATA_TYPE     = TIME

NAME          = "SCLK"

FORMAT        = "A21"

DESCRIPTION   = "Spacecraft event time in the UTC time scale expressed as a PDS compliant date-time string."

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 2

BYTES         = 8

DATA_TYPE     = ASCII_INTEGER

NAME          = "UTTIME"

FORMAT        = "I8"

DESCRIPTION   = "Time of measurement expressed in msec since midnight UTC."

UNIT          = "ms"

VALID_MINIMUM = 0

VALID_MAXIMUM = 86400001

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 3

BYTES         = 26

DATA_TYPE     = CHARACTER

FORMAT        = "A26"

NAME          = "TARGET"

DESCRIPTION   = "Name of target body."

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 4

BYTES         = 8

DATA_TYPE     = ASCII_INTEGER

NAME          = "TIME_CA"

FORMAT        = "I8"

DESCRIPTION   = "Time since closest approach."

UNIT          = "ms"

VALID_MINIMUM = -86400000

VALID_MAXIMUM = 86400000

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 5

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "TARG_POS_X"

FORMAT        = "F9.0"

DESCRIPTION   = "X component of the target body position in the IAU Saturn reference frame, included within 1 hour of closest approach."

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 6

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "TARG_POS_Y"

FORMAT        = "F9.0"

DESCRIPTION   = "Y component of the target body position in the IAU Saturn reference frame, included within 1 hour of closest approach."

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 7

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "TARG_POS_Z"

FORMAT        = "F9.0"

DESCRIPTION   = "Z component of the target body position in the IAU Saturn reference frame, included within 1 hour of closest approach."

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 8

BYTES         = 4

DATA_TYPE     = CHARACTER

NAME          = "SOURCE"

FORMAT        = "A4"

DESCRIPTION   = "Ion source used for this measurement,    Open Source Ion (osi)
Closed Source Neutral(csn)
Open Source Neutral Beam (osnb)
Open Souce Neutral Thermal (osnt)"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 9

BYTES         = 4

DATA_TYPE     = ASCII_INTEGER

NAME          = "DATA_RELIABILITY"

FORMAT        = "I4"

DESCRIPTION   = "This value is set to a non-zero value during scans in which an instrument transition may cause data reliability concerns. (Not implemented in Version 05 data files)"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 10

BYTES         = 8

DATA_TYPE     = CHARACTER

NAME          = "TABLE_SET_ID"

FORMAT        = "A8"

DESCRIPTION   = "Table set Identifier, consisting of an ID and revision number, TTTT-rrr."

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 11

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

NAME          = "COADD_CNT"

FORMAT        = "I3"

DESCRIPTION   = "Specifies the number of integration periods added together. A value of 1 indicates no coadding, 255 indicates maximum coadding - minimum data rate."

VALID_MINIMUM = 1

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 12

BYTES         = 7

DATA_TYPE     = CHARACTER

NAME          = "OSP_FIL_1_STATUS"

FORMAT        = "A7"

DESCRIPTION   = "Open source primary filament status (fil_1), OFF, LOW-EV (25eV) HIGH-EV (70eV)"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 13

BYTES         = 7

DATA_TYPE     = CHARACTER

NAME          = "OSS_FIL_2_STATUS"

FORMAT        = "A7"

DESCRIPTION   = "Open source secondary filament status (fil_2), OFF, LOW-EV (25eV) HIGH-EV (70eV)"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 14

BYTES         = 7

DATA_TYPE     = CHARACTER

NAME          = "CSP_FIL_3_STATUS"

FORMAT        = "A7"

DESCRIPTION   = "Closed source primary filament (fil_3) status, OFF, LOW-EV (27eV) HIGH-EV (71eV)"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 15

BYTES         = 7

DATA_TYPE     = CHARACTER

NAME          = "CSS_FIL_4_STATUS"

FORMAT        = "A7"

DESCRIPTION   = "Closed source secondary filament (fil_4) status, OFF, LOW-EV (27eV) HIGH-EV(71eV)"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 16

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "SEQ_TABLE"

DESCRIPTION   = "Number of the sequence table controlling the instrument  operation. This table lists a set of cycle tables, each  one of which specifies one or more mass scans."

VALID_MINIMUM = 1

VALID_MAXIMUM = 64

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 17

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "CYC_NUM"

FORMAT        = "I2"

DESCRIPTION   = "The number of the cycle within the sequence, indexes in this sequence table."

VALID_MINIMUM = 1

VALID_MAXIMUM = 31

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 18

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "CYC_TABLE"

FORMAT        = "I2"

DESCRIPTION   = "Identification number of the cycle table controlling the current mass scan. This table selects the mass, focus, trap, switching and D/A table used to operate the instrument for mass scan."

VALID_MINIMUM = 1

VALID_MAXIMUM = 64

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 19

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "SCAN_NUM"

DESCRIPTION   = "The number of the scan within the cycle, indexes the cycle table."

VALID_MINIMUM = 1

VALID_MAXIMUM = 31

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 20

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "TRAP_TABLE"

FORMAT        = "I2"

DESCRIPTION   = "Identification number of the trap table specifying  collimator, deflector, top plate and OL4 voltages  used in the mass scan."

VALID_MINIMUM = 1

VALID_MAXIMUM = 31

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 21

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "SW_TABLE"

FORMAT        = "I2"

DESCRIPTION   = "Identification number of the Switching table specifying the quadrupole rod voltages."

VALID_MINIMUM = 1

VALID_MAXIMUM = 31

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 22

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "MASS_TABLE"

FORMAT        = "I2"

DESCRIPTION   = "Identification number of the mass table specifying the 68 mass/charge values making up a mass scan."

VALID_MINIMUM = 1

VALID_MAXIMUM = 96

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 23

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "FOCUS_TABLE"

FORMAT        = "I2"

DESCRIPTION   = "Identification number of the focus table specifying OL1, OL2, OL3, endplate, ion lens, and quadrupole bias voltages."

VALID_MINIMUM = 1

VALID_MAXIMUM = 31

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 24

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

NAME          = "DA_TABLE"

DESCRIPTION   = "Identification number of the D/A table specifying the electron multiplier and detector threshold settings."

VALID_MINIMUM = 1

VALID_MAXIMUM = 16

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 25

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "VELOCITY_COMP"

FORMAT        = "F6.3"

DESCRIPTION   = "Compensation velocity used in onboard computation of lens and deflector voltages"

UNIT          = "km/s"

VALID_MINIMUM = 0

VALID_MAXIMUM = 50.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 26

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

NAME          = "IPNUM"

FORMAT        = "I2"

DESCRIPTION   = "Integration period number, indexes the mass, trap  switching and focus tables."

VALID_MINIMUM = 1

VALID_MAXIMUM = 68

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 27

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "MASS"

FORMAT        = "F6.3"

DESCRIPTION   = "Mass per unit charge detected during current integration period"

UNIT          = "AMU/Z"

VALID_MINIMUM = 0.125

VALID_MAXIMUM = 99.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 28

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "OS_LENS2"

FORMAT        = "F6.2"

DESCRIPTION   = "Open source lens 2 voltage."

UNIT          = "v"

VALID_MINIMUM = -10.00

VALID_MAXIMUM =  10.00

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 29

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "OS_LENS1"

FORMAT        = "F6.2"

DESCRIPTION   = "Open source lens 1 voltage."

UNIT          = "v"

VALID_MINIMUM = -10.00

VALID_MAXIMUM =  10.00

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 30

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "OS_LENS4"

FORMAT        = "F6.2"

DESCRIPTION   = "Open source lens 4 voltage."

UNIT          = "v"

VALID_MINIMUM = -10.00

VALID_MAXIMUM =  10.00

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 31

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "OS_LENS3"

FORMAT        = "F6.2"

DESCRIPTION   = "Open source lens 3 voltage."

UNIT          = "v"

VALID_MINIMUM = -10.00

VALID_MAXIMUM =  10.00

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 32

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "QP_LENS2"

FORMAT        = "F7.3"

DESCRIPTION   = "Quadrapole lens 2 voltage."

UNIT          = "v"

VALID_MINIMUM = -200.0

VALID_MAXIMUM = 200.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 33

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "QP_LENS1"

FORMAT        = "F7.3"

DESCRIPTION   = "Quadrupole lens 1 voltage."

UNIT          = "v"

VALID_MINIMUM = -200.0

VALID_MAXIMUM = 200.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 34

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "QP_LENS4"

FORMAT        = "F7.3"

DESCRIPTION   = "Quadrupole lens 4 voltage."

UNIT          = "v"

VALID_MINIMUM = -200.0

VALID_MAXIMUM = 200.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 35

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "QP_LENS3"

FORMAT        = "F7.3"

DESCRIPTION   = "Quadrupole lens 3 voltage."

UNIT          = "v"

VALID_MINIMUM = -200.0

VALID_MAXIMUM = 200.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 36

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "QP_BIAS"

DESCRIPTION   = "Quadrupole bias voltage."

FORMAT        = "F7.3"

UNIT          = "v"

VALID_MINIMUM = -200.0

VALID_MAXIMUM = 200.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 37

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "ION_DEFL2"

FORMAT        = "6.2"

DESCRIPTION   = "Ion deflector 2 voltage."

UNIT          = "v"

VALID_MINIMUM = -62.0

VALID_MAXIMUM = 62.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 38

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "ION_DEFL1"

FORMAT        = "6.2"

DESCRIPTION   = "Ion deflector 1 voltage."

UNIT          = "v"

VALID_MINIMUM = -62.0

VALID_MAXIMUM = 62.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 39

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "ION_DEFL4"

FORMAT        = "F6.2"

DESCRIPTION   = "Ion deflector 4 voltage."

UNIT          = "v"

VALID_MINIMUM = -62.0

VALID_MAXIMUM = 62.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 40

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "ION_DEFL3"

DESCRIPTION   = "Ion deflector 3 voltage."

FORMAT        = "F6.2"

UNIT          = "v"

VALID_MINIMUM = -62.0

VALID_MAXIMUM = 62.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 41

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "TOP_PLATE"

FORMAT        = "F6.2"

DESCRIPTION   = "Top plate lens voltage."

UNIT          = "v"

VALID_MINIMUM = -30.0

VALID_MAXIMUM = 30.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 42

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "P_ENERGY"

FORMAT        = "F7.3"

DESCRIPTION   = "Particle Energy."

UNIT          = "ev"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 50.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 43

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "ALT_T"

FORMAT        = "F9.2"

DESCRIPTION   = "Altitude of the spacecraft above the surface of the named target body, included within 1 hour of closest approach"

UNIT          = "km"

VALID_MINIMUM = -1.0E05

VALID_MAXIMUM = 1.0E05

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 44

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "VIEW_DIR_T_X"

FORMAT        = "F9.6"

DESCRIPTION   = "Components of the  UNIT vector in the direction of the INMS aperture outward normal expressed in the target centered IAU coordinate frame, included within 1 hour of closest approach."

VALID_MINIMUM = -1.0

VALID_MAXIMUM = 1.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 45

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "VIEW_DIR_T_Y"

FORMAT        = "F9.6"

DESCRIPTION   = "Components of the UNIT vector in the direction of the INMS aperture outward normal expressed in the target centered IAU coordinate frame, included within 1 hour of closest approach."

VALID_MINIMUM = -1.0

VALID_MAXIMUM = 1.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 46

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

NAME          = "VIEW_DIR_T_Z"

DESCRIPTION   = "Components of the UNIT vector in the direction of the INMS aperture outward normal expressed in the target centered IAU coordinate frame, included within 1 hour of closest approach."

VALID_MINIMUM = -1.0

VALID_MAXIMUM = 1.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 47

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_POS_T_X"

FORMAT        = "F9.2"

DESCRIPTION   = "X-component of spacecraft position with respect to the named target in the target centered IAU coordinate frame, included within 1 hour of closest approach."

UNIT          = "km"

VALID_MINIMUM = -1.0E05

VALID_MAXIMUM = 1.0E05

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 48

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_POS_T_Y"

FORMAT        = "F9.2"

DESCRIPTION   = "Y-component of spacecraft position with respect to the named target in the target centered IAU coordinate frame, included within 1 hour of closest approach."

UNIT          = "km"

VALID_MINIMUM = -1.0E05

VALID_MAXIMUM = 1.0E05

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 49

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_POS_T_Z"

FORMAT        = "F9.2"

DESCRIPTION   = "Z-component of spacecraft position with respect to the named target in the target centered IAU coordinate frame, included within 1 hour of closest approach."

UNIT          = "km"

VALID_MINIMUM = -1.0E05

VALID_MAXIMUM = 1.0E05

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 50

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_T_X"

FORMAT        = "F7.3"

DESCRIPTION   = "X-component of spacecraft velocity with respect to the named target in the target centered IAU coordinate frame, included within 1 hour of closest approach."

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 51

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_T_Y"

FORMAT        = "F7.3"

DESCRIPTION   = "Y-component of spacecraft velocity with respect to the named target in the target centered IAU coordinate frame, included within 1 hour of closest approach."

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 52

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_T_Z"

FORMAT        = "F7.3"

DESCRIPTION   = "Z-component of spacecraft velocity with respect to the named target in the target centered IAU coordinate frame, included within 1 hour of closest approach."

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 53

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_T_SCX"

FORMAT        = "F7.3"

DESCRIPTION   = "X-component of spacecraft velocity with respect to the named target in the spacecraft centered coordinate frame, included within 1 hour of closest approach."

UNIT          = "km/s"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 54

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_T_SCY"

FORMAT        = "F7.3"

DESCRIPTION   = "Y-component of spacecraft velocity with respect to the named target in the spacecraft centered coordinate frame, included within 1 hour of closest approach"

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 55

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_T_SCZ"

FORMAT        = "F7.3"

DESCRIPTION   = "Z-component of spacecraft velocity with respect to the named target in the spacecraft centered coordinate frame, included within 1 hour of closest approach"

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 56

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "LST_T"

FORMAT        = "F6.3"

DESCRIPTION   = "The local solartime at the sub-spacecraft point    on the target body, included within 1 hour of    closest approach."

UNIT          = "hr"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 24.00

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 57

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SZA_T"

FORMAT        = "F7.3"

DESCRIPTION   = "The solar zenith angle at the sub-satellite point on the target body, included within 1 hour of    closest approach"

UNIT          = "deg"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 180.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 58

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "SS_LONG_T"

FORMAT        = "F6.2"

DESCRIPTION   = "The west longitude of the sub-solar point on the target body, included within 1 hour of    closest approach"

UNIT          = "deg"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 360.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 59

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "DISTANCE_S"

FORMAT        = "F9.0"

DESCRIPTION   = "Distance of the spacecraft from Saturn's center"

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 60

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "VIEW_DIR_S_X"

FORMAT        = "F9.6"

DESCRIPTION   = "Components of the   UNIT vector in the direction of the  INMS aperture outward normal in the Saturn centered IAU coordinate frame."

VALID_MINIMUM = -1.0

VALID_MAXIMUM = 1.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 61

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "VIEW_DIR_S_Y"

FORMAT        = "F9.6"

DESCRIPTION   = "Components of the   UNIT vector in the direction of the  INMS aperture outward normal in the Saturn centered IAU coordinate frame."

VALID_MINIMUM = -1.0

VALID_MAXIMUM = 1.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 62

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "VIEW_DIR_S_Z"

FORMAT        = "F9.6"

DESCRIPTION   = "Components of the   UNIT vector in the direction of the  INMS aperture outward normal in the Saturn centered IAU coordinate frame."

VALID_MINIMUM = -1.0

VALID_MAXIMUM = 1.0

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 63

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_POS_S_X"

FORMAT        = "F9.0"

DESCRIPTION   = "X-component of spacecraft position in the Saturn centered IAU coordinate frame."

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 64

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_POS_S_Y"

FORMAT        = "F9.0"

DESCRIPTION   = "Y-component of spacecraft position in the Saturn centered IAU coordinate frame."

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 65

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_POS_S_Z"

FORMAT        = "F9.0"

DESCRIPTION   = "Z-component of spacecraft position in the Saturn centered IAU coordinate frame."

UNIT          = "km"

VALID_MINIMUM = -1.0E07

VALID_MAXIMUM = 1.0E07

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 66

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_S_X"

FORMAT        = "F7.3"

DESCRIPTION   = "X-component of spacecraft velocity in the Saturn centered IAU coordinate frame."

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 67

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_S_Y"

FORMAT        = "F7.3"

DESCRIPTION   = "Y-component of spacecraft velocity in the Saturn centered IAU coordinate frame."

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 68

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SC_VEL_S_Z"

FORMAT        = "F7.3"

DESCRIPTION   = "Z-component of spacecraft velocity in the Saturn centered IAU coordinate frame."

UNIT          = "km/s"

VALID_MINIMUM = -100.

VALID_MAXIMUM = 100.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 69

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "LST_S"

FORMAT        = "F6.3"

DESCRIPTION   = "The local solar time at the sub-spacecraft point on Saturn"

UNIT          = "hr"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 24.00

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 70

BYTES         = 7

DATA_TYPE     = ASCII_REAL

NAME          = "SZA_S"

FORMAT        = "F7.3"

DESCRIPTION   = "The solar zenith angle at the sub-satellite point on Saturn"

UNIT          = "deg"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 180.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 71

BYTES         = 6

DATA_TYPE     = ASCII_REAL

NAME          = "SS_LONG_S"

FORMAT        = "F6.2"

DESCRIPTION   = "The west longitude of the sub-solar point on Saturn"

UNIT          = "deg"

VALID_MINIMUM = 0.

VALID_MAXIMUM = 360.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 72

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_ATT_ANGLE_RA"

FORMAT        = "F9.6"

DESCRIPTION   = "Right ascension of spacecraft z axis."

UNIT          = "rad"

VALID_MINIMUM = 0.0

VALID_MAXIMUM = 6.283185

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 73

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_ATT_ANGLE_DEC"

FORMAT        = "F9.6"

DESCRIPTION   = "Declination of spacecraft z axis."

UNIT          = "rad"

VALID_MINIMUM = -1.5708

VALID_MAXIMUM = 1.5708

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 74

BYTES         = 9

DATA_TYPE     = ASCII_REAL

NAME          = "SC_ATT_ANGLE_TW"

FORMAT        = "F9.6"

DESCRIPTION   = "Rotation of spacecraft about z axis."

UNIT          = "rad"

VALID_MINIMUM = 0.0

VALID_MAXIMUM = 6.283185

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 75

BYTES         = 6

DATA_TYPE     = ASCII_INTEGER

NAME          = "C1COUNTS"

FORMAT        = "I6"

DESCRIPTION   = "High sensitivity counts."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1000000

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 76

BYTES         = 6

DATA_TYPE     = ASCII_INTEGER

NAME          = "C2COUNTS"

FORMAT        = "I6"

DESCRIPTION   = "Low sensitivity counts."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1000000

END_OBJECT    = FIELD

# Appendix E: HKG Structure File Contents

OBJECT        = FIELD

FIELD_NUMBER  = 1

NAME     = "nOriginalPacketID"

BYTES         = 10

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I10"

DESCRIPTION   = "The unique database ID of the packet"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 2

NAME          = "nEventID"

BYTES         = 10

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I10"

DESCRIPTION   = "The ID that references the INMSDB data  event the telemetry packet belongs to"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 3

NAME = "nAssocSciOrigpacketID"

BYTES         = 10

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I10"

DESCRIPTION   = "The associated science packet (the nearest  science packet chronologically) for this HK  packet. ** Not currently used."

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 4

NAME          = "tSclkTime"

BYTES         = 23

DATA_TYPE     = CHARACTER

FORMAT        = "A23"

DESCRIPTION   = "The spacecraft clock time of the packet -  does not include milliseconds (see  NSCLKMillis below for milliseconds)"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 5

NAME          = "dSclkRaw"

BYTES         = 15

DATA_TYPE     = ASCII_REAL

FORMAT        = F15.1

DESCRIPTION   = "The raw spacecraft clock time that  TSCLKTIME and NSCLKMillis were generated  from"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 6

NAME          = "tErtTime"

BYTES         = 23

DATA_TYPE     = CHARACTER

FORMAT        = "A23"

DESCRIPTION   = "The Earth Receive Time - the time the  packet was received from a NASA DSN  station"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 7

NAME        = "tProcessedTime"

BYTES         = 23

DATA_TYPE     = CHARACTER

FORMAT        = "A23"

DESCRIPTION   = "The UTC time the packet was processed by  the INMS data system"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 8

NAME          = "sMode"

BYTES         = 10

DATA_TYPE     = CHARACTER

FORMAT        = "A10"

DESCRIPTION   = "Packet mode, indicates if the packet is data or synthetic test data"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 9

NAME          = "bValid"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Indicates that the packet is  a valid packet"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 10

NAME          = "nInstMode"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Specifies the instrument operating mode"

VALID_MINIMUM = 0

VALID_MAXIMUM = 7

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 11

NAME         = "nSequenceTable"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Sequence table number"

VALID_MINIMUM = 0

VALID_MAXIMUM = 31

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 12

NAME          = "nCycleIndex"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Index of the cycle table entry"

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 13

NAME          = "nScanIndex"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "The number of the scan within the cycle"

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 14

NAME          = "nElapsedTime"

BYTES         = 5

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I5"

DESCRIPTION   = "The time elapsed since the start of the  current sequence table"

UNIT          = "s"

VALID_MINIMUM = 0

VALID_MAXIMUM = 65535

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 15

NAME          = "nCycleTable"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Cycle table number"

VALID_MINIMUM = 0

VALID_MAXIMUM = 63

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 16

NAME          = "nspare1"

BYTES         = 17

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "2-bit field.  LSB=ScanIndex MSB, MSB=CycleIndex MSB"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 17

NAME          = "nDataSumN"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Number of mass scans being co-added to reduce  telemetry requirements"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 18

NAME          = "nThermStatus"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Thermistor test status.  Performed before pressure BA test.   Only valid prior to cover removal."

VALID_MINIMUM = 0

VALID_MAXIMUM = 3

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 19

NAME          = "nThermDelta"

BYTES         = 4

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I4"

DESCRIPTION   = "Thermistor pressure AMUX value.  Amux ID=49.  Valid only prior to cover removal."

VALID_MINIMUM = -512

VALID_MAXIMUM = 512

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 20

NAME          = "nFswVersion"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Reports the flight software version"

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 21

NAME          = "nBaStatus"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Pressure test status, valid only prior to cover removal"

VALID_MINIMUM = 0

VALID_MAXIMUM = 3

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 22

NAME          = "nBaStep"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "BA DAC emission value.  Valid only prior to cover removal."

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 23

NAME          = "nBaValue"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Pressure at end of test. Valid only prior to cover removal"

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 24

NAME          = "nTcReceived"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Number of telecommands received"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 25

NAME          = "nAlfReceived"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Indicates a BIU bus error"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 26

NAME          = "nAlfRejected"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Indicates a 1750 machine error"

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 27

NAME          = "nTcRejected"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Number of telecommads rejected"

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 28

NAME          = "nInstTmMode"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Telmetry mode index"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 29

NAME          = "nBiuSpare1"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Readback of BIU discretes 6 and 7"

VALID_MINIMUM = 0

VALID_MAXIMUM = 3

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 30

NAME          = "bTgordt"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Readback of Flight computer boot mode descrete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 31

NAME       = "bAlfOverInhibit"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Readback of ALF override inhibit discrete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 32

NAME         = "bAlfBootEnable"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Readback of ALF boot enable discrete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 33

NAME          = "bDefSciEnable"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Readback of default science enable descrete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 34

NAME          = "bBiuSleep"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "readback of sleep discrete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 35

NAME          = "bBiuPord0"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "readback of BIU discrete 0"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 36

NAME          = "bHkFull"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU error flag set when houskeeping tm buffer is full"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 37

NAME          = "bHkSsysf"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU error flag set when subsystem fail bit is set  in 1553 status word"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 38

NAME          = "bHkMsgError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU error flag set when message error bit is set  in 1553 status word"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 39

NAME          = "bTmScError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU error flag set when the number words expected  in 1553 receive sub-address 29 is incorrect"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 40

NAME          = "bStTblError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU error flag set when the number words expected in  1553 receive sub-address 19 is incorrect"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 41

NAME          = "bAncbcError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU error flag set when the number words expected in 1553 receive sub-address 10 is incorrect"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 42

NAME          = "bbiuspare2"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "spare, always 0"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 43

NAME          = "bRtiSfail"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when no RTIs have been received for 2 seconds"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 44

NAME          = "bFil1On"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "State of primary open source filament"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 45

NAME          = "bFil2On"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "State of secondary open source filament"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 46

NAME          = "bFil3On"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "State of primary closed source filament"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 47

NAME          = "bFil4On"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "State of secondary closed source filament"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 48

NAME          = "bDconSpare1"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "spare"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 49

NAME          = "bFil6On"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "State of  BA pressure  gauge filament"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 50

NAME          = "nDconSpare2"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "spare"

VALID_MINIMUM = 0

VALID_MAXIMUM = 3

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 51

NAME          = "bBiuWdx"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU watchdog timer status, 1(ok) 0(expired)"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 52

NAME          = "bBiuFt"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set to 0 when a BIU bus timeout  is detected in boot code"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 53

NAME          = "bFcft"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set to 0 when a CPU Machine Error  is detected in boot code"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 54

NAME          = "bFctGo"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set when TGO reset indicator latch is set on bootup.  Always set to 0 in TM."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 55

NAME          = "bFcillad"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set to 0 when CPU accesses an  illegal address.  Always set to 0 in TM."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 56

NAME          = "bFcnpu"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set to 1 when CPU completes a  normal power up.  Always set to 0 in TM."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 57

NAME          = "bFcsuren"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set to 0 when CPU start-up RAM is enabled.  Always set to 0 in TM."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 58

NAME          = "bBiuDtsLoaded"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "BIU write status, set to 1 when BIU descriptor table is loaded on bootup."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 59

NAME          = "d1emission"

BYTES         = 7

DATA_TYPE     = ASCII_REAL

FORMAT        = "F7.4"

DESCRIPTION   = "Emission Current of primary open source filament"

UNIT          = "uA"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 25.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 60

NAME          = "d2emission"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Emission Current of secondary open source filament"

UNIT          = "uA"

VALID_MINIMUM = 0.05

VALID_MAXIMUM = 25.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 61

NAME          = "dImon1"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.6"

DESCRIPTION   = "Input current"

UNIT          = "A"

VALID_MINIMUM = 0.0

VALID_MAXIMUM = 1.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 62

NAME          = "dGround1"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Ground reference voltage"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 63

NAME          = "d1Current"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Primary open source filament current"

UNIT          = "A"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 1.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 64

NAME          = "d2Current"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Secondary open source filament current"

UNIT          = "A"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 1.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 65

NAME          = "d1Target"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Primary open source anode current"

UNIT          = "uA"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 27.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 66

NAME          = "d2Target"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Secondary open source anode current"

UNIT          = "uA"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 27.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 67

NAME          = "d1Bias50"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Open Source +40V Bias Oscillator Drive

Current "

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 68

NAME          = "d1Bias70"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Open Source primary Filament Bias Oscillator Drive Current"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 69

NAME          = "d2Bias70"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Open Source secondary Filament Bias

Oscillator Drive Current"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 70

NAME          = "dGround2"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Ground reference voltage"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 71

NAME          = "dAmString"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.4"

DESCRIPTION   = "AM Board Resistor Divider String Current"

UNIT          = "V"

VALID_MINIMUM = -645.

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 72

NAME          = "dFbString"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.4"

DESCRIPTION   = "FB Board Resistor Divider String Current"

UNIT          = "V"

VALID_MINIMUM = -513.

VALID_MAXIMUM = 4.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 73

NAME          = "dBaCollector"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "BA  pressure  gage collector current"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 74

NAME          = "dFbControl"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "-390V Regulator Control Voltage "

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 75

NAME          = "d3Emission"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Emission Current of primary closed source filament"

UNIT          = "uA"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 25.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 76

NAME          = "d4Emission"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Emission Current of secondary closed source filament"

UNIT          = "uA"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 25.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 77

NAME          = "dImon2"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.6"

DESCRIPTION   = "Input current"

UNIT          = "A"

VALID_MINIMUM = 0.0

VALID_MAXIMUM = 1.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 78

NAME          = "dEb4_4"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "anode 3 current monitor in volts"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 79

NAME          = "d3Current"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Primary closed source filament current"

UNIT          = "A"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 1.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 80

NAME          = "d4Current"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Secondary closed source filament current"

UNIT          = "A"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 1.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 81

NAME          = "d4Anode"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Primary closed source anode current"

UNIT          = "uA"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 10.2

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 82

NAME          = "d3Anode"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Secondary closed source anode current"

UNIT          = "uA"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 10.2

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 83

NAME          = "d3Bias50"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Closed Source +40V Bias Oscillator Drive Current"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 84

NAME          = "d3Bias70"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Closed  Source primary Filament Bias Oscillator Drive Current"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 85

NAME          = "d4Bias70"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Closed  Source secondary Filament Bias Oscillator Drive Current"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 86

NAME          = "dRfSupplyTemp"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

DESCRIPTION   = "RF power supply temperature"

UNIT          = "C"

VALID_MINIMUM = 13.

VALID_MAXIMUM = 32.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 87

NAME          = "dFbSupplyTemp"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

DESCRIPTION   = "FB Board temperature"

UNIT          = "C"

VALID_MINIMUM = 13.

VALID_MAXIMUM = 32.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 88

NAME          = "dEmSupplyTemp"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

DESCRIPTION   = "EM Board temperature"

UNIT          = "C"

VALID_MINIMUM = 13.

VALID_MAXIMUM = 32.5

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 89

NAME          = "dPlus5Fb"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Thermister +5 reference voltage"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 90

NAME          = "dBaGrid"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.4"

DESCRIPTION   = "BA pressure gage grid current"

UNIT          = "uA"

VALID_MINIMUM = -2.

VALID_MAXIMUM = 152.

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 91

NAME          = "dOl1"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Open Lens 1 (CEU) Current"

UNIT          = "uA"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 6.8

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 92

NAME          = "dOl2"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.5"

DESCRIPTION   = "Open Lens 2 (CEU) Current "

UNIT          = "uA"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 6.8

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 93

NAME          = "dBaCurrent"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

DESCRIPTION   = "BA pressure gage filament current"

UNIT          = "A RMS"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 1.4

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 94

NAME          = "dBaEmission"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.4"

DESCRIPTION   = "BA pressure gage emission current"

UNIT          = "uA"

VALID_MINIMUM = -1.6

VALID_MAXIMUM = 193.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 95

NAME          = "dGround3"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Ground reference voltage"

UNIT          = "V"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 96

NAME          = "dBaBias"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "BA pressure gage bias oscillator drive current"

UNIT          = "V"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 97

NAME          = "dRfAgc"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "RF automatic gain control monitor voltage"

UNIT          = "V"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 98

NAME          = "dMinus5_7"

BYTES         = 7

DATA_TYPE     = ASCII_REAL

FORMAT        = "F7.4"

DESCRIPTION   = "Monitor -5.7 Volt supply"

UNIT          = "V"

VALID_MINIMUM = -7.

VALID_MAXIMUM = 5.2

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 99

NAME          = "dFcTemp"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

DESCRIPTION   = "Flight computer board temperature"

UNIT          = "C"

VALID_MINIMUM = 18.5

VALID_MAXIMUM = 36.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 100

NAME          = "dLvpStemp"

BYTES         = 9

DATA_TYPE     = ASCII_REAL

FORMAT        = "F9.6"

DESCRIPTION   = "Low voltage power supply frame temperature"

UNIT          = "C"

VALID_MINIMUM = 23.25

VALID_MAXIMUM = 31.25

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 101

NAME          = "dEm1Current"

BYTES         = 7

DATA_TYPE     = ASCII_REAL

FORMAT        = "F7.4"

DESCRIPTION   = "Electron multiplier 1 current monitor"

UNIT          = "KV"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 38.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 102

NAME          = "dEm2Current"

BYTES         = 7

DATA_TYPE     = ASCII_REAL

FORMAT        = "F7.4"

DESCRIPTION   = "Electron multiplier 2 current monitor"

UNIT          = "KV"

VALID_MINIMUM = -0.5

VALID_MAXIMUM = 38.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 103

NAME          = "dImon3"

BYTES         = 8

DATA_TYPE     = ASCII_REAL

FORMAT        = "F8.6"

DESCRIPTION   = "Input current"

UNIT          = "A"

VALID_MINIMUM = 0.0

VALID_MAXIMUM = 1.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 104

NAME          = "dMultAna1"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Pulse Amp 1 Analog Voltage Proportional to Input Frequency "

UNIT          = "uA"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 10.2

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 105

NAME          = "dMultAna2"

BYTES         = 6

DATA_TYPE     = ASCII_REAL

FORMAT        = "F6.3"

DESCRIPTION   = "Pulse Amp 2 Analog Voltage Proportional to  Input Frequency "

UNIT          = "uA"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 1.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 106

NAME          = "dPlus5r"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Monitor 5R voltage"

UNIT          = "V"

VALID_MINIMUM = -0.1

VALID_MAXIMUM = 10.2

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 107

NAME         = "dThermPressInt"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "Internal BA pressure gage temperature"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 108

NAME         = "dThermPressExt"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.2"

DESCRIPTION   = "External BA pressure gage temperature"

UNIT          = "V"

VALID_MINIMUM = -0.05

VALID_MAXIMUM = 5.1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 109

NAME          = "dPlus13"

BYTES         = 7

DATA_TYPE     = ASCII_REAL

FORMAT        = "F7.4"

DESCRIPTION   = "Monitor +13 voltage"

UNIT          = "V"

VALID_MINIMUM = -0.2

VALID_MAXIMUM = 14.75

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 110

NAME          = "nIndex"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "AMUX index of first channel sampled for latest scan."

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 111

NAME          = "dRfFreq"

BYTES         = 5

DATA_TYPE     = ASCII_REAL

FORMAT        = "F5.3"

DESCRIPTION   = "Frequency monitor"

VALID_MINIMUM = 0.0

VALID_MAXIMUM = 0.22

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 112

NAME          = "nCpuIdleCounter"

BYTES         = 2

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Flight computer heartbeat counter"

VALID_MINIMUM = 0

VALID_MAXIMUM = 15

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 113

NAME          = "bDisc0Reset"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set to 1 on bootup to indicate last CPU reset was  due to BIU discrete 0."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 114

NAME          = "bAllSysGo"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Master INMS error flag"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 115

NAME       = "bAllSysGoLatched"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Latched version of master INMS error flag"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 116

NAME          = "bExcNoHandler"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "unhandled exception occured "

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 117

NAME          = "bSafeMode"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "not used"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 118

NAME          = "bForcedSleep"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Indicates the instrument entered Sleep state due to  detected high value on IMONs"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 119

NAME          = "bBiuAutoInit"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "3-bit field that counts BIU auto-initializations."

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 120

NAME          = "bGseTest"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "not used"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 121

NAME    = "bSystemInitialized"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Indicates system initialization complete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 122

NAME     = "bSystemConfigured"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Indicates system configuration is complete"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 123

NAME          = "bTgoDetect"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Readout of watchdog reset latch on FC board during bootup. Usually set to 1 - unreliable indicator."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 124

NAME          = "bBiuReseToCc"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Indicates that a BIU reset has occured"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 125

NAME          = "bIcError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set to 1 on bootup if reset is due to wayward  instruction counter (jump to location 0)."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 126

NAME          = "bTgoToggle"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Toggled between 1 and 0 each time the Watchdog Reset Task executes"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 127

NAME          = "nSubModuleID"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Ada exception sub-module ID (CSU)  0=SciPkt, 1= Usid, 4=UseqISR, 8=USIDLostSync"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 128

NAME          = "nTrapFlag"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Ada exception/error type: 0=Trace_Trap, 1=Error_Trap, 2=Exception_Trap"

VALID_MINIMUM = 0

VALID_MAXIMUM = 3

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 129

NAME          = "nModuleId"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I2"

DESCRIPTION   = "Ada exception module ID (CSC)  1=INMS_Exec, 2=INMS_Init, 3=AMux_IO, 4=DMux_IO, 5=DCon_IO, 6=CDS_IO, 7=Useq_IO, 8-10=spare,   11=Filament Control, 12=Ion Source Select Control, 13=Ion Focus Control, 14=HV Focus Control, 15=Quad Rod Control, 16=Ion Det. Control, 17=Ion Pump Control, 18=Pressure Sensor Monitor, 19=Power Supply Monitor, 20=Micro-Sequencer Control"

VALID_MINIMUM = 0

VALID_MAXIMUM = 63

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 130

NAME          = "bInmsBitFail"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Bootstrap Built-in-Test results"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 131

NAME          = "bTmBuffOvrflw"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when the allotted memory buffer area for  TM packets would have overflowed"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 132

NAME          = "bAlfBoot"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when an ALF Boot is performed"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 133

NAME          = "bAlfError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when any BIU or ALF error is detected  during an ALF Boot"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 134

NAME          = "bBiuMemerror"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when background BIU verification processing  detects a bad BIU memory location. No longer used."

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 135

NAME          = "bOdbfrOverflow"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Output data buffer overflow. Not used"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 136

NAME          = "bUseqOOffline"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when micro-sequencer output communication  goes offline"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 137

NAME          = "bUseqIOffline"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when micro-sequencer input communication  goes offline"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 138

NAME          = "bUseqError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when any micro-sequencer error is detected"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 139

NAME          = "b1750me"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when the 1750 Machine Error interrupt occurs  and FT register <> 0400h"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 140

NAME          = "bAmuxError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "not used"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 141

NAME          = "bCdsOOffline"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when BIU/CDS output communication goes offline"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 142

NAME          = "bCdsIOffline"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set when BIU/CDS input communication goes offline"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 143

NAME          = "bCdsError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "set if one or more of 1750ME, CDSoOffline or  CDSiOffline are set"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 144

NAME          = "bRteError"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = "Set any time bootstrap or RTE processing  detects any of the following errors: 1750 BIT Fail;  Any Memory Test fails; Unhandled Exceptions;  Machine Error; TGO Reset or Wayward IC Reboot"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 145

NAME          = "nMemChecksum"

BYTES         = 1

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I1"

DESCRIPTION   = " memory checksum error flag in FSW Version 8 or later  0(ok) 1(bad checksum)"

VALID_MINIMUM = 0

VALID_MAXIMUM = 1

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 146

NAME          = "nStmCounter"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Count of spacecraft tm mode message received  (1 per second)"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 147

NAME          = "nRtiCounter"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Count of RTIs processed"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 148

NAME          = "nMassTable"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "value of initial mass in current mass table"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 149

NAME          = "nCmdSerno1"

BYTES         = 3

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I3"

DESCRIPTION   = "Serial Number and Time of the telecommand that is  being acknowledged. Only two are maintained for  each Packet"

VALID_MINIMUM = 0

VALID_MAXIMUM = 255

END_OBJECT    = FIELD

OBJECT        = FIELD

FIELD_NUMBER  = 150

NAME          = "nSclkMilliSec"

BYTES         = 9

DATA_TYPE     = ASCII_INTEGER

FORMAT        = "I9"

DESCRIPTION   = "The milliseconds of the time of the  packet. When used in combination with TSCLKTIME  produces the exact spacecraft clock time the packet was generated"

END_OBJECT    = FIELD