*****File RADWAV.DOC

I. BACKGROUND

On September 11, 1985, the International Cometary Explorer (ICE;

originally called ISEE-3) spacecraft made in situ measurements of

particles, fields and waves as it traversed the plasma tail of comet

Giacobini-Zinner (G-Z). The time of closest approach was approximately

11:02 UT and occurred at a tailward distance of 7,800 km. The spacecraft

was inside the bow wave for roughly 3 hours and in the main tail for about

10 min (Brandt et al., 1985;  Brandt et al., 1988, and references

therein).

Data from some of the instruments on the ICE spacecraft are

included on this CD-ROM archive for the period 08-14 September, 1985.

Ephemeris data giving hourly values of the ICE spacecraft trajectory

relative to Earth, Sun and G-Z are included in a separate file. Note

that the deviation of the spacecraft from uniform linear motion was

negligible during the encounter, so that interpolation of the trajectory

is straightforward.

References:

----------

Brandt, J.C., M.B. Niedner, and T. von Rosenvinge, "The ICE Project,"

Adv. Space Res., v.5, 3-16, 1985.

Brandt, J.C., "The International Cometary Explorer (ICE) Mission to

Comets Giacobini-Zinner and Halley" in "Space Missions to Halley's Comet",

p. 99-115, R. Reinhard, B. Battrick (Editors), ESA SP-1066, ESA

Publications Division, Noordwijk, The Netherlands, ISBN 0379 6566, 1986.

Brandt, J.C., R.W. Farquhar, S.P. Maran, M.B. Niedner, Jr., and T.T.

von Rosenvinge, "The International Cometary Explorer" in "Exploration

of Halley's Comet. Based on the 20th ESLAB Symposium, held in

Heidelberg, F.R.G., 27-31 October 1986," p. 969-980.

M. Grewing, F. Praderie, R. Reinhard (Editors). Springer-Verlag,

Berlin, F.R.G. 23+984 pp. ISBN 3-540-17344-7 (F.R.G.), ISBN

0-387-17344-7 (USA), 1988.

Epstein, G., Hoang, S., Huntzinger, G., Steinberg J. -L., Fainberg, J.,

Grena, F., Mosier, S.R., and Stone, R.G.,  "The 3-Dimensional Radio Mapping

Experiment (SBH) on ISEE-C", IEEE Trans. on Geoscience Electronics, GE-16,

191, 1978.

=========================================================================

II. Radio Mapping Experiment Derived Data

This file describes the values of the density and temperature of

the electrons measured (radio mapping) during the tail crossing of comet

Giacobini-Zinner by ISEE-3/ICE as derived from spectroscopy of the

thermal noise spectrum. The data was provided to the National Space

Science Data Center (NASA/GSFC) by the Principal Investigator of the

Radio Mapping Experiment on ISEE-3/ICE, Dr. Jean-Louis Steinberg of

the Observatoire de Paris, Meudon, France. The instrument is described

in "The 3-dimensional Radio Mapping Experiment (SBH) on ISEE-C", by R.

Knoll, G.Epstein, S.Hoang, G.Huntzinger, J.L.Steinberg, J.Fainberg,

F.Grena, S.R.Mosier, and R.G.Stone, IEEE Transactions on Geoscience

Electronics, vol. GE-16, No. 3, pp.199-204, July 1978. The data cover

the time interval from 10:00 UT to 12:00 UT on September 11, 1985. The

time resolution is 54 seconds.

Since this method of measuring plasma parameters is relatively

new, users of these data are referred to the following papers:

N.Meyer-Vernet, P.Couturier, S.Hoang, C.Perche, J.-L. Steinberg,

J.Fainberg and C.Meetre, Science, Vol.232, p.370, 1986.

N.Meyer-Vernet and C.Perche, J. Geophys. Res., Vol.94, p.2405, 1989.

To derive the plasma parameters, it is assumed that the

electron distribution function can be approximated by a bi-maxwellian.

The density given below is the total electron density (both electron

populations) derived from the cut-off frequency of the thermal noise

spectrum. The temperature is that of the cold electron population

which is always the dominant one. Error bars on the two measured

quantities are given in the first of the above mentioned papers.

TIME       N       T

HHMMSS    cm-3     Deg. K

100000    9.46  0.1585E+06

100054    7.69  0.1585E+06

100148    8.11  0.1585E+06