Reformatted Marie Data:

Notes:

a)

A1is Detector A1

A2 isDetector A2

B1/2 is Detectors B1 & B2 on the same board (One Temperature, 2 detector values)

B3/4 is Detectors B2 & B3 on the same board (One Temperature, 2 detector values)

C is C Detector

Power is Power card

CPU is CPU card

PSD1 is PSD1 detector

PSD2 is PSD2 Detector

b)

There are 2 temperature devices per board; mrg.det, and mrg.brd describe each device data

c)

Byte = 8 bit byte

Word = 16 bit unsigned

Real = 6 bytes

Mrg.evn:

The raw event files (EVN*.DAT in the PDS archive) have the Mrg.evn format.

The Mrg.evn file is a binary file of _detEventRecord records. Please see the following structure:

DetectorType = ( DetA1, DetA2, DetB1T, DetB1B, DetB2T, DetB2B, DetC1,

DetP1REv1Mag, DetP1REv1Pos, DetP1REv2Mag, DetP1REv2Pos,

DetP1CEv1Mag, DetP1CEv1Pos, DetP1CEv2Mag, DetP1CEv2Pos,

DetP2REv1Mag, DetP2REv1Pos, DetP2REv2Mag, DetP2REv2Pos,

DetP2CEv1Mag, DetP2CEv1Pos, DetP2CEv2Mag, DetP2CEv2Pos);

FlagType =     ( FlagA1,   FlagA2,   FlagA3,

FlagB1,   FlagB2,   FlagB3,   FlagC1,

FlagP1_S1,   FlagP2_S1, FlagP1_S2,   FlagP2_S2);

EventsArray = Array [ DetectorType ] of word;

FlagsArray  = Array [ FlagType ] of byte;

_DetEventRecord = Record

TypeId  : Byte;    { 1 byte }

InstID  : Byte;

RunId   : Byte;

RecordId: Byte;

len     : Word;      { 16 bit unsigned }

CheckSum: word;

NumberEvents: byte;

Time    : real; { 6 bytes }

Events  : EventsArray;

Flags   : FlagsArray;

end;

Mrg.cnt:

The count files (CNT*.DAT in the PDS archive) have the Mrg.cnt format.

The Mrg.cnt file is a binary file of records. Please see the following structure:

DetCountsRec = record

x : Real;

y : _DetCountRecord;

end;

CountType =    ( CntA1F, CntA2F, CntB1F, CntB2F, CntC1F);

CountsArray = Array [ CountType  ] of word;

_DetCountRecord = Record

TypeId  : Byte;

InstID  : Byte;

RunId   : Byte;

RecordId: Byte;

len     : Word;

Time    : real;

CheckSum: word;

Counts  : CountsArray;

end;

Mrg.det:

The detector temperature files (DET*.TAB in the PDS archive) have the Mrg.det format.

Is a text file describing the detector temperatures with a UTC Time Stamp, followed by a Julian time.  The file heading lists the order of the boards.  (In the PDS archive, the file heading is simpler than shown and occupies 1 row.)

A1     A2     B1/2   B3/4   C     Power   CPU   PSD1   PSD2

_______________________________________________________________

10APR02  06:31:52.34  1052633191.8720703     28.0000000      31.0000000      29.0000000      26.0000000      25.0000000      22.0000000      26.0000000      37.0170799      37.7787284

Mrg.brd:

The board temperature files (BRD*.TAB in the PDS archive) have the Mrg.brd format.

Is a text file describing the board temperatures with a UTC Time Stamp, followed by a Julian time.  The file heading lists the order of the boards.  (In the PDS archive, the file heading is simpler than shown and occupies 1 row.)

A1     A2     B1/2   B3/4   C     Power   CPU   PSD1   PSD2

_______________________________________________________________

10APR02  06:31:52.34  1052633191.8720703     28.0000000      31.0000000      29.0000000      26.0000000      25.0000000      22.0000000      26.0000000      37.0170799      37.7787284

# Mrg.pwr

The board power files (PWR*.TAB in the PDS archive) have the Mrg.pwr format.

Is a text file describing the power consumption in mw, with a UTC Time Stamp, followed by a Julian time.  The file heading lists the order of the boards.  (In the PDS archive, the file heading is simpler than shown and occupies 1 row.)

Power Consumption (mw)

______________________

A1   A2     B1/2   B3/4   C      Power  CPU    PSD1   PSD2

____________________________________________________________

10APR02  06:31:52.34  1052633191.8720703    210.0000000      710.0000000      305.0000000      420.0000000      355.0000000      205.0000000      745.0000000      872.5065199      775.2944986